[ PROTOCOLS:NBSOSI-AGREEMENTS.DOC ]                              [ 8/88 ]

Computer Science and Technology

NBS Special Publication 500-150

Stable Implementation Agreements for Open Systems Interconnection Protocols

Version 1 Edition 3

December 1987

Based on the Proceedings of the NBS Workshop for Implementors of OSI

U.S. Department of Commerce

C. William Verity, Secretary

National Bureau of Standards

Ernest Ambler, Director

Issued January 1988

Table of Contents

1. GENERAL INFORMATION . . . . . . . . . . . . . . . . . . . . . . .    1

1.1  PURPOSE OF THIS DOCUMENT . . . . . . . . . . . . . . . . . .    1

1.2  PURPOSE OF THE WORKSHOP  . . . . . . . . . . . . . . . . . .    1

1.3  WORKSHOP ORGANIZATION  . . . . . . . . . . . . . . . . . . .    2

2. SUB NETWORKS  . . . . . . . . . . . . . . . . . . . . . . . . . .    1

2.1  INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . . .    1

2.2  SCOPE AND FIELD OF APPLICATION . . . . . . . . . . . . . . .    1

2.3  STATUS . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

2.4  ERRATA . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

2.5  LOCAL AREA NETWORKS  . . . . . . . . . . . . . . . . . . . .    1

2.5.1     IEEE 802.2 LOGICAL LINK CONTROL  . . . . . . . . .    1

2.5.2     IEEE 802.3 CSMA/CD ACCESS METHOD . . . . . . . . .    2

2.5.3     IEEE 802.4 TOKEN BUS ACCESS METHOD . . . . . . . .    3

2.5.4     IEEE 802.5 TOKEN RING ACCESS METHOD  . . . . . . .    3

2.6  WIDE AREA NETWORKS . . . . . . . . . . . . . . . . . . . . .    5

2.6.1     CCITT RECOMMENDATION X.25  . . . . . . . . . . . .    5

3. NETWORK LAYER . . . . . . . . . . . . . . . . . . . . . . . . . .    1

3.1  INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . . .    1

3.2  SCOPE AND FIELD OF APPLICATION . . . . . . . . . . . . . . .    1

3.3  STATUS . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

3.4  ERRATA . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

3.5  CONNECTIONLESS-MODE NETWORK SERVICE (CLNS) . . . . . . . . .    1

3.5.1     Provision of CLNS over Local . . . . . . . . . . .    1

3.5.2     Provision of CLNS over X.25 Subnetworks  . . . . .    1

3.5.3     Agreements on Protocols  . . . . . . . . . . . . .    2

3.5.3.1   ISO 8473  . . . . . . . . . . . . . . . . . .    2

3.5.3.2   Subnetwork Dependent Convergence Function . .    3

3.6  CONNECTION-MODE NETWORK SERVICE (CONS) . . . . . . . . . . .    3

3.6.1     Mandatory Method of Providing CONS . . . . . . . .    3

3.6.2     Additional Option:  Provision of CONS over X.25 1980

Subnetworks  . . . . . . . . . . . . . . . . . . .    3

3.6.3     Agreements on Protocols  . . . . . . . . . . . . .    4

3.6.3.1   ISO 8878  . . . . . . . . . . . . . . . . . .    4

3.6.3.2   Subnetwork Dependent Convergence Protocol (ISO

8878/Annex A) . . . . . . . . . . . . . . . .    4

3.7  ADDRESSING . . . . . . . . . . . . . . . . . . . . . . . . .    4

3.8  ROUTING  . . . . . . . . . . . . . . . . . . . . . . . . . .    5

3.8.1     Static Routing . . . . . . . . . . . . . . . . . .    5

3.8.2     End System to Intermediate System  . . . . . . . .    5

3.9  MIGRATION CONSIDERATIONS . . . . . . . . . . . . . . . . . .    6

3.9.1     X.25-1980  . . . . . . . . . . . . . . . . . . . .    6

3.10 CONFORMANCE  . . . . . . . . . . . . . . . . . . . . . . . .    7

4. TRANSPORT . . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

4.1  INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . . .    1

4.2  SCOPE AND FIELD OF APPLICATION . . . . . . . . . . . . . . .    1

4.3  STATUS . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

4.4  ERRATA . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

4.5  TRANSPORT CLASS 4  . . . . . . . . . . . . . . . . . . . . .    1

4.5.1     Transport Class  . . . . . . . . . . . . . . . . .    1

4.5.2     Protocol Agreements  . . . . . . . . . . . . . . .    1

4.5.2.1   Rules for Negotiation . . . . . . . . . . . .    1

4.5.2.2   Transport Class 4 Service Access Points or

Selectors . . . . . . . . . . . . . . . . . .    3

4.5.2.3   Retransmission Timer  . . . . . . . . . . . .    3

4.5.2.4   Keep-Alive Function . . . . . . . . . . . . .    4

4.6  TRANSPORT CLASS 0  . . . . . . . . . . . . . . . . . . . . .    6

4.6.1     Transport Class 0 Overview . . . . . . . . . . . .    6

4.6.2     Protocol Agreements  . . . . . . . . . . . . . . .    6

4.6.2.1   Transport Class 0 Service Access Points . . .    7

4.6.3     Rules for Negotiation  . . . . . . . . . . . . . .    7

5. UPPER LAYERS  . . . . . . . . . . . . . . . . . . . . . . . . . .    1

5.1  INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . . .    1

5.1.1     References . . . . . . . . . . . . . . . . . . . .    1

5.2  SCOPE AND FIELD OF APPLICATION . . . . . . . . . . . . . . .    1

5.3  STATUS . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

5.4  ERRATA . . . . . . . . . . . . . . . . . . . . . . . . . . .    2

5.4.1     ISO Defect Reports . . . . . . . . . . . . . . . .    2

5.4.2     Session Defects  . . . . . . . . . . . . . . . . .    2

5.5  ASSOCIATION CONTROL SERVICE ELEMENT  . . . . . . . . . . . .    2

5.5.1     Introduction . . . . . . . . . . . . . . . . . . .    2

5.5.2     Services . . . . . . . . . . . . . . . . . . . . .    3

5.5.2.1   ACSE Services . . . . . . . . . . . . . . . .    3

5.5.2.2   Use of Presentation Layer Services  . . . . .    3

5.5.3     Protocol agreements  . . . . . . . . . . . . . . .    3

5.5.3.1   Application Context . . . . . . . . . . . . .    3

5.5.3.2   Section Deleted . . . . . . . . . . . . . . .    4

5.5.3.3   AE Title  . . . . . . . . . . . . . . . . . .    4

5.6  PRESENTATION . . . . . . . . . . . . . . . . . . . . . . . .    4

5.6.1     Introduction . . . . . . . . . . . . . . . . . . .    4

5.6.2     Services . . . . . . . . . . . . . . . . . . . . .    5

5.6.2.1   Presentation Services . . . . . . . . . . . .    5

5.6.2.2   Use of Session Layer Services . . . . . . . .    6

5.6.3     Protocol Agreements  . . . . . . . . . . . . . . .    6

5.6.3.1   Transfer Syntaxes . . . . . . . . . . . . . .    6

5.6.3.2   Abstract Syntaxes . . . . . . . . . . . . . .    6

5.6.3.3   Presentation Context Identifier . . . . . . .    6

5.6.3.4   Mode-selector Position in SET . . . . . . . .    7

5.6.3.5   Default Context . . . . . . . . . . . . . . .    7

5.6.3.6   P-Selectors . . . . . . . . . . . . . . . . .    7

5.6.3.7   Provider Abort Parameters . . . . . . . . . .    7

5.6.3.8   Provider Aborts and Session Version . . . . .    7

5.6.3.9   CPC-type  . . . . . . . . . . . . . . . . . .    7

5.6.3.10  Presentation-context-definition-result-list .    8

5.6.3.11  RS-PPDU . . . . . . . . . . . . . . . . . . .    8

5.6.4     Presentation ASN.1 Encoding Rules  . . . . . . . .    8

5.6.4.1   Invalid Encoding  . . . . . . . . . . . . . .    8

5.6.4.2   Protocol-version, Presentation-requirements .    8

5.6.4.3   Presentation-selector . . . . . . . . . . . .    8

5.7  SESSION  . . . . . . . . . . . . . . . . . . . . . . . . . .    8

5.7.1     Introduction . . . . . . . . . . . . . . . . . . .    8

5.7.2     Services . . . . . . . . . . . . . . . . . . . . .    9

5.7.2.1   Session Services  . . . . . . . . . . . . . .    9

5.7.2.2   Use of Transport Services . . . . . . . . . .    9

5.7.3     Protocol Agreements  . . . . . . . . . . . . . . .    9

5.7.3.1   Concatenation . . . . . . . . . . . . . . . .    9

5.7.3.2   Segmenting  . . . . . . . . . . . . . . . . .   10

5.7.3.3   Reuse of Transport Connection . . . . . . . .   10

5.7.3.4   Use of Transport Expedited Data . . . . . . .   10

5.7.3.5   Use of Session Version Number . . . . . . . .   10

5.7.3.6   Receipt of Invalid SPDUs  . . . . . . . . . .   11

5.7.3.7   Invalid SPM Intersections . . . . . . . . . .   11

5.7.3.8   S-Selectors . . . . . . . . . . . . . . . . .   11

5.8  UNIVERSAL ASN.1 ENCODING RULES . . . . . . . . . . . . . . .   11

5.8.1     Tags . . . . . . . . . . . . . . . . . . . . . . .   12

5.8.2     Definite length  . . . . . . . . . . . . . . . . .   12

5.8.3     EXTERNAL Type  . . . . . . . . . . . . . . . . . .   12

5.9  CONFORMANCE  . . . . . . . . . . . . . . . . . . . . . . . .   12

5.9.1     Specific ASE Requirements for ACSE Presentation and

Session  . . . . . . . . . . . . . . . . . . . . .   12

5.9.1.1   FTAM  . . . . . . . . . . . . . . . . . . . .   13

5.9.1.1.1 Phase 2  . . . . . . . . . . . . . . . .   13

5.9.1.2   MHS . . . . . . . . . . . . . . . . . . . . .   15

5.9.1.2.1 Phase 1  . . . . . . . . . . . . . . . .   15

5.9.1.3   DS  . . . . . . . . . . . . . . . . . . . . .   16

5.9.1.3.1 Phase 1  . . . . . . . . . . . . . . . .   16

5.10 APPENDIX A:  RECOMMENDED PRACTICES . . . . . . . . . . . . .   17

6. ISO FILE TRANSFER, ACCESS AND MANAGEMENT PHASE 2  . . . . . . . .    1

6.1  INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . . .    1

6.2  SCOPE AND FIELD OF APPLICATION . . . . . . . . . . . . . . .    1

6.3  STATUS . . . . . . . . . . . . . . . . . . . . . . . . . . .    2

6.4  ERRATA . . . . . . . . . . . . . . . . . . . . . . . . . . .    3

6.5  ASSUMPTIONS  . . . . . . . . . . . . . . . . . . . . . . . .    4

6.6  PRESENTATION AGREEMENTS  . . . . . . . . . . . . . . . . . .    5

6.7  SERVICE CLASS AGREEMENTS . . . . . . . . . . . . . . . . . .    6

6.8  FUNCTIONAL UNIT AGREEMENTS . . . . . . . . . . . . . . . . .    6

6.9  FILE ATTRIBUTE AGREEMENTS  . . . . . . . . . . . . . . . . .    6

6.9.1     Mandatory Group  . . . . . . . . . . . . . . . . .    7

6.9.2     Optional Groups  . . . . . . . . . . . . . . . . .    7

6.10 DOCUMENT TYPE AGREEMENTS . . . . . . . . . . . . . . . . . .    8

6.10.1    Character Sets . . . . . . . . . . . . . . . . . .   11

6.10.1.1  IA5 Character Set . . . . . . . . . . . . . .   12

6.10.1.2  8859-1 Character Set  . . . . . . . . . . . .   14

6.10.2    Document Type Negotiation Rules  . . . . . . . . .   14

6.10.2.1  Connection Establishment  . . . . . . . . . .   14

6.10.2.2  File Creation . . . . . . . . . . . . . . . .   14

6.10.2.3  File Opening  . . . . . . . . . . . . . . . .   14

6.10.3    Relationship Between DUs, DEs and Document Types .   15

6.11 F-CANCEL ACTION  . . . . . . . . . . . . . . . . . . . . . .   16

6.12 IMPLEMENTATION INFORMATION AGREEMENTS  . . . . . . . . . . .   16

6.13 DIAGNOSTIC AGREEMENTS  . . . . . . . . . . . . . . . . . . .   16

6.14 CONCURRENCY  . . . . . . . . . . . . . . . . . . . . . . . .   18

6.15 REQUESTED ACCESS . . . . . . . . . . . . . . . . . . . . . .   18

6.16 SECURITY . . . . . . . . . . . . . . . . . . . . . . . . . .   19

6.16.1    Initiator Identity and Filestore Password    . . .   19

6.16.2    Access Passwords   . . . . . . . . . . . . . . . .   19

6.16.3    Implementation Responsibilities  . . . . . . . . .   19

6.17 REQUIREMENT FOR CONFORMANT IMPLEMENTATIONS . . . . . . . . .   19

6.17.1    Interoperable Configurations . . . . . . . . . . .   20

6.17.2    Relationship to ISO 8571--The FTAM Standard  . . .   21

6.17.3    Requirements for Document Type Support . . . . . .   21

6.17.4    Initiators . . . . . . . . . . . . . . . . . . . .   22

6.17.5    Responders . . . . . . . . . . . . . . . . . . . .   23

6.17.6    Senders  . . . . . . . . . . . . . . . . . . . . .   24

6.17.6.1  Initiator Senders . . . . . . . . . . . . . .   25

6.17.6.2  Responder Senders . . . . . . . . . . . . . .   25

6.17.7    Receivers  . . . . . . . . . . . . . . . . . . . .   25

6.17.7.1  Initiator Receivers . . . . . . . . . . . . .   26

6.17.7.2  Responder Receivers . . . . . . . . . . . . .   26

6.17.8    Minimum Ranges . . . . . . . . . . . . . . . . . .   26

6.17.9    Use of Lower Layer Services  . . . . . . . . . . .   29

6.18 IMPLEMENTATION PROFILES  . . . . . . . . . . . . . . . . . .   29

6.18.1    General Requirements for the Defined Implementation

Profiles . . . . . . . . . . . . . . . . . . . . .   30

6.18.2    Intentionally Left Empty . . . . . . . . . . . . .   30

6.18.3    Document Type Requirements for the Defined

Implementation Profiles  . . . . . . . . . . . . .   30

6.18.4    Parameters for the Defined Implementation Profiles   31

6.18.5    Parameter Ranges for the Defined Implementation

Profiles . . . . . . . . . . . . . . . . . . . . .   32

6.18.6    File Attribute Support for Implementations . . . .   32

6.19 PROVISION OF SPECIFIC FUNCTION . . . . . . . . . . . . . . .   35

6.19.1    Implementation Profile T1:  Simple File Transfer .   35

6.19.2    Implementation Profile T2:  Positional File Transfer  35

6.19.3    Implementation Profile T3:  Full File Transfer . .   36

6.19.4    Implementation Profile A1: Simple File Access  . .   37

6.19.5    Implementation Profile A2:  Full File Access . . .   37

6.19.6    Implementation Profile M1:  Management . . . . . .   38

6.20 HARMONIZATION  . . . . . . . . . . . . . . . . . . . . . . .   38

6.21 APPENDIX A:    FTAM DOCUMENT TYPES . . . . . . . . . . . . .   39

7. CCITT 1984 X.400 BASED MESSAGE HANDLING SYSTEM  . . . . . . . . .    1

7.1  INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . . .    1

7.2  SCOPE  . . . . . . . . . . . . . . . . . . . . . . . . . . .    2

7.3  STATUS . . . . . . . . . . . . . . . . . . . . . . . . . . .    3

7.4  ERRATA . . . . . . . . . . . . . . . . . . . . . . . . . . .    3

7.5  PRMD to PRMD . . . . . . . . . . . . . . . . . . . . . . . .    3

7.5.1     Introduction . . . . . . . . . . . . . . . . . . .    3

7.5.2     Service Elements and Optional User Facilities  . .    4

7.5.2.1   Classification of Support for Services  . . .    4

7.5.2.1.1 Support (S)  . . . . . . . . . . . . . .    5

7.5.2.1.2 Non Support (N)  . . . . . . . . . . . .    5

7.5.2.1.3 Not Used (N/U) . . . . . . . . . . . . .    6

7.5.2.1.4 Not Applicable (N/A) . . . . . . . . . .    6

7.5.2.2   Summary of Supported Services . . . . . . . .    6

7.5.2.3   MT Service Elements and Optional User

Facilities . . . . . . . . . . . . . . . . . . . .    6

7.5.2.4   IPM Service Elements and Optional User

Facilities . . . . . . . . . . . . . . . . . . . .    8

7.5.3     X.400 Protocol Definitions . . . . . . . . . . . .    9

7.5.3.1   Protocol Classification . . . . . . . . . . .   10

7.5.3.2   General Statements on Pragmatic Constraints .   10

7.5.3.3   MPDU Size . . . . . . . . . . . . . . . . . .   11

7.5.3.4   P1 Protocol Elements  . . . . . . . . . . . .   11

7.5.3.4.1 P1 Envelope Protocol Elements  . . . . .   11

7.5.3.5   ORName Protocol Elements  . . . . . . . . . .   16

7.5.3.6   P2 Protocol Profile (Based on [X.420])  . . .   18

7.5.3.6.1 P2 Protocol - Heading  . . . . . . . . .   19

7.5.3.6.2 P2 Protocol - BodyParts  . . . . . . . .   21

7.5.3.6.3 P2 BodyPart Protocol Elements  . . . . .   23

7.5.4     Reliable Transfer Server (RTS) . . . . . . . . . .   25

7.5.4.1   Implementation Strategy . . . . . . . . . . .   25

7.5.4.2   RTS option selection  . . . . . . . . . . . .   25

7.5.4.3   RTS Protocol Options and Clarifications . . .   26

7.5.4.4   RTS Protocol Limitations  . . . . . . . . . .   29

7.5.5     Use of Session Services  . . . . . . . . . . . . .   31

7.5.6     Data Transfer Syntax . . . . . . . . . . . . . . .   31

7.6  PRMD to ADMD and ADMD to ADMD  . . . . . . . . . . . . . . .   31

7.6.1     Introduction . . . . . . . . . . . . . . . . . . .   31

7.6.2     Additional ADMD Functionality  . . . . . . . . . .   33

7.6.2.1   Relay Responsibilities of an ADMD . . . . . .   33

7.6.2.2   P1 Protocol Classification Changes  . . . . .   34

7.6.2.3   O/R Names . . . . . . . . . . . . . . . . . .   34

7.6.2.4   P1 ADMD Name  . . . . . . . . . . . . . . . .   35

7.6.3     Interworking with Integrated UAs . . . . . . . . .   35

7.6.4     Differences with Other Profiles  . . . . . . . . .   35

7.6.4.1   TTC Profile . . . . . . . . . . . . . . . . .   35

7.6.4.2   CEPT Profile  . . . . . . . . . . . . . . . .   36

7.6.5     Connection of PRMDs to Multiple ADMDs  . . . . . .   36

7.6.6     Connection of an ADMD to a Routing PRMD  . . . . .   36

7.6.7     Management Domain Names  . . . . . . . . . . . . .   37

7.6.8     Envelope Validation Errors . . . . . . . . . . . .   37

7.6.9     Quality of Service . . . . . . . . . . . . . . . .   38

7.6.9.1   Domain Availability . . . . . . . . . . . . .   38

7.6.9.1.1 ADMD Availability  . . . . . . . . . . .   38

7.6.9.1.2 PRMD Availability  . . . . . . . . . . .   38

7.6.9.2   Delivery Times  . . . . . . . . . . . . . . .   38

7.6.10    Billing Information  . . . . . . . . . . . . . . .   39

7.6.11    Transparency . . . . . . . . . . . . . . . . . . .   39

7.6.12    RTS Password Management  . . . . . . . . . . . . .   40

7.6.13    For Further Study  . . . . . . . . . . . . . . . .   40

7.7  INTER and INTRA PRMD CONNECTIONS . . . . . . . . . . . . . .   40

7.7.1     Introduction . . . . . . . . . . . . . . . . . . .   40

7.7.2     The Relaying PRMD  . . . . . . . . . . . . . . . .   41

7.7.2.1   Relay Responsibilities of a PRMD  . . . . . .   41

7.7.2.2   Interaction with an ADMD  . . . . . . . . . .   41

7.7.3     Intra PRMD Connections . . . . . . . . . . . . . .   42

7.7.3.1   Relay Responsibilities of an MTA  . . . . . .   42

7.7.3.2   Loop Suppression within a PRMD  . . . . . . .   43

7.7.3.3   Routing Within a PRMD . . . . . . . . . . . .   44

7.7.3.3.1 Class Designations . . . . . . . . . . .   44

7.7.3.3.2 Specification of MTA Classes . . . . . .   46

7.7.3.3.3 Consequences of Using Certain Classes of

MTAs  . . . . . . . . . . . . . . . . . . . .   46

7.7.3.4   Uniqueness of MPDUidentifiers Within a PRMD .   47

7.7.4     Service Elements and Optional User Facilities  . .   47

7.7.5     X.400 Protocol Definitions . . . . . . . . . . . .   48

7.7.5.1   Protocol Classification . . . . . . . . . . .   48

7.7.5.2   P1 Protocol Elements  . . . . . . . . . . . .   48

7.7.5.3   Reliable Transfer Server (RTS)  . . . . . . .   51

7.8  ERROR HANDLING . . . . . . . . . . . . . . . . . . . . . . .   51

7.8.1     MPDU Encoding  . . . . . . . . . . . . . . . . . .   52

7.8.2     Contents . . . . . . . . . . . . . . . . . . . . .   52

7.8.3     Envelope . . . . . . . . . . . . . . . . . . . . .   52

7.8.3.1   Pragmatic Constraint Violations . . . . . . .   52

7.8.3.2   Protocol Violations . . . . . . . . . . . . .   52

7.8.3.3   O/R Names . . . . . . . . . . . . . . . . . .   52

7.8.3.4   TraceInformation  . . . . . . . . . . . . . .   53

7.8.3.5   InternalTraceInfo . . . . . . . . . . . . . .   54

7.8.3.6   Unsupported X.400 Protocol Elements . . . . .   54

7.8.3.6.1 deferredDelivery . . . . . . . . . . . .   54

7.8.3.6.2 PerDomainBilateralInfo . . . . . . . . .   55

7.8.3.6.3 ExplicitConversion . . . . . . . . . . .   55

7.8.3.6.4 alternateRecipientAllowed  . . . . . . .   55

7.8.3.6.5 contentReturnRequest . . . . . . . . . .   55

7.8.3.7   Unexpected Values for INTEGER Protocol Elements  55

7.8.3.7.1 Priority . . . . . . . . . . . . . . . .   55

7.8.3.7.2 ExplicitConversion . . . . . . . . . . .   56

7.8.3.7.3 ContentType  . . . . . . . . . . . . . .   56

7.8.3.8   Additional Elements . . . . . . . . . . . . .   56

7.8.4     Reports  . . . . . . . . . . . . . . . . . . . . .   56

7.9  MHS USE OF DIRECTORY SERVICES  . . . . . . . . . . . . . . .   56

7.9.1     Directory Service Elements . . . . . . . . . . . .   56

7.9.2     Use of Names and Addresses . . . . . . . . . . . .   57

7.10 CONFORMANCE  . . . . . . . . . . . . . . . . . . . . . . . .   58

7.10.1    Introduction . . . . . . . . . . . . . . . . . . .   58

7.10.2    Definition of Conformance  . . . . . . . . . . . .   58

7.10.3    Conformance Requirements . . . . . . . . . . . . .   60

7.10.3.1  Introduction  . . . . . . . . . . . . . . . .   60

7.10.3.2  Initial Conformance . . . . . . . . . . . . .   60

7.10.3.2.1     Interworking  . . . . . . . . . . .   61

7.10.3.2.2     Service . . . . . . . . . . . . . .   61

7.11 APPENDIX A:    INTERPRETATION OF X.400 SERVICE ELEMENTS  . .   62

7.12 APPENDIX B:    RECOMMENDED X.400 PRACTICES . . . . . . . . .   66

7.12.1    RECOMMENDED PRACTICES IN P2  . . . . . . . . . . .   66

7.12.2    RECOMMENDED PRACTICES IN RTS . . . . . . . . . . .   66

7.12.3    RECOMMENDED PRACTICES FOR ORName . . . . . . . . .   67

7.12.4    POSTAL ADDRESSING  . . . . . . . . . . . . . . . .   70

7.12.5    EDI use of X.400 . . . . . . . . . . . . . . . . .   71

7.12.5.1  Introduction and Scope  . . . . . . . . . . .   71

7.12.5.2  Model . . . . . . . . . . . . . . . . . . . .   71

7.12.5.3  Protocol Elements Supported for EDI . . . . .   73

7.12.5.4  Addressing and Routing  . . . . . . . . . . .   73

7.12.6    USA Body Parts . . . . . . . . . . . . . . . . . .   74

7.13 APPENDIX C:    RENDITION OF IA5Text AND T61String CHARACTERS   75

7.13.1    GENERATING AND IMAGING IA5Text . . . . . . . . . .   75

7.13.2    GENERATING AND IMAGING T61String . . . . . . . . .   75

7.14 APPENDIX D:    DIFFERENCES IN INTERPRETATION DISCOVERED

THROUGH TESTING OF THE MHS FOR THE CeBit 87

DEMONSTRATION . . . . . . . . . . . . . . . .   76

7.14.1    ENCODING OF RTS USER DATA  . . . . . . . . . . . .   76

7.14.2    EXTRA SESSION FUNCTIONAL UNITS . . . . . . . . . .   76

7.14.3    MIXED CASE IN THE MTA NAME . . . . . . . . . . . .   77

7.14.4    X.410 ACTIVITY IDENTIFIER  . . . . . . . . . . . .   77

7.14.5    ENCODING OF PER RECIPIENT FLAG AND PER MESSAGE FLAG  77

7.14.6    ENCODING OF EMPTY BITSTRINGS . . . . . . . . . . .   78

7.14.7    ADDITIONAL OCTETS FOR BITSTRINGS . . . . . . . . .   78

7.14.8    APPLICATION PROTOCOL IDENTIFIER  . . . . . . . . .   78

7.14.9    INITIAL SERIAL NUMBER IN S-CONNECT . . . . . . . .   78

7.14.10   CONNECTION DATA ON RTS RECOVERY  . . . . . . . . .   78

7.14.11   ACTIVITY RESUME  . . . . . . . . . . . . . . . . .   78

7.14.12   OLD ACTIVITY IDENTIFIER  . . . . . . . . . . . . .   79

7.14.13   NEGOTIATION DOWN TO TRANSPORT CLASS 0  . . . . . .   79

7.15 APPENDIX E:    WORLDWIDE X.400 CONFORMANCE PROFILE MATRIX  .   80

7.16 APPENDIX F:    INTERWORKING WARNINGS . . . . . . . . . . . .   91

8. DIRECTORY SERVICES PROTOCOLS  . . . . . . . . . . . . . . . . . .    1

8.1  INTRODUCTION . . . . . . . . . . . . . . . . . . . . . . . .    1

8.2  SCOPE AND FIELD OF APPLICATION . . . . . . . . . . . . . . .    2

8.3  STATUS     . . . . . . . . . . . . . . . . . . . . . . . . .    3

8.4  USE OF DIRECTORIES . . . . . . . . . . . . . . . . . . . . .    3

8.5  DIRECTORY ASEs, APPLICATION CONTEXTS, AND PORTS  . . . . . .    4

8.6  SCHEMAS  . . . . . . . . . . . . . . . . . . . . . . . . . .    5

8.6.1     Maintenance of Structures and Naming Rules . . . .    5

8.6.2     Maintenance of object classes and subclasses . . .    6

8.6.3     Maintenance of Attribute Types . . . . . . . . . .    6

8.6.4     Maintenance of Attribute Syntaxes  . . . . . . . .    6

8.7  CLASSIFICATION OF SUPPORT FOR ATTRIBUTE TYPES  . . . . . . .    6

8.7.1     Mandatory Support  . . . . . . . . . . . . . . . .    7

8.7.2     Optional Support . . . . . . . . . . . . . . . . .    7

8.8  INTORDUCTION TO PRAGMETIC CONSTRAINTS  . . . . . . . . . . .    8

8.9  GENERAL CONSTRAINTS  . . . . . . . . . . . . . . . . . . . .    8

8.9.1     Character Sets . . . . . . . . . . . . . . . . . .    8

8.9.2     APDU Size Considerations . . . . . . . . . . . . .    8

8.9.3     Service Control (SC) Considerations  . . . . . . .    9

8.9.4     Priority Service Control . . . . . . . . . . . . .    9

8.10 CONSTRAINTS ON OPERATIONS  . . . . . . . . . . . . . . . . .   10

8.10.1    Filters  . . . . . . . . . . . . . . . . . . . . .   10

8.10.2    Errors . . . . . . . . . . . . . . . . . . . . . .   10

8.11 CONSTRAINTS ON ATTRIBUTE TYPES . . . . . . . . . . . . . . .   10

8.11.1    Attribute Values . . . . . . . . . . . . . . . . .   10

8.12 CONFORMANCE  . . . . . . . . . . . . . . . . . . . . . . . .   15

8.12.1    DUA Conformance  . . . . . . . . . . . . . . . . .   15

8.12.2    DSA Conformance  . . . . . . . . . . . . . . . . .   16

8.12.3    Directory Systems Conformance Classes  . . . . . .   17

8.12.4    Authentication Conformance . . . . . . . . . . . .   17

8.12.5    Authentication Conformance Classes . . . . . . . .   18

8.13 DISTRIBUTED OPERATIONS . . . . . . . . . . . . . . . . . . .   18

8.13.1    Referrals and Chaining . . . . . . . . . . . . . .   19

8.14 UNDERLYING SERVICES  . . . . . . . . . . . . . . . . . . . .   19

8.14.1    ROSE . . . . . . . . . . . . . . . . . . . . . . .   19

8.14.2    Session  . . . . . . . . . . . . . . . . . . . . .   19

8.15 ACCESS CONTROL . . . . . . . . . . . . . . . . . . . . . . .   19

8.16 AUTHENTICATION . . . . . . . . . . . . . . . . . . . . . . .   19

8.17 TEST CONSIDERATIONS  . . . . . . . . . . . . . . . . . . . .   20

8.17.1    Major elements of Architecture . . . . . . . . . .   20

8.17.2    Search Operation . . . . . . . . . . . . . . . . .   21

8.18 ERRORS . . . . . . . . . . . . . . . . . . . . . . . . . . .   21

8.18.1    Permanent vs. Temporary Service Errors . . . . . .   21

8.19 APPENDIX A     Definitions . . . . . . . . . . . . . . . . .   22

8.20 APPENDIX B     Attributes and Object Classes . . . . . . . .   23

8.21 APPENDIX C     The Use of ROSE . . . . . . . . . . . . . . .   28

8.22 APPENDIX D     Guidelines  . . . . . . . . . . . . . . . . .   29

8.23 APPENDIX E     Glossary  . . . . . . . . . . . . . . . . . .   30

9. SECURITY  . . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

9.1  Definitions  . . . . . . . . . . . . . . . . . . . . . . . .    1

9.2  Matrix of Security Services and OSI Layers . . . . . . . . .    2

10.     OBJECT IDENTIFIER: STRUCTURE AND ALLOCATION  . . . . . . . .    1

10.1 Specific ASE Requirements for ACSE Presentation and Session     5

10.1.1    FTAM . . . . . . . . . . . . . . . . . . . . . . .    5

10.1.1.1  Phase 2 . . . . . . . . . . . . . . . . . . .    5

10.1.2    MHS  . . . . . . . . . . . . . . . . . . . . . . .    8

10.1.2.1  Phase 1 . . . . . . . . . . . . . . . . . . .    8

10.1.3    DS . . . . . . . . . . . . . . . . . . . . . . . .    9

10.1.3.1  Phase 1 . . . . . . . . . . . . . . . . . . .    9

11.     REFERENCES . . . . . . . . . . . . . . . . . . . . . . . . .    1

11.1 CCITT  . . . . . . . . . . . . . . . . . . . . . . . . . . .    1

11.2 ISO  . . . . . . . . . . . . . . . . . . . . . . . . . . . .    2

11.3 IEEE . . . . . . . . . . . . . . . . . . . . . . . . . . . .    8

11.4 NBS  . . . . . . . . . . . . . . . . . . . . . . . . . . . .    9

11.5 MAP  . . . . . . . . . . . . . . . . . . . . . . . . . . . .   10

11.6 TOP  . . . . . . . . . . . . . . . . . . . . . . . . . . . .   10

11.7 CEN/CENELEC  . . . . . . . . . . . . . . . . . . . . . . . .   10

11.8 SPAG . . . . . . . . . . . . . . . . . . . . . . . . . . . .   10

List of Figures

Figure 2.1  LSAP bit pattern   . . . . . . . . . . . . . . . . . . .    1

Figure 2.2  I-Field Format . . . . . . . . . . . . . . . . . . . . .    4

Figure 4.1  AK exchange on idleconnection  . . . . . . . . . . . . .    6

Figure 6.1  Model of file transfer/access  . . . . . . . . . . . . .    2

Figure 7.1  The layered structure of this implementation agreement .    1

Figure 7.2  This agreement applies to the interface between:  (A) PRMD

and PRMD; (B) PRMD and ADMD; (C) ADMD and ADMD; and  (D)

MTA and MTA  . . . . . . . . . . . . . . . . . . . . . . .    3

Figure 7.3  Interconnection of private domains . . . . . . . . . . .    4

Figure 7.4  X.409 Definition of Privately Defined BodyParts  . . . .   22

Figure 7.5  An ADMD may (b) or may not (a) serve as a relay  . . . .   33

Figure 7.6  Relaying PRMD  . . . . . . . . . . . . . . . . . . . . .   41

Figure 7.7  Intra PRMD connections . . . . . . . . . . . . . . . . .   42

Figure 7.8  MD C must know of A to route the message . . . . . . . .   42

Figure 7.9  Definition of InternalTraceInfo  . . . . . . . . . . . .   43

Figure 7.10 Defined Actions in MTASuppliedInfo . . . . . . . . . . .   44

Figure 7.11 Example of a confirguration to be avoided  . . . . . . .   47

Figure 8.1  Structure of this Implementation Agreement . . . . . . .    1

Figure 8.2  Centralized Directory Model  . . . . . . . . . . . . . .    2

Figure 8.3  Distributed Directory Model  . . . . . . . . . . . . . .    3

Figure 8.4  Access to the Directory  . . . . . . . . . . . . . . . .    5

Figure 8.5  APDU Exchange  . . . . . . . . . . . . . . . . . . . . .    8

Figure 8.6  Logical DSA Application Environment  . . . . . . . . . .    9

List of Tables

Table 5.1  Session States  . . . . . . . . . . . . . . . . . . . . .   18

Table 5.2  Incoming Events . . . . . . . . . . . . . . . . . . . . .   19

Table 6.1  Parameters for FTAM-1, -2, -3 . . . . . . . . . . . . . .    8

Table 6.2  Parameters for NBS-6, NBS-7, NBS-8  . . . . . . . . . . .   10

Table 6.3  FTAM primitive data types . . . . . . . . . . . . . . . .   11

Table 6.4  IRV Graphic Character Allocations . . . . . . . . . . . .   13

Table 6.5  Interoperable configurations  . . . . . . . . . . . . . .   21

Table 6.6  Required minimal parameter support  . . . . . . . . . . .   27

Table 6.7  Implementation profile support requirements . . . . . . .   34

Table 6.8  Implementation Profiles (NBS) and Profiles (SPAG/CEN-CLC)   38

Table 6.9  Information objects in NBS-6  . . . . . . . . . . . . . .   41

Table 6.10 Information objects in NBS-7  . . . . . . . . . . . . . .   46

Table 6.11 Information objects in NBS-8  . . . . . . . . . . . . . .   50

Table 6.12 Datatypes for keys  . . . . . . . . . . . . . . . . . . .   52

Table 6.12 Information objects in NBS-9  . . . . . . . . . . . . . .   56

Table 6.14 Basic constraints for NBS Ordered flat  . . . . . . . . .   60

Table 6.15 Identity constraints in NBS Ordered flat  . . . . . . . .   61

Table 7.1   Basic MT service elements . . . . . . . . . . . . . . .    6

Table 7.2   MT optional user facilities provided to the UA-selectable on

a per-message basis  . . . . . . . . . . . . . . . . . . .    7

Table 7.3  MT optional user facilities provided to the UA agreed for

any contractual period of time . . . . . . . . . . . . . .    7

Table 7.4   Basic IPM service elements . . . . . . . . . . . . . . .    8

Table 7.5  IPM optional facilities agreed for a contractual period of

time . . . . . . . . . . . . . . . . . . . . . . . . . . .    8

Table 7.6   IPM optional user facilities selectable on a per-message

basis  . . . . . . . . . . . . . . . . . . . . . . . . . .    9

Table 7.7   Protocol Classifications . . . . . . . . . . . . . . . .   10

Table 7.8   P1 protocol elements . . . . . . . . . . . . . . . . . .   12

Table 7.9   ORName protocol elements . . . . . . . . . . . . . . . .   17

Table 7.10  P2 heading protocol elements . . . . . . . . . . . . . .   19

Table 7.11  P2 BodyParts . . . . . . . . . . . . . . . . . . . . . .   23

Table 7.12  Checkpoint window size of IP . . . . . . . . . . . . . .   29

Table 7.13  RTS protocol elements  . . . . . . . . . . . . . . . . .   30

Table 7.14  P1 Protocol Classification Changes for a Delivering ADMD   34

Table 7.15  Delivery Time Targets  . . . . . . . . . . . . . . . . .   38

Table 7.16  Forced Nondelivery Times . . . . . . . . . . . . . . . .   39

Table 7.17  Conformant MTA Classifications . . . . . . . . . . . . .   45

Table 7.18  P1 Protocol Elements . . . . . . . . . . . . . . . . . .   49

Table 7B.1  Printable string to ASCII mapping  . . . . . . . . . . .   69

Table 7E.1  Protocol element comparison of RTS . . . . . . . . . . .   81

Table 7E.2  Protocol element comparison of P1  . . . . . . . . . . .   83

Table 7E.3  Protocol element comparison of P2  . . . . . . . . . . .   88

Table 8.1  Pragmatic Constraints for Selected Attributes.  . . . . .   12

Table 9.1  OSI Layers Desirable for Placing Security . . . . . . . .    3

1. GENERAL INFORMATION

1.1  PURPOSE OF THIS DOCUMENT

This document records stable implementation agreements of OSI

protocols among the organizations participating in the NBS Workshop

for Implementors of OSI.  Stable in the context of this document

means that:

1)   The agreements are based on final standards

(e.g., ISO-IS or CCITT Recommendations) or

nearly final (e.g., ISO-DIS) with no

significant changes expected, and,

2)   The agreements have been approved by the NBS

Workshop Plenary for progression from the "On-

Going" agreements document to the draft stable

agreements document and after a period of

review have been passed to these final stable

agreements.  The only changes allowed will be

clarifications, errata and the correction of

omissions discovered in their implementation.

For these reasons, the agreements are considered advanced enough for

use in product and test suite development, as well as for procurement

references.

Future releases of these Stable Agreements will add and/or extend

functionality offered by this version.  When required, new versions

will be introduced on a yearly basis.  It is the NBS Workshop intent

that new versions of this Stable Agreements document will be

compatible with the present version.  If this proves impractical, the

agreements will attempt to provide mechanisms and guidelines which

maximize interoperability.

1.2  PURPOSE OF THE WORKSHOP

In February, 1983, at the request of industry, NBS organized the NBS

Workshop for Implementors of OSI to bring together future users and

potential suppliers of OSI protocols.  The workshop accepts as input

the specifications of emerging standards for protocols and produces as

output agreements on the implementation and testing particulars of

these protocols.  This process is expected to expedite the development

of OSI protocols and promote interoperability of independently

manufactured data communications equipment.

1.3  WORKSHOP ORGANIZATION

The Workshop organizes its work through Special Interest Groups

(SIGs) that prepare technical documentation.  An executive committee

of SIG chairpersons led by the overall Workshop chairperson

administers the workshop.  NBS invites highly qualified technical

leaders from participating organizations to assume leadership roles in

the SIGs.  The SIGs are encouraged to coordinate with standards

organizations and user groups, and to seek widespread technical

consensus on implementation agreements through international

discussions and liaison activities.

The Workshop meets four times a year at the National Bureau of

Standards in Gaithersburg, Maryland where each SIG is required to

convene its meeting.  In addition, a plenary assembly of all workshop

delegates is convened for consideration of SIG motions and other

workshop business.  SIGs are also encouraged to hold interim meetings

at varied locations around the world.

The Workshop is an open public forum.  Registration materials,

documents, and workshop schedules are available from:

National Bureau of Standards

NBS Workshop for Implementors of OSI

Building 225, Room B-217

Gaithersburg, Maryland 20899

2. SUB NETWORKS

2.1  INTRODUCTION

This chapter provides agreements about subnetwork services used in

providing the OSI Network Layer.

2.2  SCOPE AND FIELD OF APPLICATION

These agreements cover subnetwork types including local area networks,

packet switched networks, circuit switched networks, ISDN, and others.

2.3  STATUS

This version was completed on December 18, 1987.

Ongoing work for other agreements and for future versions of these

agreements is contained in the "On-Going" agreements document,

including provisions for ISDN.

2.4  ERRATA

2.5  LOCAL AREA NETWORKS

2.5.1     IEEE 802.2 LOGICAL LINK CONTROL

The following decisions have been reached with respect to this

protocol.

l.   Link Service Access Point (LSAP)

The IEEE 802 committee has assigned the code below to

address systems using any ISO network layer protocol.  Note

that bit zero is transmitted first.

The most significant bit is bit 7, thus this bit pattern

represents hexadecimal FE.

0   l   2   3   4   5   6   7

ZDDDBDDDBDDDBDDDBDDDBDDDBDDDBDDD?

3 0 3 l 3 l 3 l 3 l 3 l 3 l 3 1 3

@DDDADDDADDDADDDADDDADDDADDDADDDY

Figure 2.1  LSAP bit pattern

2.   Type and Class

Only the connectionless type l, class l IEEE 802 link

service will be used.

2.5.2     IEEE 802.3 CSMA/CD ACCESS METHOD

The following implementation agreements have been reached with

respect to the IEEE 802.3 CSMA/CD Access Method and Physical

Layer Specifications: for 10 BASE 5:

o    The address length shall be 48 bits

The following implementation agreements have been reached with

respect to 10 BROAD 36 Networks:

1.   Single Cable Networks

-    The translator frequency shall be 192.25 Mhz

-    The channel allocations are

Reverse Channels              Forward Channels

T12, T13, T14                 L, M, N

T13, T14, 2'                  M, N, O

T14, 2', 3'                   N, O, P

2', 3', 4'                    O, P, Q

3', 4', 4A'                   P, Q, R

4', 4A', 5'                   Q, R, S

2.   Dual Cable Networks

For nontranslated dual cable networks forward and

reverse frequencies are the same.  Permissible

channel allocations are:

T12, T13, T14

T13, T14, 2'

T14, 2', 3'

2', 3', 4'

3', 4', 4A'

4', 4A', 5'

L, M, N

M, N, O

N, O, P

O, P, Q

Q, R, S

3.   When both IEEE 802.4 and IEEE 802.3 10 BROAD 36

networks coexist on the broadband cable system the

preferred channel allocations are:

Reverse             Forward

IEEE 802.3          T12, T13, T14       L, M, N

IEEE 802.4          6', FM1'            T, U

channels            3', 4'              P, Q

reserved for        4A', 5'             R, S

future use

2.5.3     IEEE 802.4 TOKEN BUS ACCESS METHOD

The following options are agreed to with respect to Draft J of

token bus:

o    Data Rate:

-    10 Mb (Broadband)

-    5 Mb (carrierband)

o    Addressing: 48 bit

o    The lmeOption, Priority Mechanism, shall be implemented

o    Broadband Channel Assignments

Forward                       Reverse

P                             3'

Q                             4'

R                             4A'

S                             5'

T                             6'

U                             FM1'

2.5.4     IEEE 802.5 TOKEN RING ACCESS METHOD

The following implementation agreements have been reached with

respect to the IEEE Standard 802.5, Token Passing Ring Access

Method and Physical Layer specification.

o    The data signalling rate shall be 4 Mbit/s

o    The address length shall be 48 bits

o    The message priority (PM) of the AMP data unit shall be

7

o    The ALL_STATIONS_THIS_RING_ADDRESS shall be

X'COOOFFFFFFFF'

o    The TRR value shall be 4 milliseconds

o    The THT value shall be 8.9 milliseconds

o    The TQP value shall be 20 milliseconds

o    The TVX value shall be 10 milliseconds

o    The TNT value shall be 2.6 milliseconds

o    The TAM value shall be 7 seconds

o    The TSM value shall be 15 seconds

o    The MAC Information field (I-field) shall be defined as

follows:

ZDDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDDDD?

3 Starting Sequence 3    I-Field       3  End Sequence  3

@DDDDDDDDDDDDDDDDDDDADDDDDDDDDDDDDDDDDDADDDDDDDDDDDDDDDDY

and the:

1)   Starting Sequence includes: SD, AC, FC, DA, SA

2)   Ending Sequence includes: FCS, ED, FS

Figure 2.2  I-Field Format

o    With the above timer and MAC I-field definitions, the

following limits are defined:

-    Protocol limits the I-field to a maximum of 4425

bytes.

-    All stations shall support I-fields that have a

minimum of one byte and a maximum of at least 2000

bytes.

2.6  WIDE AREA NETWORKS

2.6.1     CCITT RECOMMENDATION X.25

The procedures required to describe the DTE/DTE interface and the

DTE side of the DTE/DCE interface, for systems attached to

subnetworks providing an X.25 interface shall be as defined in

ISO 7776 and ISO 8208.

3. NETWORK LAYER

3.1  INTRODUCTION

This chapter presents agreements for providing the OSI network

service.  Also contained here are agreements on network layer

addressing and routing.

3.2  SCOPE AND FIELD OF APPLICATION

These agreements cover both connectionless-mode and connection-mode

network services.

3.3  STATUS

This version of the agreements was completed on December 18, 1987.

Ongoing work for other agreements and for future versions of these

agreements is contained in the "On-Going" agreements document,

including provisions for ISDN, and including specific aspects of using

the X.25 PLP over LANs.

3.4  ERRATA

3.5  CONNECTIONLESS-MODE NETWORK SERVICE (CLNS)

3.5.1     Provision of CLNS over Local Area Networks (LANS)

When providing CLNS over a LAN subnetwork, the following shall

apply:

1.   The definition of CLNS shall be as specified in ISO

8348/AD1.

2.   The protocol used to provide CLNS shall be ISO 8473

with agreements as specified in 3.5.3.1.

3.5.2     Provision of CLNS over X.25 Subnetworks

When providing CLNS over X.25 subnetworks, the following shall

apply:

1.   The definition of CLNS shall be as specified in ISO

8348/AD1.

2.   The protocol used to provide CLNS shall be ISO 8473

with agreements as specified in 3.5.3.1.

3.   The necessary subnetwork dependant convergence function

shall be as defined in ISO 8473 for operation over X.25

subnetworks, with agreements as specified in 3.5.3.2.

4.   The X.25 PLP shall be as defined in ISO 8208.

3.5.3     Agreements on Protocols

3.5.3.1   ISO 8473

1. Subsets of the protocol:

o    Implementations will not transmit PDUs encoded

using the inactive subset. Received PDUs encoded

using the inactive subset will be discarded.

o    The non-segmenting subset will not be used.

Implementations will not generate data PDUs

without a segmentation part.  However,

implementations will receive and correctly process

PDUs which do not contain the segmentation part.

2. Mandatory Functions:

o    The lifetime parameter shall be used as specified

in section 6.4 of ISO 8473.  The parameter shall

have an initial value of at least three times the

network span or  three times the maximum transit

delay (in units of 500 milliseconds), whichever is

greater.

o    The reassembly timer for an initial PDU at the

reassembly point shall be no greater than the

largest value of all lifetime parameters contained

in all derived PDUs.

3. Optional Functions:

o    The Security parameter is not defined by these

Agreements.  Implementations shall not transmit

the parameter except where defined by bilateral

agreements.

o    Partial and complete source Routing will not be

supported.

o    Partial record of Route will be supported by

Intermediate systems.

o    ISO 8473 will be followed with respect to QOS.

3.5.3.2   Subnetwork Dependent Convergence Function

1.   When 8473 SNDC functions are used over X.25

subnetworks, the default throughput class shall be used

if this facility is available.

3.6  CONNECTION-MODE NETWORK SERVICE (CONS)

The following agreements concern provision of the connection-mode

Network Service.

3.6.1     Mandatory Method of Providing CONS

When providing CONS using X.25 1984, the following shall apply:

o    The definition of the CONS is as specified in ISO 8348,

Network Service Definition.

o    The mapping of the elements of the CONS to the elements of

the X.25 Packet Level Protocol (PLP) is as specified in ISO

8878, Use of X.25 to Provide the Connection-mode Network

Service.

o    The general procedures and formats of the X.25 PLP are as

specified in ISO 8208, X.25 Packet Level Protocol for Data

Terminal Equipment.

3.6.2     Additional Option:  Provision of CONS over X.25 1980

Subnetworks

When providing CONS over an X.25 1980 subnetwork, the following

shall apply:

o    The definition of the CONS is as specified in ISO 8348,

Network Service Definition.

o    The subnetwork dependant convergence protocol required

to provide CONS shall be as specified in ISO 8878 Annex

A, and referred to as the Alternative Procedures for

Network Connection Establishment and Release, with

agreements as defined in 3.6.3.2.

3.6.3     Agreements on Protocols

3.6.3.1   ISO 8878

o    The Receipt Confirmation service will not be provided,

so the corresponding protocol elements need not be

implemented.

o    The Expedited Data service will not be provided, so the

corresponding protocol elements need not be

implemented.

o    Where the ISO 8208 diagnostic codes are not provided,

all  Cause/Diagnostic code combinations can be mapped

to the Originator/Reason code of "Undefined".

3.6.3.2   Subnetwork Dependent Convergence Protocol (ISO

8878/Annex A)

o    The Receipt Confirmation service will not be provided,

so the corresponding protocol elements need not be

implemented.

o    The Expedited Data service will not be provided, so the

corresponding protocol elements need not be

implemented.

3.7  ADDRESSING

Address formats supported will conform to ISO 8348 DAD2.

o    NSAP address formats will have a hierarchical structure.

This will reduce the size of routing tables.

o    If used in the Domain Specific Part (DSP), an NSAP selector

shall be the least significant component in the hierarchy.

The NSAP selector shall not be used to preform routing; it

is simply intended to identify the network service user at

the destination end system.  For those implementations

using an NSAP selector, there shall be one and only one

selector for each NSAP within the end system.  All NSAP

addresses identifying a given NSAP will use the same NSAP

selector value.

3.8  ROUTING

3.8.1     Static Routing

End systems and intermediate systems supporting static routing

will provide a local mechanism to update and, if necessary, to

create the local routing table.  Updating and consistency

checking will be performed by human operators.  The algorithms

and data structures used for static routing are not specified in

these agreements.  Implementors are free to perform these

functions in the manner which is most appropriate to their system

environment.

3.8.2     End System to Intermediate System

Dynamic routing between end systems and intermediate systems is

performed using the protocol described in ISO 9542, End System to

Intermediate System Routing Exchange Protocol for use in

conjunction with ISO 8473.  The following agreements apply to the

use of this protocol over LANs and point-to-point links.

1.   Implementations must support any valid NSAP format. For

the purposes of the protocol, NSAP addresses are

treated simply as octet strings.

2.   Implementations must support both Configuration

Information and Route Redirection Information.  No

subsets are permitted.

3.   All timer values must be settable using local system

management.

4.   Use of checksums must be settable using local system

management. Under normal use, checksums will be

disabled.

5.   The QOS, Security and Priority parameters should not be

used for routing.  For conformance, Intermediate

systems must transmit these parameters in RD PDUs if

they are present in the data PDU which generated the

redirect.  However, End systems must ignore them in

received RD PDUs.

6.   Both ES and IS implementations must support the

'optimization' described in Clause A.3 of ISO 9542 for

system initialization.  Its use must be selectable

using local system management.

7.   This protocol employs the same LSAP as ISO 8473.

8.   The encoding of the BSNPA address follows the syntax

rules for the data link being used.  On a LAN, for

example, it is a 48-bit MAC address.

9.   The multicast addresses corresponding to "All

Intermediate Systems on the network" (All_ISN) and "All

End Systems on the network" (All_ESN) shall default to

the following:

All_ISN = 0900 2B00 0004

All_ESN = 0900 2B00 0005

10.  The ES-IS protocol should be primarily responsible for

updating the Network Layer Routing Information Base

(RIB).  In addition, a management mechanism capable of

adding and deleting "static" entries into the RIB is

recommended.  When using the management mechanism to

add an entry, there should be no holding timer, and the

entry should be write protected from alteration by the

ES-IS protocol.

Note:     This function enables route table entries to be

made which are static in nature.  The use of

static entries is not intended for normal

operations, but rather is provided for test and

debug as well as for interoperation with other OSI

implementations which do not support the ES-IS

protocol.

3.9  MIGRATION CONSIDERATIONS

This section considers problems arising from evolving OSI standards

and implementations based on earlier versions of OSI standards.

3.9.1     X.25-1980

Until there is widespread availability of 1984 X.25 service, it

will be necessary for X.400 systems to use those existing packet-

switched public data networks which offer only pre-1984 X.25

service.  While 1980 X.25 does not provide the CONS as defined by

ISO 8348, there is no implication of non-conformance to these

Agreements resulting there from for systems using 1980 X.25 to

interchange data at the Network Layer, provided they conform in

all other respects.

This is an exception to the Agreements for providing the OSI

Network Service, granted temporarily for practical reasons.  This

exception will be removed when it is deemed to be no longer

necessary, in the judgement of the Workshop.  While this

provision is in effect, it provides an alternative method of

using 1980 X.25 to the provisions of 3.6.2

3.10 CONFORMANCE

4. TRANSPORT

4.1  INTRODUCTION

These agreements support the integration of LANs, packet networks, and

other WANs with the smallest possible set of mandatory protocol sets,

in accordance with the other agreements already reached.  Nothing here

shall preclude vendors from implementing protocol suites in addition

to the ones described in this document.

4.2  SCOPE AND FIELD OF APPLICATION

Two connection-oriented transport classed have been identified for

implementation: classes 0 and 4.  Transport class 4 is endorsed for

use over CLNS and CONS.  Transport class 0 is endorsed for use over

CONS.

4.3  STATUS

Completed March 1987.

4.4  ERRATA

4.5  TRANSPORT CLASS 4

4.5.1     Transport Class 4 Overview

Transport class 4 is mandatory for communication between systems

using the OSI CLNS and may also be used for systems using the OSI

CONS (i.e., a private MHS, etc.).

4.5.2     Protocol Agreements

The full protocol will be available including expedited data and

negotiation at connection establishment.  A disconnect request

shall be issued in response to a connect request when the maximum

number of transport connections is reached or exceeded.

4.5.2.1   Rules for Negotiation

o    In general, the ISO rules for negotiation will be used,

specifics follow.

o    All implementations will send the l6/3l window

size/sequence space in the CR TPDU.  Implementations

must all provide the l6/3l ISO option.  Implementations

must be able to accept the 4/7 in a CR TPDU.

o    The ISO maximum TPDU size is negotiable between l28 and

8K octets, always negotiated downward.  The ISO rules

are to be followed, allowing any valid size in the CR

TPDU.  TPDU size negotiation is a local implementation

issue.  Each vendor will decide how it is implemented

in their end system.

o    The security parameter is optional and user defined in

the ISO specification.  Implementations should not send

the security parameter in the CR TPDU; if received the

parameter should be ignored.

o    The use of checksums shall be as specified in ISO 8073

section 6.5.4., i.e., checksum shall be used unless

both transports explicitly agree negotiate to its non-

use.  Requesting its non-use is an implementation

choice.  All implementations must be able to operate

with checksums.

o    Use of acknowledgement time parameter is optional in

ISO 8073.  If an implementation is operating any policy

which delays the transmission of AK TPDUs, the maximum

amount of time by which a single AK TPDU may be delayed

shall be indicated to the peer transport service

provider using the acknowledgement time parameter.  The

value transmitted should be expressed in units of

milliseconds and rounded up to the nearest whole

millisecond.

o    Throughput, priority, and transit delay are optional in

the ISO specification.  Do not send in the CR TPDU;

ignore in the CC TPDU.

o    User data in the CR TPDU and the CC TPDU are optional.

No implementation should send; all implementations must

be prepared to receive.

o    An unknown parameter in any received CR TPDU shall be

ignored.

o    Known parameters with invalid values in a CR TPDU shall

be handled as follows:

Parameter                     Action

TSAP id                       Send DR TPDU

TPDU size                     ignore parm, use default

Version                       ignore parm, usedefault

Protection (Security)         implementation dependent

Checksum                      discard CR TPDU

Additional Options            Protocol Error

Alternate Protocol Classes    Protocol Error

Acknowledge Time              ignore parm

Throughput                    ignore parm

Residual Error Rate           ignore parm

Priority                      ignore parm

Transit Delay                 ignore parm

4.5.2.2   TRANSPORT CLASS 4 SERVICE ACCESS POINTS OR

SELECTORS

The TSAP selector field in the CR and CC TPDUs shall be

encoded as a variable length field and will be interpreted

as an octet string.  The length of the string cannot exceed

32 octets.

4.5.2.3   Retransmission Timer

It is recommended that the value used for the retransmission

timer be based upon the round-trip delay experienced on a

transport connection.  The implementation should maintain,

and continually update, an estimate of the round-trip delay

for the TC.  From this estimate, a value for the

retransmission timer is calculated each time it is started.

An example technique for maintaining the estimate and

calculating the retransmission timer is described below.

The value of the retransmission timer may be calculated

according to the following formula:

tT1 <DDD kE + AR

In this formula, E is the current estimate of the round-trip

delay on the transport connection, AR  is the value of the

acknowledgement time parameter received from the remote

transport service provider during connection establishment,

and k is some locally administered factor.

A value for k should be chosen to keep the retransmission

timer sufficiently small such that lost TPDUs will be

detected quickly, but not so small that false alarms are

generated causing unnecessary retransmission.

The value of E may be calculated using an exponentially

weighted average based upon regular sampling of the interval

between transmitting a TPDU and receiving the corresponding

acknowledgement.  Samples are taken by recording the time of

day when a TPDU requiring acknowledgement is transmitted and

calculating the difference between this and the time of day

when the corresponding acknowledgement is received.  New

samples are incorporated with the existing average according

to the following formula.

E <DDD E + (1 - `)(S - E)

In this formula, S is the new sample and ` is a parameter

which can be set to some value between 0 and 1.  The value

chosen for ` determines the relative weighting placed upon

the current estimate and the new sample. A large value of `

weights the old estimate more heavily causing it to respond

only slowly to variations in the round-trip delay.

A small value weights the new sample more heavily causing a

quick response to variations. (Note that setting ` to 1 will

effectively disable the algorithm and result in a constant

value for E, being that of the initial seed.)

If ` is set to 1-2-n for some value of n, the update can be

reduced to a subtract and shift as shown below.

E <DDD E + 2-n (S - E)

When sampling, if an AK TPDU is received which acknowledges

multiple DT TPDUs, only a single sample should be taken

being the round-trip delay experienced by the most recently

transmitted DT TPDU.  This attempts to minimize in the

sample any delay caused by the remote transport service

provider withholding AK TPDUs.

4.5.2.4   Keep-Alive Function

The Class 4 protocol detects a failed transport connection

by use of an 'inactivity timer'.  This timer is reset each

time a TPDU is received on a connection.  If the timer ever

expires, the connection is terminated.

The Class 4 protocol maintains an idle connection by

periodically transmitting an AK TPDU upon expiration of the

'window timer'. Thus, in a simple implementation, the

interval of one transport entity's window timer must be less

than that of its peer's inactivity timer, and vice versa.

The following agreements permit communicating transport

entities to maintain an idle connection without shared

information about timer values.

o    In accordance with ISO 8073, clause 12.2.3.9.a,

all  implementations must respond to the receipt

of a duplicate AK TPDU not containing FCC by

transmitting an AK TPDU containing the 'flow

control confirmation' parameter.

o    Implementations must always transmit duplicate AK

TPDUs without FCC on expiration of the local

window timer (see ISO 8073, clause 12.2.3.8.1).

Receipt of this TPDU by the remote transport

entity will cause it to respond with an AK TPDU

containing the 'flow control confirmation'

parameter.  When this is received by the local

transport entity, it will reset its inactivity

timer.  See figure 4.1.

o    It is a local matter for an implementation to set

the intervals of its timers to appropriate

relative values. Specifically:

o    The window timer must be greater than the

round-trip delay.  See section 4.1.4.

o    The inactivity timer must be greater than two

times the window timer; and should normally be an

even greater multiple if the transport connection

is to be resilient to the loss of an AK TPDU.

A duplicate AK TPDU (See Figure 4.1) is one which contains

the same values for YR-TU-NR, credit, and subsequence number

as the previous AK TPDU transmitted.  A duplicate AK TPDU

does not acknowledge any new data, nor does it change the

credit window.

I                  W

3                  3             3                 3

3                  3             3                 3

3              DDDDEDDDD         3    duplicate    3

3          expire  3             3       AK        3

3                  3             3                 3

DDDDEDDDD              3             3     AK + FCC    3

reset  3                  3             3                 3

3                  3             3                 3

3                  3             3                 3

3              DDDDEDDDD         3    duplicate    3

3          expire  3             3       AK        3

3                  3             3                 3

3                  3             3                 3

DDDDEDDDD              3             3     AK + FCC    3

reset  3                  3             3                 3

3                  3             3                 3

3                  3             3                 3

3              DDDDEDDDD         3                 3

3          expire  3             3                 3

3                  3             3                 3

3                  3             3                 3

Figure 4.1  AK exchange on idleconnection

4.6  TRANSPORT CLASS 0

4.6.1     Transport Class 0 Overview

Transport class 0 over X.25 is mandatory (see X.400) for use in

communicating with public MHS systems operating in accordance

with the CCITT X.400 series recommendations.  The purpose of the

agreements concerning transport class 0 is to allow connection to

these public services.  Transport class 0 over X.25 can also be

used in communicating between PRMDs (this choice is prevalent

outside North America).

4.6.2     Protocol Agreements

Transport Class 0 agreements follow:

o    The Error (ER) TPDU may be used at any time and upon

receipt requires that the recipient disconnect the

network connection, and by extension the transport

connection.

o    The allowed values for the maximum TPDU size are as

specified in ISO 8073.  They are:  128, 256, 512, 1024,

and 2048.

o    The class 0 protocol does not support multiplexing.  At

any instant, one transport corresponds to one network

connection.

o    It is recommended that the optional timers TS1 and TS2,

if implemented, be settable by local system management.

Values in the order of minutes should be supported.

o    An unlimited TSDU length must be supported.

4.6.2.1   TRANSPORT CLASS 0 SERVICE ACCESS POINTS

For communicating with public MHS systems, Section 5 of

X.410 specifies the use and format of TSAP identifiers.

4.6.3     Rules for Negotiation

The ISO rules for negotiations will be used.

5. UPPER LAYERS

5.1  INTRODUCTION

In this portion of the Implementors' Agreements, the NBS Upper Layers

SIG is primarily concerned with providing implementation agreements

for ACSE, and the Presentation and Session layers, so that systems

implemented according to these agreements can successfully

interoperate.

5.1.1     References

All documents referenced in the Upper Layers section of these

agreements can be found in the REFERENCES section of this NBS

Implementors' Agreements document.

5.2  SCOPE AND FIELD OF APPLICATION

This section does not detail particular conformance statements for

ACSE, Presentation, and Session, since what is to be implemented in

each case depends on which Application Service Elements (ASE's) and

which functional units within each ASE are used with an Application

Process.  Each ASE's SIG must specify which functional units of each

layer it requires.  However, the scope of each layer is based on the

total indicated requirements of all ASE's for which there is an active

NBS SIG.  The implementation agreements are not specified beyond that

scope.

It is not the intent of this document to specify or reproduce

standards, but when a referenced standard is unclear or has known

defects, an attempt will be made to remedy the problem herein.  Any

attempted clarification should be considered as a possible

interpretation; the ISO standard still takes precedence if there is

any conflict.  The situation with respect to defects in a standard is

somewhat different; a reported defect may be technically resolved by

the appropriate international technical committee with likely approval

by the voting members pending for several months.  Since relevant

defects can't be ignored in an implementation, this document will

recommend using defect resolutions which have the tentative approval

of the appropriate standards committees.

5.3  STATUS

This document is the first stable version of the NBS UL SIG.

5.4  ERRATA

5.4.1     ISO Defect Reports

This section lists the defect reports from ISO which are

currently recognized to be valid for the purposes of NBS

conformance.

5.4.2     Session Defects

These defects are listed for the benefit of X.400

implementations.

The following 8326 defect reports  have been incorporated into

version 1 of Session:

004, 006, 007, 009, 011, 012, 013, 014, 015, 016, 017, 020.

The following 8327 defect reports  have been incorporated into

version 1 of Session:

001, 003, 004, 005, 006, 007, 008, 009, 010, 012, 017, 018,

019, 026, 027, 030, 034, 035.

5.5  ASSOCIATION CONTROL SERVICE ELEMENT

5.5.1     Introduction

This section details the implementation requirements for the

Association Control Service Element (ACSE) of the Application

layer.  It is the intent of this section to follow the ISO ACSE

standards.  Where those specifications are inadequate, this

section should provide the necessary information.

5.5.2     Services

5.5.2.1   ACSE Services

The following ACSE service primitives are within the

possible scope of an NBS conformant system:

1.  A_ASSOCIATE request

2.  A_ASSOCIATE indication

3.  A_ASSOCIATE response

4.  A_ASSOCIATE confirm

5.  A_RELEASE request

6.  A_RELEASE indication

7.  A_RELEASE response

8.  A_RELEASE confirm

9.  A_ABORT request

10. A_ABORT indication

11. A_P_ABORT indication

5.5.2.2   Use of Presentation Layer Services

ACSE services will make use of Presentation layer services

in the manner defined in the ACSE Protocol specification.

5.5.3     Protocol agreements

Implementations shall be based on the ACSE Service definition and

the ACSE Protocol specification.

5.5.3.1   Application Context

Specific Application Contexts and their names will be

supplied and defined by  an appropriate NBS SIG.  Other

application contexts may be defined and specified as

dictated by particular application requirements.

Optional names and specifications are outlined by each

application  SIG under the heading "Specific ASE

Requirements for ACSE, Presentation, and Session".  The use

of these names implies adherence to the relevant NBS

implementors' agreements for a particular application  SIG.

The utility of  an NBS defined name (which is an OBJECT

IDENTIFIER) is left up to the application.  An NBS name may

or may not be used in the ACSE APDU.  The consequence of the

name is left up to the application entities and any a priori

agreements that they have.  In other words, it is up to the

application whether this parameter is ignored or validated

for correctness.  (Note that the consequence of this name

must also be dictated by the particular conformance test).

The UL  SIG recognizes that this parameter needs further

definition by the appropriate standards bodies.  Therefore,

the use of this parameter for association negotiation is not

recommended at this time.

5.5.3.2   Section Deleted

5.5.3.3   AE Title

A value is defined for the AE Title only to satisfy the FTAM

requirement for exchanging fields of this type.

This value does not identify an Application Entity and

carries no semantics.  The AE title maps onto the AP title

and the AE qualifier.  If the AE title is used, then both QP

title (an OBJECT IDENTIFIER) and AE qualifier (an INTEGER)

must be sent.

The value for the AP titles is {1 3 999 1 ftam-nil-ap-title

(7)} at this time.  Values for the AE qualifier are outside

the scope of these agreements.

5.6  PRESENTATION

5.6.1     Introduction

This section details the implementation requirements for the

Presentation layer.  It is the intent of this section to follow

the ISO Presentation Standards.  Where those specifications are

inadequate, this section should provide the necessary

information.

The task of the Presentation layer is to carry out the

negotiation of transfer syntaxes and to provide for the

transformation to and from transfer syntaxes.  The transformation

to and from a particular transfer syntax is a local

implementation issue and is not discussed within this section.

This section is concerned with the protocol agreements, and thus

is entirely devoted to the issues involved with the negotiation

of transfer syntaxes and the responsibilities of the

Presentation protocol.

5.6.2     Services

5.6.2.1   Presentation Services

The following functional units are within the possible scope

of an NBS conformant system:

Presentation Kernel - This functional

unit supports the basic Presentation

services required to establish a

Presentation connection, transfer

normal data, and release a

Presentation connection.  This is a

non-negotiable functional unit.

The Context Management and Context

Restoration functional units are not

within the scope of an NBS

conformant system and need not be

supported.

The requirement that the Presentation kernel functional unit

be implemented does not imply that any of the Session

functional units for expedited data, typed data, and

capability data and the corresponding Presentation service

primitives are required to be implemented.  Any service not

supported by the Session layer is also not supported by the

Presentation layer; see the section on Session Functional

Units for the possible Session functional units.  The

services provided by the Presentation layer are limited by

the services provided by the Session layer as defined in the

Session service definition ISO/IS 8326 and the Session

protocol definition ISO/IS 8327.

5.6.2.2   Use of Session Layer Services

Presentation layer services shall make use of Session layer

services in the manner defined in the Presentation Protocol

Specification.

5.6.3     Protocol Agreements

Implementations shall be based on the Presentation Service

Definition, ISO 8822  and the Presentation Protocol Definition,

ISO 8823.

5.6.3.1   Transfer Syntaxes

o    The following transfer syntax must be supported

for all mandatory abstract syntaxes: the basic

encoding rules for ASN.1.  This syntax is derived

by applying the basic encoding rules for ASN.1 to

the abstract syntax (see the Basic Encoding Rules

for ASN.1, ISO 8825).

o    The number of transfer syntaxes proposed is

dependent upon the recognized transfer syntaxes

which are available to support the particular

abstract syntaxes used by an Application Entity.

5.6.3.2   Abstract Syntaxes

o    Several abstract syntax names may map onto a

single transfer syntax name.

o    The ACSE abstract syntax shall always be present

in the defined context set.

5.6.3.3   Presentation Context Identifier

o    The presentation context identifier value shall be

encoded in no more than 2 octets.

o    Implementations must be able to handle a minimum

of 2 presentation contexts per connection.

5.6.3.4   Mode-selector Position in SET

Whenever the Mode-selector value within either a CP-PPDU or

CPA-PPDU is normal-mode (1), it shall occur as the first

element within the SET.

5.6.3.5   Default Context

If the Presentation expedited data service is required, the

default context must be explicitly present in the P-CONNECT

PPDU at Presentation connect time.

5.6.3.6   P-Selectors

Local P-selectors shall be a maximum of 4 octets.  This

applies only to P-selectors in PPDUs whose receipt by an

NBS-conformant system normally results in either a P-CONNECT

indication or a P-CONNECT confirmation being issued.

5.6.3.7   Provider Abort Parameters

No conformance requirements are implied by the use of either

the Abort-reason or the Event-identifier component of the

ARP-PPDU.  The decision to include these parameters is left

up to the implementation issuing the abort.

5.6.3.8   Provider Aborts and Session Version

The Presentation Provider Abort PPDU (ARP-PPDU) shall be

present regardless of the Session version in effect for a

given association.  This precludes the use of indefinite

length encoding of an ARP-PPDU when Session version 1 is in

effect.

5.6.3.9   CPC-type

NBS conformant implementations shall not use any CPC-type

values in the SS-user data parameter of the S-CONNECT

request unless more than one transfer syntax is proposed.

Each CPC-type represents a unique transfer syntax.  Note:

currently only 1 transfer syntax is recognized.

5.6.3.10  Presentation-context-definition-result-list

No semantics are implied by the absence of this optional

component of the CPR-PPDU.  This component is required if

the Provider-reason is absent in the CPR-PPDU.  If the

Provider-reason is present, then the Presentation-context-

definition-result-list is optional.

5.6.3.11  RS-PPDU

The Presentation-context-identifier-list shall not be

present when only the kernel functional unit is in effect.

5.6.4     Presentation ASN.1 Encoding Rules

5.6.4.1   Invalid Encoding

If a received PPDU contains any improperly encoded data

values (including data values embedded within the User Data

field of a PPDU) and an abort is issued, then either a P-U-

ABORT or a P-P-ABORT shall be issued.

5.6.4.2   Protocol-version, Presentation-requirements

Protocol-version and Presentation-requirements shall be

encoded in their primitive forms, if present.

5.6.4.3   Presentation-selector

Any type defined as a Presentation-selector shall be encoded

in its primitive form, if present.

5.7  SESSION

5.7.1     Introduction

This section details the implementation requirements for the

Session layer.  It is the intent of this section to follow the

ISO Session Standards to the fullest extent possible.  Where

those specifications are inadequate, this section should provide

the necessary information.

5.7.2     Services

5.7.2.1   Session Services

The following functional units are within the possible scope

of an NBS conformant system:

Kernel

Duplex

Expedited Data

Resynchronize

Exceptions

Activity Management

Half-duplex

Minor Synchronize

Major Synchronize

Typed Data

5.7.2.2   Use of Transport Services

The use of Transport layer services by the Session layer

functional units listed in the previous section is as

specified in the Transport Protocol Specification, ISO/IS

8073.

5.7.3     Protocol Agreements

Implementations shall be based on the Session service definition

ISO/IS 8326 and the Session protocol definition ISO/IS 8327.

5.7.3.1   Concatenation

When a category 0 SPDU is concatenated with a category 2

SPDU, the category 0 SPDU shall contain neither the Token

Item field nor User Data.  If either a Token Item field or

User Data is received in such a concatenated incoming SPDU,

the receiving Session Protocol Machine has the option of

either properly processing the fields or issuing a provider

abort on the connection.

Extended concatenation is not required and can be refused

using the normal negotiation mechanisms of the Session

protocol.

5.7.3.2   Segmenting

Session Segmenting is not required and can be refused using

the normal negotiation mechanisms of the session protocol.

5.7.3.3   Reuse of Transport Connection

Reuse of a transport connection is not required and can be

refused.

5.7.3.4   Use of Transport Expedited Data

The use of transport expedited service is as stated in the

session protocol specification: if available, transport

expedited service must be used.

5.7.3.5   Use of Session Version Number

Session versions 1 and 2 are recognized.  Each relevant NBS

SIG chooses the version or versions of Session which it

requires for a particular implementation phase, and these

choices are documented in section 5.9.1.

Session version 2 specifies the use of unlimited user data

during connection establishment as dictated by the DAD 2 to

ISO 8327 to Incorporate Unlimited User Data.

All Session version 1 implementations must be able to

negotiate version 1 operation when responding to a CONNECT

(CN) SPDU proposing both version 1 and version 2.

In addition, all Session version 1 implementations, upon

receipt of a CONNECT (CN) SPDU proposing only version 2,

should respond with a REFUSE (RF) SPDU containing a Reason

Code indicating that the proposed version is not supported.

Until pending defect reports are adopted, implementations

may disconnect.

If Session versions 1 and 2 are both proposed in the CONNECT

(CN) SPDU, then the maximum length of the User Data

parameter value in the CONNECT (CN) SPDU shall be 512 octets

and a PGI field of 193 shall be associated with this

parameter.  This implies that an implementation supporting

both Session versions 1 and 2 can establish a connection

with an implementation supporting only version 1.

If only Session version 2 is proposed in the CONNECT (CN)

SPDU, then the maximum length of the Session User Data

parameter value of the S-CONNECT service request shall be

10,240 octets.  This restriction implies that the OVERFLOW

ACCEPT (OA) SPDU and CONNECT DATA OVERFLOW (CDO) SPDU are

not used.  If the length of the User Data parameter value is

no greater than 512 octets, then an associated PGI field of

193 shall be used, otherwise a PGI field of 194 shall be

used.

When Session version 2 is negotiated, then in all SPDUs the

maximum length of the User Data parameter value with an

associated PGI field of 193 shall be 10,240 octets.  NBS

conformant Session version 2 implementations need only

support the maximum data lengths specified in the Specific

ASE Requirements section.

5.7.3.6   Receipt of Invalid SPDUs

Upon receipt of an invalid SPDU, the SPM shall take any

action in A.4.3 of the Session protocol definition ISO/IS

8327 except action d.

5.7.3.7   Invalid SPM Intersections

If the conditions described in A.4.1.2 of the Session

protocol definition ISO/IS 8327 are satisfied, the SPM shall

always take the actions described by A.4.1.2 a.

Note:     This implies that no S-P-EXCEPTION-REPORT

indications will be generated nor EXCEPTION REPORT

SPDUs sent due to invalid intersections of the

Session state table resulting from received SPDUs.

5.7.3.8   S-Selectors

S-selectors shall be a maximum of 16 octets.

5.8  Universal ASN.1 Encoding Rules

5.8.1     Tags

The maximum value of an ASN.1 basic encoding tag that need be

handled by an NBS-conformant implementation shall be 16,383.

This is the maximum unsigned number that can be represented in 14

bits, therefore, the maximum encoding of a tag occupies 3 octets.

5.8.2     Definite length

The maximum value of an ASN.1 length octets component that need

be handled by an NBS-conformant implementation shall be

4,294,967,295.  This is the maximum unsigned integer that can be

represented in 32 bits, therefore, the maximum encoding of a

length octets component will occupy 5 octets.  Also, note this

restriction does not apply to indefinite length encoding.

5.8.3     EXTERNAL Type

It is assumed that "presentation layer negotiation of encoding

rules" is always in effect, and therefore clause 32.5 of the

Specification of ASN.1, ISO 8824 never applies.

5.9  CONFORMANCE

In order for an implementation to be in conformance with the NBS

implementors' agreements, the following rules shall be adhered to:

o    A conformant implementation must  meet all of the requirements of

this specification.  All documents referenced in the Upper Layers

section shall be used as the supporting documents for all

implementations of ACSE, Presentation, or Session.  The full

references for these documents are in the REFERENCES section.

o    NBS conformant implementations shall be ISO conformant.  PICS may

contain limitations on length or value aspects of a protocol.

PICS of NBS conformant systems shall not contain restrictions

more severe than those in these implementation agreements.  Note:

an implementation may abort a connection if the constraints

specified in these agreements are violated.

o    Guidelines for implementation of standards' defects will be as

per the resolution of such defects by the appropriate ISO

standards committee.

5.9.1     Specific ASE Requirements for ACSE Presentation and

Session

The following list for each ASE the corresponding NBS SIGs

requirements of and restrictions on ACSE, Presentation, and

Session.

All listed requirements and restrictions shall be included in an

NBS conformant system and shall be implemented in accordance with

these NBS Implementor's agreements.

All OBJECT IDENTIFIERS are specified in terms of their associated

ObjectDescriptor's.  See the chapter on OBJECT IDENTIFIERs for

the values of the associated OBJECT IDENTIFIERs.

5.9.1.1   FTAM

5.9.1.1.1 Phase 2

ACSE Requirements:

all

Application Contexts:

o    "ISO FTAM" - implies the use of the

ACSE and the FTAM ASE.

Abstract Syntaxes:

o    "ISO 8650-ACSE1"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

Presentation Requirements:

Presentation Functional Units:

o    kernel

Presentation Contexts:

o    At least 3 Presentation Contexts

must be supported.

Abstract Syntaxes:

Abstract Syntaxes for conformant

Implementations

o    "FTAM-PCI"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type:

o    "FTAM unstructured binary abstract

syntax"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

Abstract Syntaxes Depending on

Implementation Profile

o    "FTAM-FADU"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

o    "FTAM unstructured text abstract

syntax"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

o    "NBS abstract syntax AS1"

Associated Transfer Syntax:

o    "Basic Encoding  of a

single ASN.1 type"

o    "NBS file directory entry abstract

syntax"

Associated Transfer Syntax:

o    "Basic Encoding of a

single Asn.1 type"

Session Requirements:

Session Functional Units:

o    kernel

o    duplex

Version Number: 2

Maximum size of User Data parameter field:

10,240

Session Options:

Session Functional Units:

o    resynchronize

only a Resynchronize Type

value of "abandon"

o    minor synchronize

Note:     The minor

synchronize

functional unit is

required whenever

the resynchronize

functional unit is

available.

5.9.1.2   MHS

5.9.1.2.1 Phase 1

Session Requirements:

Session Functional Units:

o    kernel

o    half-duplex

o    exceptions

o    activity management

o    minor synchronize

Version Number: 1

Maximum size of User Data parameter field:

512

Session Notes:

o    Restricted use is made by the RTS

of the session services implied by

the functional units selected.

Specifically,

.    No use is made of S-TOKEN-

GIVE, and

.    S-PLEASE-TOKENS only asks for

the data token.

o    In the S-CONNECT SPDU, the Initial

Serial Number should not be

present.

o    The format of the Connection

Identifier in the S-CONNECT SPDU is

described in Version 5 of the

X.400-Series Implementors' Guide.

5.9.1.3   DS

5.9.1.3.1 Phase 1

ACSE Requirements:

all

Application Context:

Abstract Syntaxes:

o    "ISO 8650-ACSE1"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

o    "Directory Services"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

Presentation Requirements:

Presentation Functional Units:

o    kernel

Presentation Contexts:

o    At least 2 Presentation Contexts

must be supported.

Session Requirements:

Session Functional Units:

o    kernel

o    duplex

Version Number: 2

Maximum size of User Data parameter field:

10,240

5.10   APPENDIX A:  RECOMMENDED PRACTICES

Reflect Parameter Values

The optional "Reflect Parameter Values" parameter in the provider

ABORT SPDU shall be encoded so as to represent the Session connection

state, the incoming event and the first invalid SPDU field exactly at

the moment a protocol error was detected.

The first octet encodes the Session state as a number relative to 0 as

detailed in Table 1.

The second octet encodes the incoming event as a number relative to 0

as detailed in Table 2.

The third octet contains the SI, PGI, or PI Code of any SI field, PGI

unit or PI unit in error.

NOTE:  The remaining 6 octets are undefined herein.

Table 1 - Session States

State     rel.#   Description

1        0      Idle, no transport connection

1B       1      Wait for T-connect confirm

1C       2      Idle, transport connected

2A       3      Wait for the ACCEPT SPDU

3        4      Wait for the DISCONNECT SPDU

8        5      Wait for the S-CONNECT response

9        6      Wait for the S-RELEASE response

16        7      Wait for the T-DISCONNECT indication

713        8      Data Transfer state

1A       9      Wait for the ABORT ACCEPT SPDU

4A      10      Wait for the MAJOR SYNC ACK SPDU or PREPARE

SPDU

4B      11      Wait for the ACTIVITY END ACK SPDU or PREPARE

SPDU

5A      12      Wait for the RESYNCHRONIZE ACK SPDU or PREPARE

SPDU

5B      13      Wait for the ACTIVITY INTERRUPT SPDU or PREPARE

SPDU

5C      14      Wait for the ACTIVITY DISCARD ACK SPDU or

PREPARE

SPDU

6       15      Wait for the RESYNCHRONIZE SPDU or PREPARE

SPDU

10A      16      Wait for the S-SYNC-MAJOR response

10B      17      Wait for the S-ACTIVITY-END response

11A      18      Wait for the S-RESYNCHRONIZE response

11B      19      Wait for the S-ACTIVITY-INTERRUPT response

11C      20      Wait for the S-ACTIVITY-DISCARD response

15A      21      After PREPARE, wait for the MAJOR SYNC ACK SPDU or the

ACTIVITY END ACK

15B      22      After PREPARE, wait for the RESYNCHRONIZE SPDU or the

ACTIVITY DISCARD SPDU

15C      23      After PREPARE, wait for the RESYNCHRONIZE ACK SPDU, or

the ACTIVITY INTERRUPT ACK SPDU or the ACTIVITY

DISCARD ACK SPDU

18       24      Wait for GIVE TOKENS ACK SPDU

19       25      Wait for a recovery request or SPDU

20       26      Wait for a recovery SPDU or request

21       27      Wait for the CAPABILITY DATA ACK SPDU

22       28      Wait for the S-CAPABILITY-DATA response

1D      29      Wait for the CONNECT DATA OVERFLOW SPDU

2B      30      Wait for the OVERFLOW ACCEPT SPDU

15D      31      After PREPARE, wait for the ABORT SPDU

Table 2 - Incoming Events

Event            Rel.# Description

SCONreq           0    S-CONNECT request

SCONrsp+          1    S-CONNECT accept response

SCONrsp-          2    S-CONNECT reject response

SDTreq            3    S-DATA request

SRELreq           4    S-RELEASE request

SRELrsp+          5    S-RELEASE accept response

SUABreq           6    S-U-ABORT request

TCONcnf           7    T-CONNECT confirmation

TCONind           8    T-CONNECT indication

TDISind           9    T-DISCONNECT indication

TIM              10    Time out

AA               11    ABORT ACCEPT

AB-nr            12    ABORT - no reuse

AC               13    ACCEPT

CN               14    CONNECT

DN               15    DISCONNECT

DT               16    DATA TRANSFER

FN-nr            17    FINISH - no reuse

RF-nr            18    REFUSE - no reuse

SACTDreq         19    S-ACTIVITY-DISCARD request

SACTDrsp         20    S-ACTIVITY-DISCARD response

SACTEreq         21    S-ACTIVITY-END request

SACTErsp         22    S-ACTIVITY-END response

SACTIreq         23    S-ACTIVITY-INTERRUPT request

SACTIrsp         24    S-ACTIVITY-INTERRUPT response

SACTRreq         25    S-ACTIVITY-RESUME request

SACTSreq         26    S-ACTIVITY-START request

SCDreq           27    S-CAPABILITY-DATA request

SCDrsp           28    S-CAPABILITY-DATA response

SCGreq           29    S-CONTROL-GIVE request

SEXreq           30    S-EXPEDITED-DATA request

SGTreq           31    S-TOKEN-GIVE request

SPTreq           32    S-TOKEN-PLEASE request

SRELrsp-         33    S-RELEASE response reject

SRSYNreq(a)      34    S-RESYNCHRONIZE request abandon

SRSYNreq(r)      35    S-RESYNCHRONIZE request restart

SRSYNreq(s)      36    S-RESYNCHRONIZE request set

SRSYNrsp         37    S-RESYNCHRONIZE response

SSYNMreq         38    S-SYNC-MAJOR request

SSYNMrsp         39    S-SYNC-MAJOR response

SSYNmreq         40    S-SYNC-MINOR request

SSYNmrsp         41    S-SYNC-MINOR response

STDreq           42    S-TYPED-DATA request

SUERreq          43    S-U-EXCEPTION-REPORT request

AB-r             44    ABORT - reuse SPDU

Table 2 - continued

Event            Rel.# Description

AD               45    ACTIVITY DISCARD SPDU

ADA              46    ACTIVITY DISCARD ACK SPDU

AE               47    ACTIVITY END SPDU

AEA              48    ACTIVITY END ACK SPDU

AI               49    ACTIVITY INTERRUPT SPDU

AIA              50    ACTIVITY INTERRUPT ACK SPDU

AR               51    ACTIVITY RESUME SPDU

AS               52    ACTIVITY START SPDU

CD               53    CAPABILITY DATA SPDU

CDA              54    CAPABILITY DATA ACK SPDU

ED               55    EXCEPTION DATA SPDU

ER               56    EXCEPTION REPORT SPDU

EX               57    EXPEDITED DATA SPDU

FN-r             58    FINISH - reuse SPDU

GT               59    GIVE TOKENS SPDU

GTA              60    GIVE TOKENS ACK SPDU

GTC              61    GIVE TOKENS CONFIRM SPDU

MAA              62    MAJOR SYNC ACK SPDU

MAP              63    MAJOR SYNC POINT SPDU

MIA              64    MAJOR SYNC ACK SPDU

MIP              65    MINOR SYNC POINT SPDU

NF               66    NOT FINISHED SPDU

PR-MAA           67    PREPARE (MAJOR SYNC ACK) SPDU

PR-RA            68    PREPARE (RESYNCHRONIZE ACK) SPDU

PR-RS            69    PREPARE (RESYNCHRONIZE) SPDU

PT               70    PLEASE TOKENS SPDU with Token Item

Parameter

RA               71    RESYNCHRONIZE ACK SPDU

RF-r             72    REFUSE - reuse SPDU

RS-a             73    RESYNCHRONIZE - abandon SPDU

RS-r             74    RESYNCHRONIZE - restart SPDU

RS-s             75    RESYNCHRONIZE - set SPDU

TD               76    TYPED DATA SPDU

CDO              77    CONNECT DATA OVERFLOW SPDU

OA               78    OVERFLOW ACCEPT SPDU

PR-AB            79    PREPARE (ABORT) SPDU

6. ISO FILE TRANSFER, ACCESS AND MANAGEMENT PHASE 2

6.1  INTRODUCTION

This section defines Implementors' Agreements based on ISO File

Transfer, Access and Management (FTAM), as defined in ISO 8571. This

International Standard has four parts.  Part 1 of the IS gives general

concepts, Part 2 defines the Virtual Filestore (VFS), Part 3 defines

the File Service, and Part 4 defines the File Protocol.

FTAM, as described in the IS, is based on the following ISO documents:

ACSE Service and Protocol (ISO 8649, ISO 8650), Presentation Service

and Protocol (ISO 8822, ISO 8823), ASN.1 Abstract Syntax Notation and

Basic Encoding Rules (ISO 8824, ISO 8825), and Session Service and

Protocol (ISO 8326, ISO 8327).  These services and protocols are

defined architecturally in the OSI Reference Model (ISO 7498).  These

Agreements provide detailed guidance for the implementor, and

eliminate ambiguities in interpretations.

The general agreements reached with respect to the ISO File Transfer,

Access and Management Protocol (FTAM) are that the Phase 2 FTAM

specification (this section) is based on the International Standard

(IS).

6.2  SCOPE AND FIELD OF APPLICATION

These FTAM Phase 2 Agreements cover transfer of and access to files

between the Filestores of two end systems, including the management of

a Virtual Filestore.  One end system acts in the Initiator role and

initiates the file transfer/access, while the other end system acts in

the Responder role and provides access to the file in the Virtual

Filestore.  This paper describes Agreements for the actions and

attributes of the Virtual Filestore, and the service provided by the

file service provider to file service users, together with the

necessary communications between the Initiator and Responder.

ZDDDDDDDDDDDDDD?

3   Virtual    3

3  Filestore   3

3      2       3

@DDDDDDRDDDDDDDY

:

:

ZDDDDDDDDDDDDDD?    ZDDDDDDDDDDDDDD?    ZDDDDDDPDDDDDDD?    ZDDDDDDDDDDDDDD?

3    Real      3    3     End      3    3     End      3    3     Real     3

3  Filestore   CDDDD4   System 1   FMMMM5   System 2   CDDDD4   Filestore  3

3      1       3    3- Initiator - 3    3- Responder - 3    3       2      3

@DDDDDDDDDDDDDDY    @DDDDDDDDDDDDDDY    @DDDDDDDDDDDDDDY    @DDDDDDDDDDDDDDY

Figure 6.1  Model of file transfer/access

Note: Agreements apply on the double lines of Figure 1.

The mapping between the Virtual Filestore and the Real Filestore

together with the local data management system is not part of

these Agreements.

These Agreements define General Agreements is in section 6.5 through

6.16, minimum functionality (Conformant Implementations) in section

6.17, and functionality for several Implementation Profiles which are

tailored to different classes of user requirements in sections 6.18

and 6.19.

6.3  STATUS

This version of the FTAM Implementation Agreements was completed

December 18, 1987.  No further enhancements will be made to this

version.  See the next section, ERRATA.

Note: These Agreements were updated from the previous March 1987 DIS

based Agreements.

6.4  ERRATA

ZDDDDDDDDDDDBDDDDDDDDDBDDDDDDDDDDDBDDDDDDDDDDDBDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3CP no      3Type     3ref. doc   3section    3Comment                                        3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 2/88-1  3technical3NBS 500-15036.10.2.2,  3Check of already established presentation      3

3           3         3           36.10.2.3   3context for a document type not at CREATE time 3

3           3         3           3           3but at OPEN time                               3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 2/88-2  3editorial3NBS 500-15036, App. A, 3Appendix A, Part 4 of Object Identifiers       3

3           3         3           3Part 4     3removed                                        3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 2/88-3  3editorial3NBS 500-15036.5        3Clarification that current AET definition      3

3           3         3           3new bullet 3carries no semantics and other values are      3

3           3         3           37          3outside scope.                                 3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 2/88-4  3technical3NBS 500-15035.9        3Clarification for UL specs that ABORT possible 3

3           3         3           3           3if UL constraints are violated                 3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 2/88-5  3editorial3NBS 500-15036, App. A  3Replace 'organ-code' by 'organization-code' for3

3           3         3           3           3all NBS Object Identifiers                     3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 2/88-6  3editorial3NBS 500-15036.5        3Clarification that <shared ASE information>    3

3           3         3           3new bullet 3and <application context name> agreements are  3

3           3         3           38,9        3general agreements                             3

3           3         3           36.18.3     3                                               3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 2/88-7  3editorial3NBS 500-15036.18.2     3Move 6.18.2 to new Section 6.17.9              3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 2/88-8  3editorial3NBS 500-1503Table 6.4  3Rendition of currency sign in table 6.4        3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 2/88-9  3editorial3NBS 500-15036.2, 6.17  3Clarification of meaning of General Agreements,3

3           3         3           3           3Conformance and Implementation Profiles        3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 2/88-10 3editorial3NBS 500-15036.5        3Definition of 's', 'os' for a parameter.       3

3           3         3           3new bullet 3Clarification of support level for <initiator  3

3           3         3           310, 6.18.3 3identity>, <filestore password>, <access       3

3           3         3           3bullet 5   3password> parameter                            3

3           3         3           36.16.1     3                                               3

3           3         3           36.16.2     3                                               3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 5/88-1  3editorial3NBS 500-1503Table 6.7  3Clarification fo support of T&M service class  3

3           3         3           3Note 4     3                                               3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 5/88-2  3editorial3NBS 500-15036.18.3     3Remove <charging> value zero                   3

3           3         3           3bullet 6   3                                               3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 5/88-3  3editorial3NBS 500-1503Table 6.9  3Updated definition of Datatype 2               3

CDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3CP 5/88-4  3editorial3NBS 500-15036.5, new   3More clarification to CP 2/88, use of AETitle  3

3           3         3           3bullet 7   3values                                         3

@DDDDDDDDDDDADDDDDDDDDADDDDDDDDDDDADDDDDDDDDDDADDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

6.5  ASSUMPTIONS

1.   FTAM protocol machines must be able to parse and process at a

minimum 7K octets of FTAM PCI and FTAM user data (including

grouped FPDUs) as they would be encoded with the ASN.1 Basic

Encoding Rules.  It is recommended, however, that  Presentation

user data not be restricted in size.

2.   In order to maximize interoperability, it is important that the

implementations of FTAM service providers do not unnecessarily

restrict the service user's ability to generate arbitrary file

service requests.  Otherwise, they may not be able to work with

FTAM Responders whose operation is constrained by their mapping

of the FTAM Virtual Filestore to their local filestore.  For

example, error procedures should only be invoked when an error

actually occurs, not at the point of the specification of

options which might result in an error.

3.   Implementations must be able to parse all valid optional

parameters if they are present in the PDU.  Only those optional

parameters specified as supported in these Agreements are

required to be implemented.  If these parameters are not present,

a default value is assigned locally.  A Responder should not

refuse a request solely because a parameter that is optional in

the FTAM standard, but is supported in these Agreements, is not

present.

4.   Consideration of any standardized service interface is not

covered by these Agreements.

5.   These Agreements define no restrictions for the values used for

the <communication quality of service> parameter in <F-

INITIALIZE>.

6.   FTAM is defined in phases.  The Phase 1 FTAM implementation

specification is based on the second ISO Draft Proposal, dated

April 1985, and the ISO Draft Proposal 8824 and 8825.

The Phase 2 FTAM specification (this section) is based on the

International Standard (IS).  THERE IS NO BACKWARD COMPATIBILITY

WITH NBS FTAM PHASE 1.  Backward compatibility is impossible,

since Phase 1 uses Session services directly, while Phase 2 uses

ACSE and Presentation services.  Furthermore, there are

differences in Filestore, PDU Abstract Syntax, FADU Abstract

Syntax, and Transfer Syntax.  There also are differences in the

transparency mechanisms and service class negotiations.

The <implementation information> parameter of <F-INITIALIZE> FPDU

as defined in ISO 8571-4, 20.3 is used to pass 'user version'

information with respect to different FTAM phases of the NBS

Implementors Agreements or with respect to FTAM profiles of other

bodies (see section 6.13 6.12 of this document).  It is the goal

of these Agreements to use the 'user version' mechanism to

provide at least one level of backward compatibility for all

future NBS FTAM Phases, facilitating backward compatibility for

future FTAM products, assuming different new versions of the

respective IS's also enable backward compatibility.

7.   Section 5.5.3 5.5.3.3 defines a value (that carries no semantics)

for the AETitle that can be used by FTAM ASEs for communication.

Other values for the AETitle are outside the scope of these

Agreements.

For the Called-AETitle, Calling-AETtitle and Responding AETitle

the association shall not be rejected/aborted if the value

specified in 5.5.3.3 is sent or if any of the parameters are not

sent.  The association may be rejected/aborted if a value other

than specified in 5.5.3.3 is sent.

Use of values outside the scope of these Agreements is

discouraged until agreed upon semantics have been associated with

AETitles.

8.   Use of <shared ASE information> parameter and <charging>

parameter is not defined within the scope of the Agreements.

9.   Use of <application context name> parameter is not defined within

the scope of these Agreements.  This parameter does not prohibit

the establishment of an FTAM association.

10.  These Agreements use the term 'supported' for a parameter to mean

that the syntax and semantics of that parameter shall be

implemented.  However, it is not a requirement that the parameter

be used in all instances of communication, unless stated

otherwise.

Also these Agreements use the term 'optionally supported' for a

parameter to mean that it is left to the implementation whether

the semantics of that parameter are implemented or not.

6.6  PRESENTATION AGREEMENTS

The following Abstract Syntaxes are recognized in these agreements:

"FTAM FADU"

"FTAM PCI"

"FTAM unstructured text abstract syntax"

"FTAM unstructured binary abstract syntax"

"NBS abstract syntax AS1"

"NBS file directory entry abstract syntax"

The following Transfer Syntax is supported:

"Basic Encoding of a single ASN.1 type"

(See Appendix A, Part 3)

6.7  SERVICE CLASS AGREEMENTS

Implementation Agreements have been reached for the following service

classes.

o    File Transfer

o    File Access

o    File Management

o    File Transfer and Management

o    Unconstrained

6.8  FUNCTIONAL UNIT AGREEMENTS

Implementation agreements have been reached for the following

functional units.

o    Kernel

o    Read

o    Write

o    File Access

o    Limited File Management

o    Enhanced File Management

o    Grouping

Implementation of the Recovery, Restart Data Transfer, and FADU

Locking functional units is not specified.

6.9  FILE ATTRIBUTE AGREEMENTS

Implementation of the Kernel Group of file attributes is defined.  If

the optional Storage Group and Security Group are implemented, aspects

of their implementation are defined.  Implementation of the Private

Group is not specified.

Responses to an attribute value request shall always include one of

the following (as specified in ISO 8571-2, clause 9.4):

o    An actual file attribute value.

o    A value indicating that no value is available, optionally with a

diagnostic.

o    No value and an error code, optionally with a diagnostic

indicating that the attribute is not supported.

6.9.1     Mandatory Group

Only the Kernel Group of attributes is required.  A value for

<filename>, <permitted actions>, and <contents type> will always

be available.

A minimum range is required for <filename> values as specified in

ISO 8571-2.  No maximum length or format restrictions apply.  A

system that does not support <filename> values with a sequence of

more than one Graphic String or extended <filename>

characteristics may reject a request involving such a <filename>.

All systems must be able to interpret a <filename> value with a

sequence of one Graphic String.  Requests using such a single

component <filename> value with a sequence of one Graphic String

are responded to using a single component <filename> value.

Responses to requests involving <filename> values having two or

more Graphic Strings are not defined here but may be interpreted

via bilateral or other external agreements.  Use of <filename>

values with a sequence of more than one Graphic String is

discouraged.

Apart from the minimum conformance requirements specified in ISO

8571-2, file names have to be specified in the naming convention

of the responding FTAM implementation.  It is a local

implementation matter of the FTAM Responder, whether or not an

additional name mapping onto the real FIlestore's file name

convention is supported.

In order to enable interworking with all FTAM Responders' virtual

Filestores, it is recommended that FTAM Initiators impose no

restrictions on the attribute range supported for file names

beyond those specified in ISO 8571-2.

For the purpose of interworking according to these Agreements the

<contents type> attribute is limited to the <document type name>

format.  The <constraint set name, abstract syntax name> form is

outside the scope of these Agreements.  It should always be

parsed correctly when received, but may result in an error.

6.9.2     Optional Groups

If the optional Security Group of file attributes is implemented,

an actual value must be available for the <access control>

attribute.

The <access control> attribute is a SET OF <access control

element>.  The minimum requirement in these Agreements is the

support of one <access control element>, according to the base

standard.  The terms <concurrency access>, <identity>, and

<passwords> are each optionally supported.  Details of their use

shall be specified in the PICS.  Use of the <location> term is

not specified in these Agreements.

Implementation of the Private Group is not specified.

6.10 DOCUMENT TYPE AGREEMENTS

These document types are defined.

FTAM-1    "ISO FTAM unstructured text"

FTAM-2    "ISO FTAM sequential text"

FTAM-3    "ISO FTAM unstructured binary"

NBS-6     "NBS-6 FTAM sequential file"

NBS-7     "NBS-7 FTAM random access file"

NBS-8     "NBS-8 FTAM indexed file"

NBS-9     "NBS-9 FTAM file directory file"

Detailed document type definitions are given in Appendix 6A and in ISO

8571-2, Annex B.

Note: Document types NBS-1 to NBS-5 are not defined in these

Agreements.  The numbering starts with NBS-6 because of the original

DIS version of these Agreements.

An implementation claiming conformance to these Agreements which also

supports any or all of the document types FTAM-1, FTAM-2, and FTAM-3

as defined in ISO 8571-2, Annex B, must minimally support the

combinations of parameter values as specified in Table 6.1.

Table 6.1  Parameters for FTAM-1, -2, -3

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3            Universal             Maximum              String          3

3            Class Number          String Length6       Significance    3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                       3

3 FTAM-1     General String 1(27)    134 or less       'not-significant'3

3            IA5String 2(22)                                            3

3                                                                       3

3 FTAM-2     Graphic String 3,4(25)  134 or less5      'not-significant'3

3                                                                       3

3                                                                       3

3 FTAM-3     <not applicable>        512 or less       'not-significant43

3                                                                       3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Notes:

1.   The minimum level of support for General String is the IA5 G0

character set and the 8859-1 G0 and G1 character sets, and IA5 C0

set.

2.   The support for IA5 String is the IA5 G0 character set and the

IA5 C0 set.

3.   The minimum level of support for Graphic String is the IA5 G0

character set and the 8859-1 G0 and G1 sets.

4.   This is the default when the parameter is not specified.

5.   The implementation need not support Data Units whose total

character count exceeds 134.

6.   As per Table 6.3.

For the use of FTAM-2 only the FADU identities of 'begin', 'end',

'first', and 'next' are required for conformant implementations.

For the document types NBS-6, NBS-7 and NBS-8 parameters are used for

which the Agreements apply as specified in Table 6.2.

Table 6.2  Parameters for NBS-6, NBS-7, NBS-8

ZDDDDDDDDDDDDDDBDDDDDDDDDDDDDBDDDDDDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDDDDD?

3 Parameter    3 PrimType    3     String-length     3   Length-1      3   Length-2      3

CDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDDDDD4

3              3             3                       3                 3                 3

3 int          3 INTEGER     3 Number of octets      3                 3                 3

3              3             3 required to represent,3                 3                 3

3              3             3 in 2's complement     3                 3                 3

3              3             3 format, the largest   3                 3                 3

3              3             3 integer to be passed  3                 3                 3

3              3             3                       3                 3                 3

3 bit          3 BIT STRING  3 Number of bits in     3                 3                 3

3              3             3 string                3                 3                 3

3              3             3 (non-varying)         3                 3                 3

3              3             3                       3                 3                 3

3 ia5          3 IA5String   3 Max number of         3                 3                 3

3              3             3 characters in string  3                 3                 3

3              3             3                       3                 3                 3

3 graphic      3 Graphic     3 Max number of         3                 3                 3

3              3 String      3 characters in string  3                 3                 3

3              3             3                       3                 3                 3

3              3             3                       3                 3                 3

3 general      3 General     3 Max number of         3                 3                 3

3              3 String      3 characters in string  3                 3                 3

3              3             3                       3                 3                 3

3              3             3                       3                 3                 3

3 octet        3 OCTET STRING3 Max numbers of octets 3                 3                 3

3              3             3 in string             3                 3                 3

3              3             3                       3                 3                 3

3 private-     3 Floating    3                       3 The minimum     3 Number of bits  3

3 class-number 3 Point       3                       3 number of bits  3 required to     3

3              3 Number      3                       3 required to be  3 represent the   3

3              3             3                       3 maintained in   3 largest unbiased3

3              3             3                       3 the mantissa for3 integer exponent3

3              3             3                       3 relative        3 in 2's          3

3              3             3                       3 precision       3 complement      3

3              3             3                       3                 3                 3

3              3             3                       3                 3                 3

3              3             3                       3                 3                 3

3              3             3                       3                 3                 3

3 univer-      3 UTCTime     3 <not applicable>      3                 3                 3

3 time         3             3                       3                 3                 3

3              3             3                       3                 3                 3

3 gen-time     3 Generalized 3 <not applicable>      3                 3                 3

3              3 Time        3                       3                 3                 3

3              3             3                       3                 3                 3

3 boolean      3 BOOLEAN     3 <not applicable>      3                 3                 3

3              3             3                       3                 3                 3

3 null         3 NULL        3 <not applicable>      3                 3                 3

3              3             3                       3                 3                 3

@DDDDDDDDDDDDDDADDDDDDDDDDDDDADDDDDDDDDDDDDDDDDDDDDDDADDDDDDDDDDDDDDDDDADDDDDDDDDDDDDDDDDY

Note: The string length parameter specifies the actual number of

characters from the referenced character set.  It does not include any

escape sequences or overhead from the encoding.

The primitive data types and minimal size ranges that an

implementation must accept for storage are given in Table 6.3.

Table 6.3  FTAM primitive data types

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Primitive Data Type          Minimum Range (Octets)3

3                                                   3

3ASN.1   INTEGER                       1 - 2        3

3ASN.1   BIT STRING                    0 - 1        3

3ASN.1   IA5String                     0 - 134      3

3ASN.1   GeneralString                 0 - 134      3

3ASN.1   GraphicString                 0 - 134      3

3ASN.1   OCTET STRING                  0 - 512      3

3ASN.1   BOOLEAN                                    3

3ASN.1   NULL                                       3

3ASN.1   GeneralizedTime                            3

3ASN.1   UniversalTime                              3

3NBS-AS1 FloatingPointNumber  mantissa 1-23 bits    3

3                             exponent 0-8  bits    3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Notes:

1.   The primitive data types and their maximum ranges for a specific

file as described by the parameters above are maintained in the

<contents type> file attribute.  The <contents type> file

attribute value is established at the file's creation and cannot

be changed via FTAM for the life of the file.  This implies that

the data element types and ranges and data unit formats are fixed

for all accessors of that file as long as the file exists.

2.   The syntax for floating point numbers is part of the definition

of NBS abstract syntax AS1 in Annex 6A Part 3.  It is derived

from existing standards IEC 559 and IEEE 754.

6.10.1    Character Sets

Implementation of a character set in FTAM is understood as:

o    a transfer syntax is defined for the character set

o    document types are defined using the character set in

their abstract syntactic definition

o    documents of those types are stored in the Virtual File

Store as defined in the character set specification.

They are written into the VFS and read from the VFS as

defined by the abstract syntax and the transfer syntax

for the document type.  It is not in the scope of FTAM

Agreements to specify the local representation of those

documents in the Real Filestore, nor to specify

rendition of graphic characters or control characters

on character imaging devices.  These renditions are

possible agreements between applications using FTAM for

their communication.

The character sets IA5 and ISO 8859-1 shall always be

implemented.

6.10.1.1  IA5 Character Set

The International Reference Version (IRV) of IA5 is

available for use when there is no requirement to use a

national or an application-oriented version.  In information

interchange, the IRV is assumed unless a particular

agreement exists between sender and receiver of the data.

The graphic characters allocated to the IRV are as specified

in Table 6.4.

Table 6.4. IRV Graphic Character Allocations

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3 Graphic          Name                   Coded Representation 3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                              3

3    #             Number sign                     2/3         3

3                                                              3

3    o             Currency sign                   2/4         3

3                                                              3

3    @             Commercial at                   4/0         3

3                                                              3

3    [             Left square bracket             5/11        3

3                                                              3

3    \             Reverse solidus                 5/12        3

3                                                              3

3    ]             Right square bracket            5/13        3

3                                                              3

3    ^             Circumflex accent               5/14        3

3                                                              3

3    '             Grave accent                    6/0         3

3                                                              3

3    {             Left curly bracket              7/11        3

3                                                              3

3    3             Vertical line                   7/12        3

3                                                              3

3    }             Right curly bracket             7/13        3

3                                                              3

3    ~             Tilde, overline                 7/14        3

3                                                              3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

It should be noted that no substitution is allowed when using the IRV

and that the facility of combined vertical and horizontal movements of

the active position does not apply to any format effectors.

It is permitted to use composite graphic characters and there is no

limit to their number.  Because of this freedom, their processing and

imaging may cause difficulties at the receiving end.  Therefore

agreement between sender and receiver of the data is recommended if

composite characters are used.

Note:     Attention is drawn to the fact that different national

character sets exist.

(See ISO 646 or CCITT Recommendation T.50 for more information)

6.10.1.2  8859-1 Character Set

The Latin Alphabet No.1 (ISO 8859-1) is used to specify the

printable characters of G0 and G1.  C0 control characters

and their associated rules are taken from the IA5

definition.

6.10.2    Document Type Negotiation Rules

6.10.2.1  Connection Establishment

In connection establishment the <contents type list>

parameter is used only to establish presentation contexts.

Both the <document type name> form and the <abstract syntax

name> form are supported.

6.10.2.2  File Creation

An <F-CREATE request> FPDU must contain a <document type

name> value in its <initial attributes> parameter.

If the specified document type requires parameterization,

then these parameters must be supplied, otherwise the <F-

CREATE request> may be rejected.

Notes:

1.   It is understood that <permitted actions> sub-

field of <initial attributes> parameter will

always be used at <F-CREATE request>.  The value

may be changed by the Responder.

2.   If the <document type name> used requires DU

syntax parameters and one of the parameters

specifies 'FloatingPointNumber' as a primitive

data type, the request may be rejected, in case

the optional type 'FloatingPointNumber' is not

supported by the Responder.

6.10.2.3  File Opening

The <document type name> form (with appropriate parameters

as specified in 8871-2, clause 12.3) shall always be used

when proposing a <contents type>; as an alternative the

'ContentsTypeUnknown' value may be used in the <F-OPEN

request>.  An <F-OPEN response> shall use the <document type

name> option (with appropriate parameters) in the <contents

type> field.

This allows the receiving entity to use the <document type

name> attributed to the file instead of receiving a

<constraint set name> and <abstract syntax name> pair, which

does not reflect the file information contained in the FTAM

and NBS document types.

This document type name is either a value from the set of

base document type names as negotiated upon connection

establishment or a document type name, for which an

appropriate presentation context was established.

Notes:

1.   An <F-OPEN response> without a <document type name>

(but carrying the <constraint set name> and <abstract

syntax name> form) may cause the Initiator to issue an

<F-CLOSE request>.

2.   If the <document type name> used requires DU syntax

parameters and one of the parameters specifies

'FloatingPointNumber' as a primitive data type, the

request may be rejected, in case the optional type

'FloatingPointNumber' is not supported by the

Responder.

6.10.3    Relationship Between DUs, DEs and Document Types

"Abstract Syntax" is used to refer to the syntactic information

which is architecturally passed between the Application and

Presentation Layers.  The Abstract Syntax defines Data Element

(DE) types which are not necessarily ASN.1 primitive types.  A

Data Element (DE) is the smallest piece of data whose identity is

necessarily preserved by the Presentation Service.  Data types

may be made up of other data types.  Data Elements are not

defined in terms of other Data Elements.

A Data Unit (DU) is a sequence of one or more Data Elements.

Architecturally, entire, single DEs are passed into and out of

the application process.  In a real implementation, DUs may be

passed.

To maintain DU boundaries during transfer, file structuring

information must be passed (ISO8571-FADU definition in ISO 8571-

2, clause 7.5).  A Data Element is referred to as a File-

Contents- Data-Element in the ISO8571-FADU definition.

Document types refer to aspects of local processing and storage.

They describe:

o    structural relationship between DUs,

o    structure of DUs, called DU syntax, and

o    DE types found in the file.

Because document types pertain to local processing and storage,

the DU syntax makes assertions about the syntax and the size of

DUs (records) in storage.  Parameters on the document types

provide this information about the syntax and size of the DUs.

6.11 F-CANCEL ACTION

When an F-CANCEL is sent or received, the following occurs:

o    no more data is sent,

o    checkpoint numbers are removed, and

o    state of the file is implementation dependent.

Note:     When mapping F-CANCEL on P-RESYNCHRONIZE (abandon) it is

required that P-SYNC-MINOR be used after F-READ/F-WRITE (see

ISO 8571-4 clauses 13, 14).

6.12 IMPLEMENTATION INFORMATION AGREEMENTS

o    The <implementation information> parameter of <F-INITIALIZE>

FPDU is not required by these Agreements.

o    It may be used to pass user version information as a series of

values, separated by ';'.

o    The following will indicate conformance to the NBS Phase 2

Agreements:  NBS-Phase2.

Note:     The list of possible values may be enlarged for future

FTAM phases or FTAM profiles of other bodies.

o    This parameter is for information only; it is not used for

negotiation.

The establishment of an FTAM regime should not be rejected only

because of an unknown <implementation information> value.

6.13 DIAGNOSTIC AGREEMENTS

o    The <diagnostic> parameter is supported; a value in the

<response> PDU is needed when the <action result> or <state

result> is not zero.  (The nature of these agreements is to

provide <diagnostic> information when any result parameter is not

'success'.)

o    General catch-all diagnostic action is discouraged.

o    The <further details> subfield is supported.  It will be encoded

as GraphicString, but is restricted to IA5 (IRV, graphic

characters) and ISO 8859-1 only.

o    Use of F-P-ABORT for other than protocol errors and catastrophic

situations is discouraged.

o    When returning an error status in a file management related

diagnostic (i.e., <F-READ-ATTRIBUTE response> or <F-

CHANGE-ATTRIBUTE response>), identify the erroneous attribute by

using the first two characters of <further details> to hold a

2-digit number (encoded as IA5String) from the <F-READ-ATTRIBUTE

request> attributes abstract syntax definition (ISO 8571-4,

clause 20.3).

00   Filename

01   Permitted Actions

02   Contents Type

03   Storage Account

04   Date and Time of Creation

05   Date and Time of Last Modification

06   Date and Time of Last Read Access

07   Date and Time of Last Attribute Modification

08   Identity of Creator

09   Identity of Last Modifier

10   Identity of Last Reader

11   Identity of Last Attribute Modifier

12   File Availability

13   File Size

14   Future Filesize

15   Access Control

16   Legal Qualifications

17   Private Use

The set of file management diagnostics, found in ISO 8571-3 Annex

A, must be supported.

o    In the case where a specific parameter can in no way be

accommodated then the request fails and a <diagnostic> indicating

one such parameter should be returned by the responder.  In the

case where a negotiable parameter cannot be accommodated with

exactly the value requested but is negotiated to a different

value (as defined in the standard) then the request formally

succeeds but informative <diagnostics> indicating those

parameters negotiated should be returned.

o    In order to provide for robust applications using FTAM, well

defined and precise diagnostics are required to be returned by

responding implementations whenever an action cannot be carried

out precisely as requested with respect to non-negotiable

parameters.  All such applicable diagnostics will be returned in

those cases.  An action is carried out precisely as requested

with  respect to a parameter when the value of that parameter on

the <request> FPDU is equal to the value in effect during or

subsequent to the action, depending on whether the action is

regime control.

Diagnostics exist to signal 'parameter not supported' and

Responder implementations shall issue all appropriate

diagnostics.  The <further details> subfield of the <diagnostic>

parameter shall specify the parameter which is not implemented.

6.14 CONCURRENCY

The <concurrency control> used by default on actions requested by an

<F-SELECT indication> or <F-CREATE indication> service are:

'shared'       for read and read attribute

'exclusive'    for all other actions

The default for actions not requested is specified as 'not required'

as per ISO 8571-3.

Note: A local implementation may choose to be more restrictive in

order to assure file consistency for concurrent accessors.

FADU locking is not required.

6.15 REQUESTED ACCESS

The <requested access> parameter on <F-SELECT> or <F-CREATE> is used

to specify the actions which the Initiator may perform during the file

selection.  The value of the <requested access> parameter is compared

by the Responder to the <access control> and <permitted actions> file

attributes and concurrency controls (including those requested by the

Initiator) currently in place on the file.  If the value of the

<requested access> parameter is not consistent with either <access

control>, <permitted actions>, or concurrency controls in place, then

the <F-SELECT> or <F-CREATE> must be rejected.

<requested access> is consistent with <access control> if, for each

action requested, that action either requires no password, or the

required password has been specified on the <F-SELECT request> or <F-

CREATE request>.

<requested access> is consistent with <permitted actions> if, for each

action requested, that action is allowed by the <permitted actions>

file attribute.

<requested access> is consistent with <concurrency control> requested

on the <F-SELECT> or <F-CREATE> if, for each action requested, that

action has not been specified as 'not required' or 'no access' in the

<concurrency control> parameter.

<requested access> is consistent with concurrency controls in place on

the file if for each action requested no other accessor of the file

has set the concurrency control for that action to either 'exclusive'

or 'no access'.

6.16 SECURITY

6.16.1    Initiator Identity and Filestore Password

The <initiator identity> and <filestore password> parameters for

an implementation acting as an Initiator are supported.  These

parameters are optional for an implementation acting as a

Responder.

The syntax of <initiator identity> and <filestore password> is

system-dependent.  <initiator identity> and <filestore password>

will represent account information on the local system, which

may be different from the <account> parameter.

6.16.2    Access Passwords

The <access passwords> and <create password> parameters for an

implementation acting as an Initiator are supported if the

Security Group of attributes is supported.  These parameters for

an implementation acting as a Responder are optionally supported

if the Security Group is supported.

6.16.3    Implementation Responsibilities

It is the responsibility of each local system to provide security

for its own real filestore.  Encryption of passwords will not be

done by FTAM.

A user of the file service must be known by the Responder.

"Known" is defined by the local Filestore, and is dependent on

the level of security provided by the local Filestore.

6.17 REQUIREMENT FOR CONFORMANT IMPLEMENTATIONS

This section gives the criteria to be satisfied by every

implementation of FTAM that conforms to these Agreements.

Conformance to these Agreements is stated in terms of the different

roles occupied by FTAM implementations.  The interoperability of

certain configurations of these roles motivates this approach.

Interoperable configurations of these roles are given in section

6.17.1.

The only function provided by every conformant implementation is the

transfer of unstructured binary files in their entirety.  It must be

recognized that such simple transfer, while commonly understood and

generally important, will not support all applications of FTAM.

Section 6.18 defines Implementation Profiles of FTAM services and

protocol that can provide other specific functions.  Those other

functions exploit the access and management capabilities of FTAM. The

unconstrained service class (with appropriately chosen functional

units) can be used to provide the functions of any of the

Implementation Profiles.  Users of FTAM must consider carefully what

functions they require.  They must examine all the Implementation

Profiles and select according to their needs.

Implementation conforming to these Agreements require adherence to the

General Agreements in sections 6.5 through 6.16 of these Agreements.

6.17.1    Interoperable Configurations

Any implementation conforming to this specification must be able

to act in at least one of the following role combinations:

1.   initiator and receiver,

2.   initiator and sender,

3.   responder and sender,

4.   responder and receiver.

Minimal implementations of combination 1 will interoperate with

minimal implementations of combination 3.  Minimal

implementations of combination 2 will interoperate with minimal

implementations of combination 4.

Any implementations of roles l and 3 will be able to interoperate

at the intersection of their capabilities (which will be at least

the minimal capabilities described in sections 6.17.3 to 6.17.8).

Any implementations of roles 2 and 4 will be able to interoperate

at the intersection of their capabilities (which will be at least

the minimal capabilities described in sections 6.17.3 to 6.17.8).

These role combinations and this interoperability are shown in

Table 6.5 below.

Table 6.5  Interoperable configurations

ZDDDDDDDDDDDDDDDDDDDDDDDRDDDDDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDDDDDDDDDDD?

3                       :      Initiator       3      Responder        3

3                       GDDDDDDDDDBDDDDDDDDDDDDEDDDDDDDDDBDDDDDDDDDDDDD4

3                       :  sender 3  receiver  3  sender 3  receiver   3

3MMMMMMMMMMMQMMMMMMMMMMMNMMMMMMMMMXMMMMMMMMMMMMXMMMMMMMMMXMMMMMMMMMMMMM3

3           3 sender    :         3            3         3      x      3

3 Initiator CDDDDDDDDDDDWDDDDDDDDDEDDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDD3

3           3 receiver  :         3            3    x    3             3

CDDDDDDDDDDDEDDDDDDDDDDDWDDDDDDDDDEDDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDD4

3           3 sender    :         3     x      3         3             3

3 Responder CDDDDDDDDDDDWDDDDDDDDDEDDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDD4

3           3 receiver  :    x    3            3         3             3

3           3           :         3            3         3             3

@DDDDDDDDDDDADDDDDDDDDDDPDDDDDDDDDADDDDDDDDDDDDADDDDDDDDDADDDDDDDDDDDDDY

6.17.2    Relationship to ISO 8571--The FTAM Standard

Any implementation in conformance to ISO 8571 (as defined in ISO

8571-4, clause 22 (Conformance)), in addition to the

implementation of the minimal protocols and roles enumerated in

sections 6.17.3 to 6.17.8, is considered to be in conformance

with these Agreements.  Any implementation violating any of the

conformance statements in ISO 8571-4 is considered to be in

violation of these Agreements.

6.17.3    Requirements for Document Type Support

The document type FTAM-3 shall be supported for purposes of

transfer and storage.  The details regarding support for FTAM-3

in the FTAM dialogue are given in section 6.10.

Support of document types other than FTAM-3 is not required for

conformant implementations.  Support for document types described

in these Agreements also entails support for:

o    the semantics given in their description and further

qualified in 6.10

o    the preferred transfer syntax "Basic Encoding of a

single ASN.1 type"

6.17.4    Initiators

Every implementation of an FTAM Initiator shall support:

o    the kernel protocol and its mandatory parameters with

minimum ranges [Minimum required ranges are specified

in section 6.17.8.],

o    the grouping protocol and the <threshold> parameter

with a value of at least 2 for use in the file transfer

class,

o    at least one of the read or write protocols [Specific

conformance for reading and writing is defined in

sections 6.17.6 and 6.17.7.],

and support the applicable procedures defined in ISO 8571-4

clauses 8.1 (FTAM regime establishment), 8.2 (FTAM regime

termination), 8.3 (File selection), 8.4 (File deselection), 8.9

(File open), 8.10 (File close), 8.11 (Begin group), 8.12 (End

group), and 10 (File general actions).  To support the above

protocols and procedures the implementation shall always support

the kernel functional unit and additionally shall be able to:

o    request the grouping and at least one of the read or

write functional units,

o    request the file transfer class with the <service

class> parameter,

o    request the document type FTAM-3 using the <document

type name> form of the <contents type> parameter,

o    request the <FTAM quality of service> parameter with

value 0 and accept in all cases the returned value 0,

and

o    request a <communication quality of service> consistent

with the transport definition in these Agreements

as part of the Filestore initialization procedures in ISO 8571-4

clause 8.1, FTAM regime establishment.

Initiators must be able to operate under all circumstances if the

above minimum values are successfully negotiated and returned on

an <F-INITIALIZE response> PDU.  Initiators must be able to

operate with any downward negotiation of requested parameter

values as described in the standard.

Should the supporting services break down, such that FTAM

communication is impossible, the FTAM protocol machine shall

notify the user with an <F-P-ABORT indication> and <diagnostic>

value with identifier 1011, as well as any known <further

details>.

Note: Interworking may not be possible between Initiators not

supporting attributes of the Storage Group and Security Group,

and Responders requiring these attributes to be used.

6.17.5    Responders

Every implementation of an FTAM Responder shall support:

o    the kernel protocol and its mandatory parameters with

minimum ranges [Minimum required ranges are specified

in section 6.17.8.],

o    the grouping protocol and the <threshold> parameter

with a value of at least 2 for use in the file transfer

class,

o    at least one of the read or write protocols [Specific

conformance for reading and writing is defined in

sections 6.17.6 and 6.17.7],

and support the applicable procedures, defined in ISO 8571-4

clauses 9.1 (FTAM regime establishment), 9.2 (FTAM regime

termination), 9.3 (File selection), 9.4 (File deselection), 9.9

(File open), 9.10 (File close), 9.11 (Begin group), 9.12 (End

group), and 10 (File general actions).  To support the above

protocols and procedures the implementation shall always support

the kernel functional unit and additionally shall be able to:

o    accept requests for the grouping and at least one of

the read or write functional units,

o    accept requests for the file transfer class with the

<service class> parameter,

o    accept the document type FTAM-3 using the <document

type name> form of the <contents type> parameter,

o    accept requests for an <FTAM quality of service>

parameter with any value but may respond with the value

0, and

o    accept requests for a <communication quality of

service> consistent with the transport definition in

these agreements

as part of the filestore initialization procedures in ISO 8571-4

clause 9.1, FTAM regime establishment.

Responders must be able to operate under all circumstances if the

above minimum values are requested on an <F-INITIALIZE request>

PDU. Responders must not negotiate upward in the sense described

in the standard.

Responders must complete each action requested and supported in a

manner consistent with its description in ISO 8571-2 clauses 10

(Actions on complete files) and 11 (Actions for file access), and

must interpret each supported attribute in a manner consistent

with its definition in ISO 8571-2 clause 12 (File attributes).

Under circumstances where actions cannot be carried out either as

requested or consistently with ISO 8571-2 clause 10 (Actions on

complete files) and 12 (Actions for file access), the Responder

must return at least one diagnostic indicating:

o    if the failure was due to either a protocol or

Filestore failure, and then:

--   precisely which action failed,

--   at least one of the parameters that could not be

accommodated  with the diagnostic type indicating

at least the degree of failure, as given by the

action and state result parameter, or

o    that the failure was due to unforeseen system shutdown.

Should the supporting services break down, such that FTAM

communication is impossible, the FTAM protocol machine shall

notify the user with an <F-P-ABORT indication> and <diagnostic>

with identifier 1011, as well as inform the user of any known

<further details>.

6.17.6    Senders

Every implementation of an FTAM sender shall support the read

functional unit as Responder or the write functional unit as

Initiator, and support the applicable procedures defined in ISO

8571-4 clauses 11 (State of the bulk data transfer activity), 12

(Bulk data transfer protocol data units), 15 (Bulk data transfer

sending entity actions), 17.1 (Discarding), and 17.2 (Cancel).

To support those procedures the implementation shall be able to

send files of the document type FTAM-3 and shall be able to send

them as user data in PPDUs in blocks of up to 7168 octets.

6.17.6.1  Initiator Senders

Every implementation of an FTAM sender which is also an FTAM

Initiator shall support:

o    the write functional unit and protocol, and

o    for the document type FTAM-3 the following bulk

data transfer specification parameters:

FADU operation      replace

FADU identity       first

and support the applicable procedures, defined in ISO 8571-4

clause 13 (Bulk data transfer initiating entity actions).

6.17.6.2  Responder Senders

Every implementation of an FTAM sender which is also an FTAM

Responder shall support:

o    the read functional unit and protocol, and

o    for the document type FTAM-3 the following bulk

data transfer specification parameters:

FADU identity       first

Access context      UA

and support the applicable procedures, defined in ISO 8571-4

clause 14 (Bulk data transfer responding entity actions).

6.17.7    Receivers

Every implementation of an FTAM receiver shall support the read

functional unit as Initiator or the write functional unit as

Responder, and support the applicable procedures, defined in ISO

8571-4 clauses 11 (State of the bulk data transfer activity), 12

(Bulk data transfer protocol data units), 16 (Bulk  data transfer

receiving entity actions), 17.1 (Discarding), and 17.2 (Cancel).

To support those procedures the implementation shall be able to

receive files of the document type FTAM-3 and shall be able to

receive them as user data in PPDUs in blocks of at least 7168

octets.

6.17.7.1  Initiator Receivers

Every implementation of an FTAM receiver which is also an

FTAM Initiator shall support:

o    the read functional unit and protocol, and

o    for the document type FTAM-3 the following bulk

data transfer specification parameters:

FADU identity       first

Access context      UA

and support the applicable procedures, defined in ISO 8571-4

clause 13 (Bulk data transfer initiating entity actions).

6.17.7.2  Responder Receivers

Every implementation of an FTAM receiver which is also an

FTAM Responder shall support:

o    the write functional unit and protocol, and

o    for the document type FTAM-3 the following bulk

data transfer specification parameters:

FADU operation      replace

FADU identity       first

and support the applicable procedures, defined in ISO 8571-4

clause 14 (Bulk data transfer responding entity actions).

6.17.8    Minimum Ranges

Any implementation of any conformant FTAM configuration shall be

able to receive and meaningfully process all mandatory parameters

for all functional units supported as well as the <diagnostic>

parameter within at least the minimum ranges of values given in

Table 6.6.  A conforming implementation may support a wider range

of values for any parameter.

Table 6.6  Required minimal parameter support

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3      Parameter                               Minimum Range                 3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3general               diagnostic  Values as specified in ISO 8571-3 Annex A 3

3                                  (Diagnostic parameter values) Tables 44,  3

3                                  45 and 46 which correspond directly to    3

3                                  mandatory parameters.                     3

3                   action result  All values.                               3

3                    state result  All values.                               3

3                                                                            3

3F_INITIALIZE                                                                3

3                                                                            3

3               functional units1  'read' (for initiator/receivers and       3

3                                  responder/senders) and 'grouping'         3

3                                  or                                        3

3                                  'write' (for initiator/senders and        3

3                                  responder/receivers) and 'grouping'       3

3presentation context management2   'Not required.'                          3

3                      all others  As specified in sections 6.17.4 and       3

3                                   6.17.5 above.                            3

3                                                                            3

3                                                                            3

3F_SELECT                                                                    3

3                      attributes  Only <filename> is used with a minimum    3

3                                  supportable length of 8 characters.  Any  3

3                                  other attribute supported for other       3

3                                  services must have minimum supported      3

3                                  lengths as in ISO 8571-2 clause 15        3

3                                  (Minimum attribute ranges) Table 2.       3

3                                                                            3

3               requested access   'read' for initiator/receivers            3

3                                  'read' for responder/senders              3

3                                  'replace' for initiator/senders           3

3                                  'replace' for responder/receivers         3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 6.6  Required minimal parameter support, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3      Parameter                               Minimum Range                 3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3F_OPEN                                                                      3

3                  processing mode  'read' for initiator/receivers           3

3                                   'read' for responder/senders             3

3                                   'replace' for initiator/senders          3

3                                   'replace' for responder/receivers        3

3                                                                            3

3                  contents type   'FTAM-3'                                  3

3                                                                            3

3F_READ                                                                      3

3                  FADU identity   'first'                                   3

3                  access context  'UA'                                      3

3F_WRITE                                                                     3

3                  FADU operation  'read' for initiator/receivers            3

3                                  'read' for responder/senders              3

3                                  'replace' for initiator/senders           3

3                                  'replace' for responder/receivers         3

3                                                                            3

3                  FADU identity   'first'                                   3

3                                                                            3

3F_BEGIN_GROUP                                                               3

3                                                                            3

3                      threshold3  For file transfer (a minimal required     3

3                                  function) 2.                              3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

For any other supported parameters, minimum ranges are taken from

the minimum ranges for the attribute corresponding to each as in

ISO 8571-2 Table 4.

DDDDDDDDDDDDDDD

Notes:

1.   The parameters, functional units, and presentation context

management are not ordered, so "minimum value" cannot be

formally defined.  The above values are those required for

conformance to these Agreements but no value conformant to

ISO 8571 for use in other applications is regarded to be in

violation of these Agreements.

2.   Other functional units (and service classes) for defined

implementations may also be valid provided that they are

implemented in accordance with these Agreements,

specifically section 6.17.8.

3.   Every implementation must support the <threshold> value 2 to

provide the basic required function of file transfer; any

other value in other applications is acceptable.

6.17.9    Use of Lower Layer Services

o    Support for the Presentation Context Management

functional unit is not required.

o    Implementations will support the Session, Presentation,

and ACSE requirements as stated in section 5.

Note:  Implementation of the Session Resynchronize  and the  Minor

Synchronize functional units is highly recommended, since the F-

CANCEL service may be less effective when mapped to S-DATA.

6.18 IMPLEMENTATION PROFILES

This section defines Implementation Profiles for the specific

functions of:

o    File Transfer

o    File Access

o    File Management.

Those definitions are expressed in terms of:

o    Document Types

o    Attributes

o    Service Classes (both service elements and their

parameters).

This by no means defines all possible Implementation Profiles.

The following Implementation Profiles are defined:

T1:  Simple File Transfer

T2:  Positional File Transfer

T3:  Full File Transfer

A1:  Simple File Access

A2:  Full File Access

M1:  Management.

Implementation Agreements have been reached for the following service

classes.  Note that any given implementation may support more than one

service class.

o    File Transfer

o    File Access

o    File Management

o    Unconstrained

o    File Transfer and Management

Support of an Implementation Profile requires adherence to:

1.   corresponding definition in  8571-3 clause 8 and any related

procedures in 8571-4 clause 8-17,

2.   requirements given in sections 6.5-6.18 of these Agreements, and

3.   requirements for parameter and attribute support as defined in

section 6.17.8.

6.18.1    General Requirements for the Defined Implementation

Profiles

o    Implementations will be able to act either as Initiator or

Responder or both.

o    Implementations must support diagnostics as described in

section 6.13 of these Agreements.

o    Implementations that support the file access service class

will support access to sequential files.  Support of

sequential files entails hierarchy of depth and arc length

equal to 1.  Other hierarchy depth and arc lengths are not

precluded by these agreements.

6.18.2    Intentionally Left Empty

6.18.3    Document Type Requirements for the Defined

Implementation Profiles

Implementations conformant to Implementation Profiles defined in

Table 6.7 will support the following document types with the

caveats and procedures given.  Those document types are defined

in Appendix 6A and section 6.10 of these Agreements, and in ISO

8571-2.

o    FTAM-1

o    FTAM-2

o    FTAM-3

o    NBS-6

o    NBS-7

Note:     Support of this document type entails the

naming of FADUs by  their position in

preorder traversal.

Caveat:   Other methods of naming FADUs depend on the

system, application, and specific file, and

as such are not described here.

o    NBS-8

o    NBS-9

Support for any document type requires the ability to transfer

and store the abstract syntax given in its definition.  These

Agreements do not specify techniques or formats for storage.

Caveat:   Specific abstract syntaxes for the parameterized

document types NBS-6,7,8 are not specified in these

Agreements.

Any document type supported must be identifiable by its document

type name as given in ISO 8571-2 and in Appendix 6A of these

Agreements and, where defined, the parameterization scheme given

in section 6.10 of these Agreements.

For conformance to NBS-9 a Responder is only required to return

the <filename> attribute, subject to local security and access

control.  All other requested attributes need not be returned.

Systems supporting the NBS-9 document type shall make available

an NBS-9 document called 'DIRLIS'.  This document can be used to

obtain a listing of files and their associated attributes from a

remote Filestore.

File security issues related to NBS-9 are subject to the security

agreements outlined in section 6.16.

6.18.4    Parameters for the Defined Implementation Profiles

o

o

o    Implementations will support the <contents type list>

parameter on the  <F-INITIALIZE> service element.  The

initiating service must supply a value for this

parameter.

o    Implementations will support the <diagnostic> parameter

as stated in section 6.13 of these Agreements.

o    The <initiator identity> parameter is supported.    Use

must be consistent with section 6.16 of these

Agreements.

o    Implementations are not precluded from using other

parameters for security and/or accounting.  Responders

must state the semantics applying to <account> and

<charging> parameters.  The Responder's minimum

implementation is to accept but ignore the <account>.

and to return a <charging> value of zero.

6.18.5    Parameter Ranges for the Defined Implementation

Profiles

Parameter ranges for Implementations Profiles are as stated for

primitive data types in section 6.10 of these agreements.

6.18.6    File Attribute Support for Implementations

Implementations of the Implementation Profiles will support file

attributes or attribute groups in the following ways.

o    mandatory

This feature is mandatory in the ISO 8571-2

standard and shall therefore be implemented by all

implementations claiming conformance to these

Agreements.

o    supported

This feature shall be implemented by all

implementations claiming conformance to these

Agreements (for attributes, this implies that at

least the minimum range of attribute values, as

defined in ISO 8571-2 clause 15, shall be

supported).  Conformant implementations shall also

be able to interwork with other implementations

that do not support this feature by negotiating

out the corresponding features.

o    optionally supported

Implementations claiming conformance to these

Agreements may or may not implement this feature

(for attributes, this implies that at least either

the minimum range of attribute values, as defined

in ISO 8571-2 clause 15, shall be supported or

that the 'no value available' result shall be

supplied).  If an attribute group with a support

level of 'optionally supported' is chosen to be

supported, then all the attributes of this group

that are classified as 'mandatory' or 'supported'

shall be supported.

o    not supported

This feature is outside the scope of these

Agreements.

Kernel Group                                         mandatory

o Filename                                        mandatory

o Permitted Actions                               mandatory

o Contents Type                                   mandatory

Storage Group                                        optionally supported

o Storage Account                                 optionally supported

o Date and Time of Creation                       optionally supported

o Date and Time of Last Modification              optionally supported

o Date and Time of Last Read Access               optionally supported

o Date and Time of Last Attribute Modification    optionally supported

o Identity of Creator                             optionally supported

o Identity of Last Modifier                       optionally supported

o Identity of Last Reader                         optionally supported

o Identity of Last Attribute Modifier             optionally supported

o File Availability                               supported

o Filesize                                        supported

o Future Filesize                                 optionally supported

Security Group                                       optionally supported

o Access Control                                  supported

o Legal Qualifications                            optionally supported

Private Group                                        not supported

Table 6.7 Implementation profile support requirements

ZDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                          3              Service Class                      3

3 Functional Unit          3    T        M         A        T&M      UNCST   3

CDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDDDDDBDDDDDDDDDBDDDDDDDDDBDDDDDDDDDBDDDDDDDDD4

3                          3         3         3         3         3         3

3 Kernel                   3 T1,T2,T33   M1    3   A1,A2 3         3         3

3 Read     (See note 3.)   3 T1,T2,T33         3   A1,A2 3         3         3

3                          3         3         3         3         3         3

3 Write    (See note 3.)   3 T1,T2,T33         3   A1,A2 3         3         3

3                          3         3         3         3         3         3

3 Limited File Mgmnt.      3SeeNote 63   M1    3SeeNote 63   See   3   See   3

3 Enhanced File Mgmnt.     3         3   M1    3         3         3         3

3 Grouping                 3 T1,T2,T33   M1    3         3         3         3

3 File Access              3         3         3   A1,A2 3         3         3

3                          3         3         3         3         3         3

3       Document Types     3         3         3         3         3         3

3                          3         3         3         3         3         3

3 FTAM-1                   3 T1,T2,T33  [M1]   3   A1,A2 3         3         3

3 FTAM-2                   3 T2,T3   3  [M1]   3   A1,A2 3         3         3

3 FTAM-3                   3 T1,T2,T33  [M1]   3   A1,A2 3   Note  3   Note  3

3                          3         3         3         3         3         3

3 NBS-6                    3 [T2],T3 3  [M1]   3  [A1],A23    4    3    5    3

3                          3         3         3         3         3         3

3 NBS-7                    3 [T2],T3 3  [M1]   3  [A1],A23         3         3

3                          3         3         3         3         3         3

3 NBS-8                    3 T3      3  [M1]   3   A2    3         3         3

3                          3         3         3         3         3         3

3 NBS-9                    3[T1],[T2]3  [M1]   3         3         3         3

3                          3[T3]     3         3         3         3         3

@DDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDDDDDADDDDDDDDDADDDDDDDDDADDDDDDDDDADDDDDDDDDY

Notes: to 6.18.3 and Table 6.7

1. The Management Implementation Profile is only to be implemented in

conjunction  with one of the Transfer or Access Profiles.

2. Profile T2 is subset of T3.  A1 and T1 are subsets of A2 and T2,

respectively.

3. Profiles T1, T2, and T3 require the support of read and/or write

functional units.

4. Support of the <File Transfer and Management> service class is

optional.  If an implementation is capable of supporting

Implementation Profile M1 and one of the T-Implementation Profiles,

the Initiator may choose to request the <File Transfer and Management>

service class.  Any implementation so doing must be prepared for the

possibility of rejection of the request by the Responder.

Support of the <File Transfer and Management> service class is

optional.  The rules for including it in a request and for the

response to it are as given in ISO 8571-3, clause 10.1.  Any

implementation including TM in the request must be prepared for the

possibility that it might be removed from the response.

5. The support of the <Unconstrained> service class is optional.  There

are no constraints on this service class beyond those of ISO 8571.

6. Limited File Management is not required for the T- and A-

Implementation Profiles, but very often it will be a user request to

have limited file management functionality available together with

file transfer and file access functions.  So Limited File Management

may be added as an option to the T- and A- Implementation Profiles.

7. [] in Table 6.7 specifies that the document type is optional for the

respective Implementation Profile.  For M1 the support level depends

on the T- or A- Implementation Profile, in conjunction with which M1

is implemented.

6.19 PROVISION OF SPECIFIC FUNCTION

6.19.1    Implementation Profile T1:  Simple File Transfer

Implementation Profile T1 provides the function of transferring

entire files at the external file service level for files with an

unstructured constraint set.  This includes support of the

document types:

o    FTAM-1    "ISO FTAM unstructured text"

o    FTAM-3    "ISO FTAM unstructured binary"

o    NBS-9     "NBS-9 file directory file"        (optional)

This Implementation Profile supports file transfer and not file

access, that is, the ability to:

o    read a complete file

and/or

o    write (replace, extend) to a file.

6.19.2    Implementation Profile T2:  Positional File Transfer

Implementation Profile T2 provides the function of transferring

files at the external file service level for files with an

unstructured or flat constraint set.  This includes support of

the document types:

o    FTAM-1    "ISO FTAM unstructured text"

o    FTAM-2    "ISO FTAM sequential text"

o    FTAM-3    "ISO FTAM unstructured binary"

o    NBS-6     "NBS-6 FTAM sequential file"       (optional)

o    NBS-7     "NBS-7 FTAM random access file"    (optional)

o    NBS-9     "NBS-9 file directory file"        (optional)

This Implementation Profile supports file transfer and not file

access, that is, the ability to:

o    read a complete file or a single FADU which is

identified by position

and/or

o    write (replace, extend, insert depending on constraint

set and document type) to a file or an FADU.

This Implementation Profile is upwardly compatible to T1 for the

transfer of unstructured files.

6.19.3    Implementation Profile T3:  Full File Transfer

Implementation Profile T3 provides the function of transferring

files at the external file service level for files with an

unstructured, flat or general hierarchical constraint set.  This

includes support of the document types:

o    FTAM-1    "ISO FTAM unstructured text"

o    FTAM-2    "ISO FTAM sequential text"

o    FTAM-3    "ISO FTAM unstructured binary"

o    NBS-6     "NBS-6 FTAM sequential file"

o    NBS-7     "NBS-7 FTAM random access file"

o    NBS-8     "NBS-8 FTAM indexed file"

o    NBS-9     "NBS-9 file directory file"        (optional)

This Implementation Profile supports file transfer and not file

access, that is, the ability to:

o    read a complete file or a single FADU which is

identified by key or by position

and/or

o    write (replace, extend, insert depending on constraint

set and document type) to a file or an FADU.

This Implementation Profile is upwardly compatible to T1 for the

transfer of unstructured files.

6.19.4    Implementation Profile A1: Simple File Access

Implementation Profile A1 provides the function of transfer of

and access to files with unstructured or flat constraint sets at

the external file service level.  This includes support of the

document types:

o    FTAM-1    "ISO FTAM unstructured text"

o    FTAM-2    "ISO FTAM sequential text"

o    FTAM-3    "ISO FTAM unstructured binary"

o    NBS-6     "NBS-6 FTAM sequential file"       (optional)

o    NBS-7     "NBS-7 FTAM random access file"    (optional)

This Implementation Profile supports file transfer and file

access, that is the ability to:

o    read a complete file or FADUs which are identified by

position,

o    write (replace, extend, insert depending on constraint

set and document type) to a file or an FADU,

o    locate and erase within files.

6.19.5    Implementation Profile A2:  Full File Access

Implementation Profile A2 provides the function of transfer of

and access to files with unstructured or flat constraint sets at

the external file service level.  This includes support of the

document types:

o    FTAM-1    "ISO FTAM unstructured text"

o    FTAM-2    "ISO FTAM sequential text"

o    FTAM-3    "ISO FTAM unstructured binary"

o    NBS-6     "NBS-6 FTAM sequential file"

o    NBS-7     "NBS-7 FTAM random access file"

o    NBS-8     "NBS-8 FTAM indexed file"

This Implementation Profile supports file transfer and file

access, that is, the ability to:

o    read from a complete file, or from a series of FADUs

which are identified by key or by position,

o    write (replace, extend, insert depending on constraint

set and document type) to a file or an FADU,

o    locate and erase within files.

6.19.6    Implementation Profile M1:  Management

Implementation Profile M1 provides the function for an Initiator

to manage the files within the Virtual Filestore, to which access

is provided by the Responder.  Management includes the services

of:

o    creating a file

o    deleting a file

o    reading attributes of a file

o    changing attributes of a file.

6.20 HARMONIZATION

The Implementation Profiles for File Transfer, File Access and

Management correspond to the Profiles of SPAG (Standards Promotion and

Application Group) in Europe, so that interworking will be possible.

Those Profiles are described in the 'Guide to the Use of Standards'

(GUS); they are the basis for the Functional Standards as defined by

CEN/CENELEC (Comite Europeenne de Normalization).

Table 6.8 Implementation Profiles (NBS) and Profiles (SPAG/CEN-CLC)

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3  Implementation Profile         SPAG/CEN-CLC        3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDDDDDDDDDDD4

3                             3                       3

3         T1                  3          A/111        3

3         T2                  3          A/112        3

3         T3                  3          A/113        3

3         A1                  3          A/122        3

3         A2                  3          A/123        3

3         M1                  3          A/13         3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDDDDDDDDDDDDDDDDDDDY

6.21   APPENDIX A:    FTAM DOCUMENT TYPES

Part 1:  Document Types

Part 2:  Constraint Sets

Part 3:  Abstract Syntaxes

Part 4:  Intentionally Left Empty

Part 1:  Document Types

NBS-6 Sequential file document type

1. Entry Number:   NBS-6

2. Information objects

Table 6.9 Information objects in NBS-6

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDRDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3 document type name            : {iso identified-organization icd (9999)     3

3                               : organization-code  (1) document             3

3                               : type (5) sequential (6)}                    3

3                               :           "NBS-6 FTAM sequential file"      3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 abstract syntax names:        : {iso identified-organization icd (9999)     3

3 a) name for asname1           : organization-code  (1) abstract-            3

3                               : syntax (2) nbs-as1 (1)}                     3

3                               :           "NBS abstract syntax AS1"         3

3 b) name for asname2           : {iso standard 8571 abstract-syntax(2) ftam- 3

3                               :  fadu (2)}                                  3

3                               :                 "FTAM FADU"                 3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 transfer syntax names:        : {joint-iso-ccitt asn1 (1) basic-encoding (1)3

3                               : }   "Basic Encoding of a single ASN.1 type" 3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMJMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3parameter syntax:                                                            3

3  PARAMETERS ::= SEQUENCE OF CHOICE {Parameter0, Parameter1, Parameter2}     3

3  Parameter0 ::= universal-class-number-0 [0] INTEGER {univer-time (23),     3

3                                                       gen-time (24),        3

3                                                       boolean (1),          3

3                                                       null (5) }            3

3 Parameter1  ::= [1] SEQUENCE  {                                             3

3                 universal-class-number-1 INTEGER { int       (2),           3

3                                                    bit       (3),           3

3                                                    ia5      (22),           3

3                                                    graphic  (25),           3

3                                                    general  (27),           3

3                                                    octet     (4)},          3

3                 string-length INTEGER }                                     3

3 Parameter2  ::= [2] SEQUENCE {                                              3

3                 private-class-number INTEGER {float (0)},                   3

3                 length-1     INTEGER,                                       3

3                 length-2     INTEGER }                                      3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMKMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 file model                    : {iso standard 8571 file-model (3)           3

3                               :  hierarchical (1)}                          3

3                               :  "FTAM hierarchical file model"             3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 constraint set                : {iso standard 8571 constraint-set (4)       3

3                               :  sequential-flat (2)}                       3

3                               :  "FTAM sequential flat constraint set"      3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMJMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 file contents:                                                              3

3                Datatype1 ::= PrimType -- as defined in Annex 6 A, Part 3    3

3                Datatype2 ::= CHOICE   { Node-Descriptor-Data-Element,       3

3                                         Enter-Subtree-Data-Element , Exit-  3

3                                         Subtree-Data-Element}               3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

3. Scope and field of application

The document type defines the contents of a file for storage, for

transfer and access by FTAM.

Note: Storage refers to apparent storage within the Virtual Filestore.

4. References

ISO 8571, Information Processing Systems - Open Systems

Interconnection - File Transfer, Access and Management

5. Definitions

This definition makes use of the terms data element, data unit and

file access data unit as defined in ISO 8571-1.

6. Abbreviations

FTAM File Transfer, Access and Management

7. Document semantics

The document consists of zero, one or more file access data units,

each of which consists of zero, one or more data elements.  The order

of each of these elements is significant.

The document structure takes any of the forms allowed by the FTAM

hierarchical file model as constrained by the sequential flat

constraint set (see Table 6.9)  These definitions appear in ISO 8571-

2.  As additional constraints FADU identity will be limited to

'begin', 'end', 'first' and 'next'.

For a specific file the number of data elements in a data unit is

given by the parameters.  Each data element is a data type from the

set of primitive data types defined in the Annex 6.A, Part 3 of this

document.  Each data unit contains the same data element types in the

same order as all other data units.  These types are determined by the

parameters 0 through 2.

Note: The string length values are the actual number of characters

from the specified character set, they do not include any escape

sequences or overhead from the encoding.

8. Abstract syntactic structure

The abstract syntactic structure of the document is a hierarchically

structured file as defined in the ASN.1 module ISO8571-FADU in ISO

8571, in which each of the file access data units has the abstract

syntactic structure of NBS-AS1 as defined by the parameters.

9. Definition of transfer

9.1     Datatype definitions

The file consists of data values which are of either

a)   Datatype1 defined in Table 6.9, where the PrimType in

the datatype is given by the NBS-AS1 definition; or

b)   Datatype2 defined in Table 6.9, the ASN.1 datatype

declared as "Data-Element" in the ASN.1 module ISO8571-

FADU.

9.2     Presentation data values

The document is transferred as a series of presentation data

values, each of which is either

a)   one value of the ASN.1 datatype "Datatype1", carrying

one of the data elements from the document.  All values

are transmitted in the same (but any ) presentation

context defined to support the abstract syntax name

"asname1" or

b)   a value of "Datatype2".  All values are transmitted in

the same (but any) presentation context defined to

support the abstract syntax name "asname2".

Notes:

1.   Specific carrier standards may impose additional constraints

on the presentation context to be used, where the above

permits a choice

2.   Any document type defined in this entry either makes no use

of Datatype2, or starts with a Datatype2 transmission.

Boundaries between presentation data values in the same

presentation context, and boundaries between P-DATA primitives,

are chosen locally by the sending entity at the time of

transmission, and carry no semantics of the document type.

Receivers which support this document type shall accept a

document with any of the permitted transfer options (e.g.

document type parameters and transfer syntaxes).

9.3     Sequence of presentation data values

The sequence of presentation data values of type a) and the

sequence of presentation data values of types a) and b) is the

same as the sequence of data elements within a Data Unit, and

Data Units in the hierarchical structure, when flattened

according to the definition of the hierarchical file model in ISO

8571-2.

10.     Transfer syntax

An implementation supporting this document type shall support the

transfer syntax generation rules named in Table 6.9 for all

presentation data values transferred.  An implementation may

optionally support other named transfer syntaxes.

11.     ASE specific specifications for FTAM

11.1    Simplification and relaxation

11.1.1  Structural simplification

This simplification loses information.

The document type NBS-6 may be simplified to the document type

FTAM-3 (allowed only when reading the file).  The octet

representation of the transferred data is unpredictable.  It will

usually correspond to the data values as stored in the local Real

Filestore of the Responder.

11.2    Access context selection

A document of type NBS-6 may be accessed in any one of the access

contexts defined in the sequential flat constraint set.  The

presentation data units transferred in each case are those

derived from the structuring elements defined for that access

context in ISO 8571-2.

11.3    The INSERT operation

When the <INSERT> operation is applied at the end of file the

transferred material shall be the series of FADUs which would be

generated by reading any NBS-6 document with the same parameter

values in access context FA.

NBS-7 Random access file

1. Entry number:   NBS-7

2. Information objects

Table 6.10 Information objects in NBS-7

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDRDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3 document type name            : {iso identified-organization icd (9999)     3

3                               : organization-code  (1) document             3

3                               : type (5) random-file (7)}                   3

3                               :              "NBS-7 FTAM random access file"3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 abstract syntax names:        : {iso identified-organization icd (9999)     3

3 a) name for asname1           : organization-code  (1) abstract-            3

3                               : syntax (2) nbs-as1 (1)}                     3

3                               :         "NBS abstract syntax AS1"           3

3 b) name for asname2           : {iso standard 8571 abstract-syntax(2) ftam- 3

3                               :  fadu (2)}          "FTAM FADU"             3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 transfer syntax names:        : {joint-iso-ccitt asn1 (1) basic-encoding (1)3

3                               : }    "Basic Encoding of a single ASN.1 type"3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMJMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3parameter syntax:                                                            3

3  PARAMETERS ::= SEQUENCE OF CHOICE {Parameter0, Parameter1, Parameter2}     3

3  Parameter0 ::= universal-class-number-0 [0] INTEGER {univer-time (23),     3

3                                                       gen-time (24),        3

3                                                       boolean (1),          3

3                                                       null (5) }            3

3 Parameter1  ::= [1] SEQUENCE  {                                             3

3                 universal-class-number-1 INTEGER { int       (2),           3

3                                                    bit       (3),           3

3                                                    ia5      (22),           3

3                                                    graphic  (25),           3

3                                                    general  (27),           3

3                                                    octet     (4)},          3

3                 string-length INTEGER }                                     3

3 Parameter2  ::= [2] SEQUENCE {                                              3

3                 private-class-number INTEGER {float (0)},                   3

3                 length-1     INTEGER,                                       3

3                 length-2     INTEGER }                                      3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMKMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 file model                    : {iso standard 8571 file-model (3)           3

3                               :  hierarchical (1)}                          3

3                               :  "FTAM hierarchical file model"             3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 constraint set                : {iso identified-organization icd (9999)     3

3                               : organization-code (1) constraint-set (4) nbs3

3                               : -ordered flat (2)}                          3

3                               :     "NBS ordered flat constraint set"       3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMJMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 file contents:                                                              3

3                Datatype1 ::= PrimType -- as defined in Annex 6 A, Part 3    3

3                Datatype2 ::= CHOICE   { Node-Descriptor-Data-Element,       3

3                                         Enter-Subtree-Data-Element }        3

3                                         Exit-Subtree-Data-Element }         3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

3. Scope and field of application

The document type defines the contents of a file for storage, for

transfer and access by FTAM.

Note: Storage refers to apparent storage within the Virtual Filestore.

4. References

ISO 8571, Information Processing Systems - Open Systems

Interconnection -File Transfer, Access and Management

5. Definitions

This definition makes use of the terms data element, data unit and

file access data unit as defined in ISO 8571-1.

6. Abbreviations

FTAM File Transfer, Access and Management

7. Document semantics

The document consists of zero, one or more file access data units,

each of which consists of zero, one or more data elements.  The order

of each of these elements is significant.

The document structure takes any of the forms allowed by the FTAM

hierarchical file model as constrained by the NBS-ordered-flat

constraint set (see Table 6.10).  These definitions appear in Appendix

6 A, Part 2 of this document.

For a specific file the number of data elements in a data unit is

given by the parameters.  Each data element is a data type from the

set of primitive data types defined in the Annex 6.A, Part 3 of this

document.  Each data unit contains the same data element types in the

same order as all other data units.  These types are determined by the

parameters 0 through 2.

Note: The string length values are the actual number of characters

from the specified character set, they do not include any escape

sequences or overhead from the encoding.

8. Abstract syntactic structure

The abstract syntactic structure of the document is a hierarchically

structured file as defined in the ASN.1 module ISO8571-FADU in ISO

8571, in which each of the file access data units has the abstract

syntactic structure of NBS-AS1 as defined by the parameters.

9. Definition of transfer

9.1     Datatype definitions

The file consists of data values which are of either

a)   Datatype1 defined in Table 6.10, where the PrimType in

the datatype is given by the NBS-AS1 definition; or

b)   Datatype2 defined in Table 6.10, the ASN.1 datatype

declared as "Data-Element" in the ASN.1 module ISO8571-

FADU.

9.2     Presentation data values

The document is transferred as a series of presentation data

values, each of which is either

a)   one value of the ASN.1 datatype "Datatype1", carrying

one of the data elements from the document.  All values

are transmitted in the same (but any ) presentation

context defined to support the abstract syntax name

"asname1" or

b)   a value of " Datatype2".  All values are transmitted in

the same (but any) presentation context defined to

support the abstract syntax name " asname2".

Notes:

1.   Specific carrier standards may impose additional constraints

on the presentation context to be used, where the above

permits a choice

2.   Any document type defined in this entry either makes no use

of Datatype2, or starts with a Datatype2 transmission.

Boundaries between presentation data values in the same

presentation context, and boundaries between P-DATA primitives,

are chosen locally by the sending entity at the time of

transmission, and carry no semantics of the document type.

Receivers which support this document type shall accept a

document with any of the permitted transfer options (e.g.

document type parameters and transfer syntaxes).

9.3     Sequence of presentation data values

The sequence of presentation data values of type a) and the

sequence of presentation data values of types a) and b) is the

same as the sequence of data elements within a Data Unit, and

Data Units in the hierarchical structure, when flattened

according to the definition of the hierarchical file model in ISO

8571-2.

10.     Transfer syntax

An implementation supporting this document type shall support the

transfer syntax generation rules named in Table 6.10 for all

presentation data values transferred.  Implementation may

optionally support other named transfer syntaxes.

11.     ASE specific specifications for FTAM

11.1    Simplification and relaxation

11.1.1  Structural simplification

This simplification loses information.

The document type NBS-7 may be accessed as a document type FTAM-3

(allowed only when reading the file) by specifying document type

FTAM-3 in the <contents type> parameter in <F-OPEN request>, and

limiting access context to UA on F-READ.

The octet representation of the transferred data is

unpredictable.  It will usually correspond to the data values as

stored in the local Real Filestore of the Responder.

A document of type NBS-7 can be accessed as a document of type

NBS-6 (allowed only when reading the file) by specifying document

type NBS-6 with appropriate data type parameters in the <contents

type> parameter on the <F-OPEN request>.

11.2    Access context selection

A document of type NBS-7 may be accessed in any one of the access

contexts defined in the NBS-ordered-flat constraint set.  The

presentation data units transferred in each case are those

derived from the structuring elements defined for that access

context in ISO 8571-2.

11.3    The INSERT operation

When the <INSERT> operation is applied at the end of file the

transferred material shall be the series of FADUs which would be

generated by reading any NBS-7 document with the same parameter

values in access context FA.

NBS-8 Indexed sequential file

1. Entry Number:  NBS-8

2. Information objects

Table 6.11 Information objects in NBS-8

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDRDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3 document type name            : {iso identified-organization icd (9999)     3

3                               : organization-code  (1) document             3

3                               : type (5) indexed-file (8)}                  3

3                               :                   "NBS-8 FTAM indexed file" 3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 abstract syntax names:        : {iso identified-organization icd (9999)     3

3 a) name for asname1           : organization-code  (1) abstract-            3

3                               : syntax (2) nbs-as1 (1)}                     3

3                               :           "NBS abstract syntax AS1"         3

3 b) name for asname2           : {iso standard 8571 abstract-syntax(2) ftam- 3

3                               :  fadu (2)}                                  3

3                               :                    "FTAM FADU"              3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 transfer syntax names:        : {joint-iso-ccitt asn1 (1) basic-encoding (1)3

3                               : }                                           3

3                               :   "Basic Encoding of a single ASN.1 type"   3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMJMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3parameter syntax:                                                            3

3  PARAMETERS ::= SEQUENCE           {DataTypes, KeyType, KeyPosition}        3

3                                                                             3

3  DataTypes  ::= SEQUENCE OF CHOICE {Parameter0, Parameter1, Parameter2}     3

3                                                                             3

3  KeyType    ::= CHOICE {Parameter0, Parameter1, Parameter2}                 3

3                                                                             3

3             --  Parameter0, Parameter1, Parameter2, as defined for the      3

3             --  document types NBS-6, NBS-7                                 3

3                                                                             3

3  KeyPosition::= position INTEGER                                            3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMKMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 file model                    : {iso standard 8571 file-model (3)           3

3                               :  hierarchical (1)}                          3

3                               :  "FTAM hierarchical file model"             3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 constraint set                : {iso standard 8571 constraint-set (4)       3

3                               :  ordered-flat (3) }                         3

3                               :  "FTAM ordered flat constraint set"         3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMJMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 file contents:                                                              3

3                Datatype1 ::= PrimType -- as defined in Annex 6 A, Part 3    3

3                Datatype2 ::= CHOICE   { Node-Descriptor-Data-Element,       3

3                                         Enter-Subtree-Data-Element }        3

3                                         Exit-Subtree-Data-Element }         3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

3. Scope and field of application

The document type defines the contents of a file for storage, for

transfer and access using FTAM.

Note: Storage refers to apparent storage within the Virtual Filestore.

4. References

ISO 8571, Information Processing Systems - Open Systems

Interconnection -File Transfer, Access and Management

5. Definitions

This definition makes use of the terms data element, data unit and

file access data unit as defined in ISO 8571-1.

6. Abbreviations

FTAM File Transfer, Access and Management

7. Document semantics

The document consists of zero, one or more file access data units,

each of which consists of zero, one or more data elements.  The order

of each of these elements is significant.

The document structure takes any of the forms allowed by the FTAM

hierarchical file model as constrained by the FTAM ordered flat

constraint set (see Table 6.11).  These definitions appear in ISO

8571-2.

The following additional requirements are specified for the use of the

ordered flat constraint set:

o    The FADU identities 'first', 'last', and 'traversal number'

are not required for conformant implementations

o    The identities 'next' and 'previous' are allowed for all

FADUs

Each data element is a data type from the set of primitive data types

defined in Appendix 6A, Part 3 of this document.  Each data unit

contains the same data element types in the same order as all other

data units.  These types and their respective maximum lengths are

defined by the <DataTypes> parameter.

Note: The length values refer to the number of characters from the

applicable type, not to the number of octets in the encoding, nor to

the line length in any rendition of the document, where these are

different.

Each data unit in the file has a key associated with it.  The key of

each data unit is of the same data type as the key of all other data

units in the file and is a single data element from the set of

primitive data types defined in Appendix 6A, Part 3.

The type and length of the key are defined by the <KeyType> parameter.

The primitive data types and minimum size ranges of each unit which an

implementation must accept as a key value are given in the following

Table 6.12.

Table 6.12  Datatypes for keys

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3     Key Type                Minimum Range (octets)        Order                          3

3                                                                                          3

3    ASN.1 INTEGER            (1-2)                         increasing numeric value       3

3    ANS.1 IA5String          (0-16)                        lexical order                  3

3    ASN.1 GraphicString      (0-16)                        lexical order                  3

3    ANS.1 GeneralString      (0-16)                        lexical order                  3

3    ANS.1 OCTET STRING       (0-16)                        increasing value               3

3    ASN.1 GeneralizedTime                                  increasing time value          3

3    ASN.1 UniversalTime                                    increasing time value          3

3    NBS-AS1 FloatingPointNumber                            increasing numeric value       3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

The position of the key in the data unit is specified by the

<position> parameter.

position = 0 implies the key is not part of the data

position > 0 specifies the actual data element in the data unit.

8. Abstract syntactic structure

The abstract syntactic structure of the document is a hierarchically

structured file as defined in the ASN.1 module ISO8571-FADU in ISO

8571, in which each of the file access data units has the abstract

syntactic structure of NBS-AS1 as defined by the parameters.

9. Definition of transfer

9.1     Datatype definitions

The file consists of data values which are of either

a)   Datatype1 defined in Table 6.11, where the PrimType in the

datatype is given by the NBS-AS1 definition; or

b)   Datatype2 defined in Table 6.11, the ASN.1 datatype declared

as "Data-Element" in the ASN.1 module ISO8571-FADU.

9.2     Presentation data values

The document is transferred as a series of presentation data

values, each of which is either

a)   one value of the ASN.1 datatype "Datatype1", carrying one of

the data elements from the document.  All values are

transmitted in the same (but any) presentation context

defined to support the abstract syntax name "asname1" or

b)   a value of "Datatype2".  All values are transmitted in the

same (but any) presentation context defined to support the

abstract syntax name "asname2".

Notes:

1.   Specific carrier standards may impose additional constraints

on the presentation context to be used, where the above

permits a choice

2.   Any document type defined in this entry either makes no use

of Datatype2, or starts with a Datatype2 transmission.

Boundaries between presentation data values in the same

presentation context, and boundaries between P-DATA primitives,

are chosen locally by the sending entity at the time of

transmission, and carry no semantics of the document type.

Receivers which support this document type shall accept a

document with any of the permitted transfer options (e.g.

document type parameters and transfer syntaxes).

9.3     Sequence of presentation data values

The sequence of presentation data values of type a) and the

sequence of presentation data values of types a) and b) is the

same as the sequence of data elements within a Data Unit, and

Data Units in the hierarchical structure, when flattened

according to the definition of the hierarchical file model in ISO

8571-2.

10.     Transfer syntax

An implementation supporting this document type shall support the

transfer syntax generation rules named in Table 6.11 for all

presentation data values transferred.  Implementation may

optionally support other named transfer syntaxes.

11.     ASE specific specifications for FTAM

11.1    Simplification and relaxation

11.1.1  Structural simplification

This simplification loses information.

The document type NBS-8 may be accessed as a document type FTAM-3

(allowed only when reading the file) by specifying document type

FTAM-3 in the <contents type> parameter in <F-OPEN request>, and

limiting access context to UA on F-READ.

The octet representation of the transferred data is

unpredictable.  It will usually correspond to the data values as

stored in the local Real Filestore of the Responder.

A document of type NBS-8 can be accessed as a document of type

NBS-6 (allowed only when reading the file) by specifying document

type NBS-6 with appropriate data type parameters in the <contents

type> parameter on the <F-OPEN request>.  The traversal order of

the FADUs must be maintained.

Note: The traversal order is as reading the file as NBS-8 in key

order.

11.2    Access context selection

A document of type NBS-8 may be accessed in any one of the access

contexts defined in the FTAM ordered flat constraint set.  The

presentation data units transferred in each case are those

derived from the structuring elements defined for that access

context in ISO 8571-2.

11.3    The INSERT operation

When the <INSERT> operation is applied the transferred material

shall be the series of FADU which would be generated by reading

any NBS-8 document with the same parameter values in access

context FA.

The insertion of a new FADU after an already existing FADU will

be indicated via a diagnostic on TRANSFER-END.

11.4    The EXTEND operation

This operation is excluded for the use with this document type.

NBS-9 File directory file

1. Entry Number:   NBS-9

2. Information objects

Table 6.12 Information objects in NBS-9

ZDDDDDDDDDDDDDDDDDDDDDDDDRDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3 document type name     : {iso identified-organization icd (9999)      3

3                        : organization-code  (1) document              3

3                        : type (5) file directory (9)}                 3

3                        :       "NBS-9 FTAM file directory file"       3

FMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 abstract syntax names: : {iso identified-organization icd (9999)      3

3                        : organization-code  (1) abstract              3

3                        : syntax (2) nbs-as2 (2)}                      3

3                        :  "NBS file directory entry abstract syntax"  3

FMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 transfer syntax names: : {joint-iso-ccitt asn1 (1) basic-encoding (1)}3

3                        : "Basic Encoding of a single ASN.1 type"      3

FMMMMMMMMMMMMMMMMMMMMMMMMJMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 parameter syntax                                                      3

3                                                                       3

3 PARAMETERS ::= attribute-names [0] IMPLICIT BIT STRING {              3

3                                                                       3

3             -- Kernel group                                           3

3                                                                       3

3                  read_filename (0),                                   3

3                  read_permitted-actions (1),                          3

3                  read_contents-type (2),                              3

3                                                                       3

3             -- Storage group                                          3

3                                                                       3

3                  read_storage-account (3),                            3

3                  read_date-and-time-of-creation (4),                  3

3                  read_date-and-time-of-last-modification (5),         3

3                  read_date-and-time-of-last-read-access (6),          3

3                  read_date-and-time-of-last-attribute-modification(7),3

3                  read_identity-of-creator (8),                        3

3                  read_identity-of-last-modifier (9),                  3

3                  read_identity-of-last-reader (10),                   3

3                  read_identity-of-last-attribute-modifier (11),       3

3                  read_file-availability (12),                         3

3                  read_filesize (13),                                  3

3                  read_future-filesize (14),                           3

3                                                                       3

3             -- Security group                                         3

3                                                                       3

3                  read_access-control (15),                            3

3                  read_legal-qualifications (16),                      3

3                                                                       3

3             -- Private group                                          3

3                                                                       3

3                  read_private-use (17) }                              3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 6.12 Information objects in NBS-9 continued.

ZDDDDDDDDDDDDDDDDDDDDDDDDRDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3 file model             : {iso standard 8571 file-model (3)            3

3                        :  hierarchical (1)} FTAM hierarchical file    3

3                        :  "FTAM hierarchical file model"              3

FMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 constraint-set         : {iso standard 8571 constraint-set (4)        3

3                        :  unstructured (1)} FTAM unstructured         3

3                        : "FTAM unstructured constraint set"           3

FMMMMMMMMMMMMMMMMMMMMMMMMJMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 File contents:                                                        3

3                                                                       3

3                Datatype1 ::= FileDirectoryEntry                       3

3                              --As defined by NBS-AS2 in Appendix A,   3

3                              --Part 3 of this document                3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

3. Scope and field of Application

This document defines the contents of a file for transfer (not for

storage) using FTAM.

4. References

ISO 8571, Information Processing Systems - Open Systems

Interconnection -File Transfer, Access and Management.

5. Definitions

This definition makes use of the terms data element, data unit and

file access data unit as defined in ISO 8571-1

6. Abbreviations

FTAM File Transfer, Access and Management.

7. Document Semantics

The document consists of one file access data unit, which consists

only of zero, one or more data elements of type <FileDirectoryEntry>

(defined in NBS-AS2).

The document structure takes any of the forms allowed by the FTAM

hierarchical file model as constrained by the unstructured constraint

set.  These definitions appear in ISO 8571-1.

The parameter of the document type is used on <F-OPEN request> to

specify the desired attributes of each of the files on the Filestore,

when reading the document.

8. Abstract syntactic structure

The abstract syntactic structure of the document is a series of file

directory entries, each of which is defined by the

<FileDirectoryEntry> definition in NBS-AS2.

Additional constraints are defined for this document type:  File

access actions are restricted to Read.  File-directory files may be

Selected, Opened, Read, Closed, and Deselected.  They may not be

Created or Deleted.  They may not be Written or Modified (except as a

side effect of actions performed on individual files contained within

a file directory).

9. Definition of transfer

9.1     Datatype definition

The file consists of zero or more values of Datatype1 defined in

Table 6.13.

9.2     Presentation data values

The document is transferred as a series of presentation data

values.  Each presentation data value shall consist of one value

of the ASN.1 data type "Datatype1", carrying one of the file

directory entries from the document.

All values are transmitted in the same (but any) presentation

context established to support the abstract syntax name "asname1"

declared in Table 6.13.

9.3     Sequence of presentation data values

The sequence of presentation data values is the same as the

sequence of file directory entries within the Data Unit in the

file.

10.     Transfer syntax

An implementation supporting this document type shall support the

transfer syntax generation rules named in Table 6.13 for all

presentation data values transferred.  Implementations shall

optionally support other named transfer syntaxes.

11.     ASE specific specifications for FTAM

11.1    Simplification and relaxation

Relaxation is allowed to any bitstring combination of the

document type parameter.

Part 2:  Constraint Sets

NBS Ordered flat constraint set

1. Field of application

The NBS-ordered flat constraint set applies to files which are

structured into a sequence of individual FADUs and to which access may

be made on an FADU basis by position in the sequence.

2. Basic constraints

Table 6.14 Basic constraints for NBS Ordered flat

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDRDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3 Constraint set descriptor    : "NBS ordered flat constraint set"      3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 Constraint set identifier    : {iso identified-organization icd (9999)3

3                              : organization-code  (1)                 3

3                              : constraint-set (4) nbs-ordered-flat(1)}3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 Node name                    : None                                   3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 File access actions          : Locate, Read, Insert, Erase, Replace   3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 Qualified action             : None                                   3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 Available access contexts    : HA, FA, UA                             3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 Creation state               : Root node without an associated data   3

3                              : unit                                   3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 Location after open          : Root node                              3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 Beginning of file            : Root node                              3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 End of file                  : No node selected; 'previous' gives last3

3                              : node in traversal sequence,'current'and3

3                              :'next'give an error.                    3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 Read whole file              : Read in access context FA or UA with   3

3                              : FADU identity of 'begin'.              3

FMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMNMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMMM5

3 Write whole file (append)    : Transfer the series of leaf FADUs which3

3                              : would be generated by reading the whole3

3                              : file in access context FA; perform the 3

3                              : transfer with an FADU identity of 'end'3

3                              : and a file access action of 'insert'.  3

3                              :                                        3

3 Write whole file (replace)   : Transfer the series of leaf FADUs which3

3                              : would be generated by reading the whole3

3                              : file in access-context HA; perform the 3

3                              : transfer with FADU identity 'begin' and3

3                              : file action of 'replace'.              3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDPDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

3. Structural constraints

The root node shall not have an associated data unit; all children of

the root node shall be leaf nodes and may have an associated data

unit; all arcs from the root node shall be of length one.

4. Action constraints

Insert: The <Insert> action is allowed only at the end of file.  If

the FADU identity is 'end' the new node is inserted following all

existing nodes in the file.  If the FADU identity is 'traversal

number', the number must be at least one greater than the traversal

number of the last existing node.  Any nodes between the last existing

node and the new node are empty, i.e. nodes without data.  If the FADU

identity is a 'traversal number' not greater than that of the last

existing node, an error will occur.

Erase: The Erase action is only allowed at the root node to empty the

file, with FADU identity of 'begin'.  The result is a solitary root

node without an associated data unit.

Note: It is the intention when using this constraint set to allow for

emptying an FADU, i.e. leaving an FADU with a DU of data length 0 (or

without a DU); afterwards data may be reinserted into this hole.  In

order to empty an FADU, the <Replace> operation may be used with new

data of length zero (or with an FADU whose <data exists> bit is set to

'false' and no DU).  Refilling the hole is accomplished by a

<Replace> operation with the new DU (or with the new FADU, whose <data

exists> bit is set to 'true' and the new DU).

5. Identity constraints

The FADU identity associated with the file action shall be one of the

identities 'begin', 'end', 'first', 'last', 'current', 'next',

'previous' or a 'traversal number' greater than or equal to one.  The

actions with which these identities can be used are given in the

following table.

Table 6.15 Identity constraints in NBS Ordered flat

ZDDDDDDDDBDDDDDDDBDDDDDBDDDDDBDDDDDDBDDDDDDDBDDDDDDBDDDDDDDDBDDDDDDDDDD?

3 Action 3 Begin 3 End 3First3 Last 3Current3 Next 3Previous3Traversal 3

CDDDDDDDDEDDDDDDDEDDDDDEDDDDDEDDDDDDEDDDDDDDEDDDDDDEDDDDDDDDEDDDDDDDDDD4

3        3       3     3     3      3       3      3        3          3

3 Locate 3 valid 3valid3valid3 valid3 valid 3 valid3 valid  3  valid   3

3        3       3     3     3      3       3      3        3          3

3 Read   3 whole 3     3leaf 3 leaf 3 leaf  3 leaf 3 leaf   3  leaf    3

3        3       3     3     3      3       3      3        3          3

3 Insert 3       3valid3     3      3       3      3        3  leaf    3

3        3       3     3     3      3       3      3        3          3

3 Erase  3 whole 3     3     3      3       3      3        3          3

3        3       3     3     3      3       3      3        3          3

3 Replace3 whole 3     3leaf 3 leaf 3 leaf  3 leaf 3 leaf   3  leaf    3

3        3       3     3     3      3       3      3        3          3

@DDDDDDDDADDDDDDDADDDDDADDDDDADDDDDDADDDDDDDADDDDDDADDDDDDDDADDDDDDDDDDY

Part 3:  Abstract Syntaxes

Abstract Syntax NBS-AS1

Abstract syntax name:  {iso identified-organization icd (9999)

organization-code  (1) abstract-syntax (2) nbs-as1

(1)}

"NBS abstract syntax AS1"

This is an abstract syntax for the set of presentation data values, each

of which is a value of the ASN.1 type NBS-AS1.PrimType

NBS-AS1 DEFINITIONS ::=

BEGIN

PrimType ::= CHOICE {  INTEGER,

BIT STRING,

BOOLEAN,

IA5String,

GraphicString,

GeneralString,

OCTET STRING,

UTCTime,

GeneralizedTime,

NULL,

FloatingPointNumber }

--   The support for IA5String is the IA5 G0

--   character set and the IA5 C0 set

--   The minimum level of support for

--   GraphicString is the IA5 G0 character set and

the 8859-1 G0 and G1 sets

--   The minimum level of support for

GeneralString is the IA5 G0 character set and

the 8859-1 G0 and G1 character sets, and IA5

C0 set.

FloatingPointNumber ::=     [PRIVATE 0]    CHOICE {

finite [0] IMPLICIT SEQUENCE

{    Sign,

mantissa BIT STRING,

--   first bit must be 1

exponent INTEGER},

infinity [1] IMPLICIT Sign,

signalling-nan [2] IMPLICIT NaN,

quiet-nan [3] IMPLICIT NaN,

zero [4] IMPLICIT NULL }

Sign    ::= INTEGER { positive (0), negative (1) }

NaN     ::= INTEGER

END

For this abstract syntax the following transfer syntax can be used

{joint-iso-ccitt asn1 (1) basic-encoding (1)}

"Basic Encoding of a single ASN.1 type"

Notes:  1.   The mantissa is a number in the range (1/2 <mantissa<1).

2.   The value is equal to mantissa * 2 exponent.

3.   The first bit in the mantissa is most significant.

4.   See IEEE 754 for definitions of terminology, such as NaN.

5.   A minimum length range (in bits) is required for the

components of <FloatingPointNumber>, as follows: mantissa 1-

23 bits, and exponent 0-8 bits.

Abstract Syntax NBS-AS2

Abstract syntax name:       { iso identified-organization icd (9999)

organization-code  (1) abstract-syntax (2)

nbs-as2 (2) }

"NBS file directory entry abstract syntax"

This is an abstract syntax for the set of presentation data values, each

of which is a value of the ASN.1 Type NBS-AS2.FileDirectoryEntry.

NBS-AS2 DEFINITIONS ::=

BEGIN

FileDirectoryEntry  ::=[PRIVATE 2] Read-Attributes

Read-Attributes     ::=ISO8571-FTAM.Read-Attributes

END

For this abstract syntax the following transfer syntax will be used

{    joint-iso-ccitt asn1 (1) basic-encoding (1) }

"Basic Encoding of a single ASN.1 type"

Abstract Syntax "FTAM unstructured text abstract syntax"

This abstract syntax is defined as DataType1 (File Contents) in Table 19

of ISO 8571-2, Annex B.

Abstract Syntax "FTAM unstructured binary abstract syntax"

This abstract syntax is defined as DataType1 (File Contents) in Table 21

of ISO 8571-2, Annex B.

Part 4: Intentionally Left Empty

7. CCITT 1984 X.400 BASED MESSAGE HANDLING SYSTEM

7.1  INTRODUCTION

This is an implementation agreement developed by the Implementor's

Workshop sponsored by the U.S. National Bureau of Standards to promote

the useful exchange of data between devices manufactured by different

vendors.  This agreement is based on, and employs protocols developed

in accord with, the OSI Reference Model.  While this agreement

introduces no new protocols, it eliminates ambiguities in

interpretations.

This is an implementation agreement for a Message Handling System

(MHS) based on the X.400-series of Recommendations (1984) and Version

5 of the X.400 Series Implementor's Guide from the CCITT.  It is

recommended that product vendors consult later versions of this guide.

Figure 7.1 displays the layered structure of this agreement.

This agreement can be used over any Transport protocol class.  In

particular, this MHS agreement can be used over the Transport protocol

class 0 used over CCITT X.25, described in section 4.6 of this

document.  In addition, this MHS agreement can be used over the

Transport profiles used in support of MAP (Manufacturing Automation

Protocol) or TOP (Technical and Office Protocols).  Note that the MAP

or TOP environment must support the reduced Basic Activity Subset

(BAS) as defined in X.410.

The UAs and MTAs require access to directory and routing services.  A

Directory Service is to be provided for each (vendor-specific) domain.

Except insofar as they must be capable of providing addressing and

routing described hereunder, these services and associated protocols

are not described by this agreement.

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                                       3

3     User Agent Layer                        CCITT X.420               3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                       3

3     Message Transfer Agent Layer            CCITT X.411               3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                       3

3     Reliable Transfer Service Layer         CCITT X.410               3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                       3

3     Presentation Layer                      CCITT X.410 sec. 4.2      3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                       3

3     Session Layer                           See Section 5.1.1         3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Figure 7.1  The layered structure of this implementation agreement

7.2  SCOPE

This agreement applies to Private Management Domains (PRMDs) and

Administration Management Domains (ADMDs).  Four boundary interfaces

are specified:

(A)  PRMD to PRMD;

(B)  PRMD to ADMD;

(C)  ADMD to ADMD.

(D)  MTA to MTA (within a PRMD, e.g., for MTAs from different

vendors.)

In case A, the PRMDs do not make use of MHS services provided by an

ADMD.  In cases B and C, UAs associated with an ADMD can be the source

or destination for messages.  Furthermore, in cases A and B, a PRMD

can serve as a relay between MDs, and in cases B and C an ADMD can

serve as a relay between MDs.  Figure 7.2 illustrates the interfaces

to which the agreement applies.

X.400 protocols other than the Message Transfer Protocol (P1) and the

Interpersonal Messaging Protocol (P2) are beyond the scope of this

agreement.  Issues arising from the use of other protocols or relating

to P1 components in support of other protocols are outside the scope

of this document.  This agreement describes the minimum level of

services provided at each interface shown in Figure 7.2.  Provision

for the use of the remaining services defined in the X.400 Series of

Recommendations is outside the scope of this document.

With the exception of intra domain connections, this agreement does

not cover message exchange between communicating entities within a

domain even if these entities communicate via P1 or P2.  Bilateral

agreements between domains may be implemented in addition to the

requirements stated in this document.  Conformance to this agreement

requires the ability to exchange messages without use of bilateral

agreements.

PRMD = Private Management Domain

ADMD = Administration Management Domain              ZDDDDD?

3PRMD 3

@DDBDDY

ZDDDDDDDDDDDDDD?                  |                   ---3----A

3         PRMD 3                  |                   ZDDADD?

3 ZDDDDDDD?    CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4PRMD 3

3 3 MTA A 3    3                  |                   @DDBDDY

3 @DDDBDDDY    3   |              A                   ---3----B

3  ---3---D    3   |                                  ZDDADD?

3 ZDDDADDD?    CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4ADMD 3

3 3 MTA B 3    3   |                                  @DDBDDY

3 @DDDDDDDY    3   |                                  ---3----C

@DDDDDDDDDDDDDDY   B                                  ZDDADD?

3ADMD 3

@DDDDDY

Figure 7.2     This agreement applies to the interface between:  (A)

PRMD and PRMD; (B) PRMD and ADMD; (C) ADMD and ADMD;

and  (D)  MTA and MTA

7.3  STATUS

This version of the X.400 based Message Handling System implementation

agreements was completed on December 17, 1987.  No further

enhancements will be made to this version.  See the next

section--Errata.

7.4  ERRATA

7.5  PRMD to PRMD

7.5.1     Introduction

This section is limited in scope to issues arising from the direct

connection (interface A in Figure 7.2) of two PRMDs.  "Direct" means

that no ADMD or relaying PRMD provides MHS services to facilitate

message interchange.  "Direct" does not exclude those instances for

which Administrations happen to be ADMDs but are not providing X.400

services, that is, they are used only to provide lower layer services

such as X.25.  Figure 7.3 schematically represents the scope of this

section.

These issues relate to the use of the UAL (User Agent Layer) and MTL

(Message Transfer Layer) services, protocol elements, recommended

practices and constraints.  In particular, this section addresses the

P1 and P2 protocols and their related services in a direct connection

environment.  This section describes the minimum level of services

provided by a PRMD.  Provision for the use of the remaining services

defined in the X.400 Series of Recommendations is beyond the scope of

this section.

ZDDDDDDDDDDDDDDDDDDDD?                                 ZDDDDDDDDDDDDDDDDDDDDD?

3  Private Domain X  3                                 3  Private Domain Y   3

3                    3                                 3                     3

3                    3                                 3                     3

3                    3                                 3                     3

3                    3                                 3                     3

3      ZDDDDD?       3                                 3     ZDDDDDD?        3

3   ZD>3 UA  3<DDDDDDEDDDDDDDDDDDDD P2 DDDDDDDDDDDDDDDDEDDDD>3  UA  3<D?     3

3   3  CDDDDD4       3                                 3     CDDDDDD4  3     3

3   3  3 MTA 3<DDDDDDEDDDDDDDDDDDDD P1 DDDDDDDDDDDDDDDDEDDDD>3  MTA 3  3     3

3   3  @DDDDDY       3                                 3     @DDDDDDY  3     3

3   3    /3\         3                                 3        /3\    3     3

3         3          3                                 3         3     3     3

3  000000000000000   3                                 3  00000000000000     3

3  0the00000000000   3                                 3  0the0000000000     3

3  0remainder of00   3                                 3  0remainder of0     3

3  0Domain X000000   3                                 3  0Domain Y00000     3

3  0NETWORK0000000   3                                 3  0NETWORK000000     3

3  000000000000000   3                                 3  00000000000000     3

@DDDDDDDDDDDDDDDDDDDDY                                 @DDDDDDDDDDDDDDDDDDDDDY

Figure 7.3  Interconnection of private domains

7.5.2     Service Elements and Optional User Facilities

This section identifies those service elements and optional user

facilities that must be provided in support of P1 and P2.

7.5.2.1   Classification of Support for Services

The classification of UA and MT-Service elements is used to

define characteristics of equipment.  Equipment can claim

SUPPORT or NON-SUPPORT of a Service; in the case of

UA-service elements, a separate classification is given for

Origination and Reception.

The service provider is defined as the entity providing the

service, in this case, the MTL or the UAL.  The service user

is either the MHS user or the UAL.  The classification of

provider and user relates to the sublayer for which the

service element is defined.

7.5.2.1.1 Support (S)

a)   Support means:

o    The service provider makes the service

element available to the service user.

o    The service user gives adequate support to

the MHS to invoke the service element or

makes information associated with the service

element available.

b)   Support for Origination means that:

o    The service provider makes the service

element available to the service user for

invocation.

o    The service user gives adequate support to

the end user of the MHS to invoke the service

element.

c)   Support for Reception means that:

o    The service provider makes information

associated with the service element available

to the service user.

Note: A UA- or MT-service element can carry

information from originator to recipient only if:

o    the service element is available to the

originator,

o    the service element is available to the

recipient, and

o    all intermediate steps carry the information.

7.5.2.1.2 Non Support (N)

This means that the service provider is not required to

make the service element available to the service user.

However, the service provider should not regard the

occurrence of the corresponding protocol elements as an

error and should be able to relay such elements.

Implementations making a profile available should

indicate deviations (additions or deletions) with

respect to the requirement in the profile.

7.5.2.1.3 Not Used (N/U)

This means that although the Recommendation allows this

service element, this profile does not use it.

7.5.2.1.4 Not Applicable (N/A)

This means that this service element does not apply in

this particular case (for originator or recipient).

7.5.2.2   Summary of Supported Services

a)   Within a PRMD, a User Agent must support all P2 BASIC

IPM Services (X.400) and all P2 ESSENTIAL IPM Optional

user facilities (X.401) subject to the qualifiers

listed in Appendix 7A.

b)   Within a PRMD, a MTA must support all BASIC MT Services

(X.400) and all ESSENTIAL MT optional user facilities

(X.401) subject to the qualifiers listed in Appendix

7A.

c)   No support is required of the additional optional user

facilities of X.401.

7.5.2.3   MT Service Elements and Optional User Facilities

Tables 7.1 through 7.3 show the message transfer (MT)

service elements and optional user facilities.

Table 7.1 Basic MT service elements

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                            Support (S) or  3

3     Service Elements                       Non-support (N) 3

3                                                            3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3 Access Management                                  N/U1    3

3 Content Type Indication                            S       3

3 Converted Indication                               S       3

3 Delivery Time Stamp Indication                     S       3

3 Message Identification                             S       3

3 Non-delivery Notification                          S       3

3 Original Encoded Information Types Indication      S       3

3 Registered Encoded Information Types               N/U1    3

3 Submission Time Stamp Indication                   S       3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

DDDDDDDDDDDDDDDD

1    Not applicable to co-resident UA and MTA.

Table 7.2    MT optional user facilities provided to the UA-selectable on a

per-message basis

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                                            3

3                                                   Support (S) or           3

3  MT Optional User Facilities      Categorization  Non-support (N)          3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3 Alternate Recipient Allowed               E              S                 3

3 Conversion Prohibition                    E              S                 3

3 Deferred Delivery                         E              N2                3

3 Deferred Delivery Cancellation            E              N2                3

3 Delivery Notification                     E              S                 3

3 Disclosure of Other Recipients            E              N3                3

3 Explicit Conversion                       A              N                 3

3 Grade of Delivery Selection               E              S                 3

3 Multi-destination Delivery                E              S                 3

3 Prevention of Non-delivery Notification   A              N                 3

3 Probe                                     E              N4                3

3 Return of Contents                        A              N                 3

3                                                                            3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Table 7.3    MT optional user facilities provided to the UA agreed for

any contractual period of time

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                               Support (S) or  3

3 MT Optional User Facilities   Categorization   Non-Support (N)3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                               3

3Alternate Recipient Assignment         A               N       3

3Hold for Delivery                      A               N/U     3

3Implicit Conversion                    A               N       3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

DDDDDDDDDDDDD

E: Essential optional user facility.

A: Additional optional user facility.

2  A local facility subject to qualifiers in Appendix 7A.

3  Support not required for an originating MT user; support must be

provided for recipient MT users.

4  Subject to qualifiers in Appendix 7A.

7.5.2.4   IPM Service Elements and Optional User Facilities

Tables 7.4 through 7.5 show the IPM service elements and

optional user facilities.

Table 7.4 Basic IPM service elements

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                               3

3                                    Origination   Reception    3

3        Service  Elements              by UAs        by UAs    3

3                                                               3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3Access Management                        N/U5           N/U5   3

3Content Type Indication                  S              S      3

3Converted Indication                     N/A            S      3

3Delivery Time Stamp Indication           N/A            S      3

3Message Identification                   S              S      3

3Non-delivery Notification                S              N/A    3

3Original Encoded Information             S              S      3

3   Types Indication                                            3

3Registered Encoded Information Types     N/A            N/A5   3

3Submission Time Stamp Indication         S              S      3

3IP-message Identification                S              S      3

3Typed Body                               S              S      3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

5    Does not apply to co-resident UA and MTA.

Table 7.5    IPM optional facilities agreed for a contractual period of

time

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                              3

3                                               Support (S) or 3

3     Service Elements       Categorization     Non-Support (N)3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3Alternate Recipient Assignment      A                N        3

3Hold for Delivery                   A                N        3

3Implicit Conversion                 A                N        3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Table 7.6  IPM optional user facilities selectable on a per-message basis

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                             3

3                                       Origination  Reception3

3 IPM Optional User Facilities             by UAs      by UAs 3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3Alternate Recipient Allowed               A (N)         A (N)3

3Authorizing  Users Indication             A (N)         E (S)3

3Auto-forwarded Indication                 A (N)         E (S)3

3Blind Copy Recipient Indication           A (N)         E (S)3

3Body Part Encryption Indication           A (N)         E (S)3

3Conversion Prohibition                    E (S)         E (S)3

3Cross-referencing Indication              A (N)         E (S)3

3Deferred Delivery                         E (N)6        N/A  3

3Deferred Delivery Cancellation            A (N/U)6      N/A  3

3Delivery Notification                     E (S)         N/A  3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3Disclosure of Other Recipients            A (N)         E (S)3

3Expiry Date Indication                    A (N)         E (S)3

3Explicit Conversion                       A (N)         N/A  3

3Forwarded IP-message Indication           A (N)         E (S)3

3Grade of Delivery Selection               E (S)         E (S)3

3Importance Indication                     A (N)         E (S)3

3Multi-destination Delivery                E (S)         N/A  3

3Multi-part Body                           A (N)         E (S)3

3Non-receipt Notification                  A (N)         A (N)3

3Obsoleting Indication                     A (N)         E (S)3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3Originator Indication                     E (S)         E (S)3

3Prevention of Non-delivery Notification   A (N)         N/A  3

3Primary and Copy Recipients Indication    E (S)         E (S)3

3Probe                                     A (N)         N/A  3

3Receipt Notification                      A (N)         A (N)3

3Reply Request Indication                  A (N)         E (S)3

3Replying IP-message Indication            E (S)         E (S)3

3Return of Contents                        A (N)         N/A  3

3Sensitivity Indication                    A (N)         E (S)3

3Subject Indication                        E (S)         E (S)3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

DDDDDDDDDDD

6    A local facility subject to qualifiers in Appendix 7A.

7.5.3     X.400 Protocol Definitions

This section reflects the agreements of the NBS/OSI Workshop

regarding P1 and P2 protocol elements.

7.5.3.1   Protocol Classification

The protocol classifications are defined below in table

7.7:

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3   1)  UNSUPPORTED = X                                                     3

3       These elements may be generated, but no specific processing should  3

3       be expected in a relaying or delivering domain.  A relaying domain  3

3       must at least relay the semantics of the element.  The absence of   3

3       these elements should not be assumed, in a relaying or delivering   3

3       domain, to convey any significance.                                 3

3                                                                           3

3   2)  SUPPORTED = H                                                       3

3       These elements may be generated.  However, implementations are not  3

3       required to be able to generate these elements.  Appropriate        3

3       actions shall be taken in a relaying or delivering domain.          3

3                                                                           3

3   3)  GENERATABLE = G                                                     3

3       Implementations must be able to generate and handle these protocol  3

3       elements, although they are not necessarily present in all          3

3       messages generated by implementations of this profile.              3

3       Appropriate actions shall be taken in a relaying or delivering      3

3       domain.                                                             3

3                                                                           3

3   4)  REQUIRED = R                                                        3

3       Implementations of this profile must always generate this protocol  3

3       element.  However, its absence cannot be regarded as a protocol     3

3       violation as other MHS implementations may not require this         3

3       protocol element.  Appropriate actions shall be taken in a          3

3       relaying or delivering domain.                                      3

3                                                                           3

3   5)  MANDATORY = M                                                       3

3       This must occur in each message as per X.411 or X.420 as            3

3       appropriate; absence is a protocol violation.  Appropriate actions  3

3       shall be taken in a relaying or delivering domain.                  3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Table 7.7  Protocol Classifications

7.5.3.2   General Statements on Pragmatic Constraints

a)   Where a protocol element is defined as a choice of

Numeric String and Printable String (i.e.,

Administration Domain Name and Private Domain

Identifier), then a numeric value encoded as a

printable string is equivalent to the same value

encoded as a numeric string.  This does not apply to

the Country Name protocol element.

b)   The maximum number of recipients in a single MPDU is

32K - 1 (that is, 32767).  However, no individual

limits on the number of occurrences (recipients) are

placed on the following protocol elements: Authorizing

Users, Primary Recipients, Copy Recipients, Blind Copy

Recipients, Obsoletes and Cross References.

Additionally, there is no limit on the number of Reply

to Users.  This is a local matter for the originating

system.

c)   Use of strings.  A Printable String is defined in terms

of the number of characters, which is the same number

of octets.  For T.61 strings the number of octets is

twice the number of characters specified.

d)   The ability to generate maximum size elements is not

required, with the exception of the component fields in

the Standard Attribute List, in which case it is

required.

7.5.3.3   MPDU Size

The following agreements govern the size of MPDUs:

o    All MTAEs must support at least one MPDU of at least

two megabyte.

o    The size of the largest MPDU supported by a UAE is a

local matter.

7.5.3.4   P1 Protocol Elements

7.5.3.4.1 P1 Envelope Protocol Elements

Table 7.8 contains Protocol Elements and their

classes.

Table 7.8   P1 protocol elements

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Element                          Class         Restrictions and Comments    3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3MPDU                                                                        3

3   UserMPDU                        G                                        3

3   DeliveryReportMPDU              G                                        3

3                                                                            3

3   ProbeMPDU                       H                                        3

3                                                                            3

3UserMDPU                                                                    3

3   UMPDUEnvelope                   M                                        3

3   UMPDUContent                    M                                        3

3                                                                            3

3UMPDUEnvelope                                                               3

3   MPDUIdentifier                  M                                        3

3   originator ORname               M                                        3

3   originalEncodedInformationTypes G                                        3

3                                         If this field is absent, then the  3

3                                         Encoded Information Type is        3

3                                         "unspecified".                     3

3   ContentType                     M                                        3

3   UAContentID                     H     Maximum length = 16 characters.    3

3   Priority                        G                                        3

3   PerMessageFlag                  G     Maximum length = 2 octets.         3

3   deferredDelivery                X                                        3

3   PerDomainBilateralInfo          X     No limit on number of occurrences. 3

3   RecipientInfo                   M     Maximum number = 32K - 1 occurr-   3

3                                         ences. More severe limitations are 3

3                                         by bilateral agreement.            3

3   TraceInformation                M                                        3

3UMPDUContent                       M                                        3

3                                                                            3

3MPDUIdentifier                                                              3

3   GlobalDomainIdentifier          M                                        3

3   IA5String                       M     Maximum length = 32 characters,    3

3                                         graphical subset only.  Refer to   3

3                                         T.50 for clarification of graphical3

3PerMessageFlag                           subset.                            3

3   discloseRecipients              H                                        3

3   conversionProhibited            G                                        3

3   alternateRecipientAllowed       H                                        3

3   contentReturnRequest            X                                        3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table  7.8 P1 protocol elements, Continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Element                          Class   Restrictions and Comments          3

3                                                                            3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3PerDomainBilateralInfo                                                      3

3   CountryName                     M     Maximum length = 3 characters.     3

3   AdministrationDomainName        M     Maximum length = 16 characters.    3

3   BilateralInfo                   M     Maximum depth = 8; maximum         3

3                                         length = 1024 octets               3

3                                         (including encoding).              3

3 RecipientInfo                                                              3

3   recipient                       M                                        3

3   ExtensionIdentifier             M     Maximum value = 32K - 1 (32767).   3

3   perRecipientFlag                M     Maximum length = 2 octets.         3

3   ExplicitConversion              X                                        3

3                                                                            3

3perRecipientFlag                                                            3

3   ResponsibilityFlag              M                                        3

3   ReportRequest                   M                                        3

3   UserReportRequest               M                                        3

3                                                                            3

3TraceInformation                         Reference should be made to        3

3                                         Version 5 of the X.400 Imple-      3

3                                         mentor's Guide for information     3

3   GlobalDomainIdentifier          M     related to Trace sequencing.       3

3   DomainSuppliedInfo              M                                        3

3                                                                            3

3DomainSuppliedInfo                                                          3

3   arrival                         M                                        3

3   deferred                        X                                        3

3   action                          M                                        3

3     0=relayed  (value)            G                                        3

3     1=rerouted (value)            H     Rerouting is not required.         3

3   converted                       H                                        3

3   previous                        H                                        3

3                                                                            3

3ORName                                   See section 7.5.3.5                3

3                                                                            3

3EncodedInformationTypes                                                     3

3   bit string                      M     Delivery can only occur if match   3

3                                         is made with Registered Encoded    3

3                                         Information Types.  Individual     3

3                                         vendors may impose limits.         3

3                                         Maximum length = 4 octets.         3

3   G3NonBasicParameters            X                                        3

3   TeletexNonBasicParameters       X                                        3

3   PresentationCapabilities        X                                        3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7.8 P1 protocol elements, Continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                                            3

3Element                         Class    Restrictions and Comments          3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3DeliveryReportMPDU                                                          3

3   DeliveryReportEnvelope          M                                        3

3   DeliveryReportContent           M                                        3

3                                                                            3

3                                                                            3

3                                                                            3

3DeliveryReportEnvelope                                                      3

3   report                          M                                        3

3   originator                      M                                        3

3   TraceInformation                M                                        3

3                                                                            3

3DeliveryReportContent                                                       3

3   original                        M                                        3

3   intermediate                    G     See comment at end of table.       3

3   UAContentID                     G                                        3

3   ReportedRecipientInfo           M     Maximum number = 32K - 1           3

3                                         occurrences.                       3

3   returned                        H     Can only be issued if specifically 3

3                                         requested in the originating       3

3                                         message.                           3

3   billingInformation              X     Maximum depth = 8; maximum         3

3                                         length = 1024 octets (including    3

3                                         encoding).                         3

3                                                                            3

3                                                                            3

3ReportedRecipientInfo                                                       3

3   recipient                       M                                        3

3   ExtensionsIdentifier            M                                        3

3   PerRecipientFlag                M                                        3

3   LastTraceInformation            M                                        3

3   intendedRecipient               H                                        3

3   SupplementaryInformation        X     Maximum length = 64 characters.    3

3                                         Value is pending verification by   3

3                                         the CCITT SG VIII or XI.           3

3                                                                            3

3                                                                            3

3                                                                            3

3                                                                            3

3LastTraceInformation                                                        3

3   arrival                         M                                        3

3   converted                       G                                        3

3   Report                          M                                        3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.

Table 7.8 P1 protocol elements, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                                            3

3Element                          Class   Restrictions and Comments          3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3Report                                                                      3

3   DeliveredInfo                   G     Generated if delivery is reported. 3

3   NondeliveredInfo                G     Generated if failure to deliver    3

3                                         is reported.                       3

3DeliveredInfo                                                               3

3   delivery                        M                                        3

3   typeofUA                        R     This element must be generated with3

3                                         a PRIVATE value by PRMDs.          3

3                                                                            3

3NonDeliveredInfo                                                            3

3   ReasonCode                      M                                        3

3   DiagnosticCode                  H     Whenever possible, use a meaningful3

3                                         diagnostic code.                   3

3                                                                            3

3ProbeEnvelope                                                               3

3   probe                           M                                        3

3   originator                      M                                        3

3   ContentType                     M                                        3

3   UAContentID                     H     Maximum length = 16 characters.    3

3   original                        G     If this field is absent, then the  3

3                                         Encoded Information Type is        3

3                                         "unspecified".                     3

3                                                                            3

3                                                                            3

3   TraceInformation                M                                        3

3                                                                            3

3   PerMessageFlag                  G                                        3

3                                                                            3

3   contentLength                   H                                        3

3   PerDomainBilateralInfo          X                                        3

3   RecipientInfo                   M     Maximum number = 32K - 1           3

3                                         occurrences.                       3

3GlobalDomainIdentifier                                                      3

3   CountryName                     M     Maximum length = 3 characters.     3

3   AdministrationDomainName (4)    M     Maximum length = 16 characters or  3

3                                         digits.                            3

3                                                                            3

3   PrivateDomainIdentifier         R     Maximum length = 16 characters or  3

3                                         digits.  This element must be      3

3                                         generated by PRMDs.                3

3                                                                            3

3                             End of Definitions                             3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Notes on Table 7.8

Comment on intermediate TraceInformation in the

DeliveryReportContent in table 7.8:  Audit and confirmed reports

should not be requested  by other than the originating domain for

two reasons.  First, the return path of the report may be

different from the path taken by the original message, and it may

exclude the domain that added the request for audit and confirmed

to the message.  Second, if the return path is different from the

path of the original message, the originating domain would

receive intermediate trace information that it did not request.

7.5.3.5   ORName Protocol Elements

Only form 1 variant 1 O/R names are supported.

Table 7.9 contains ORName protocol elements.

These agreements interpret 1984 X.400 Section 3.4 to mean

that the attributes in the ORName in the MPDU must identify

exactly one UA, and that all the values for the attributes

specified in the ORName must be identical to the values of

the corresponding attributes associated with the recipient

UA. Underspecified names that are unique are deliverable.

Overspecified ORNames, ORNames with mismatching fields, and

ambiguous names are to be non-delivered or sent to the

alternate recipient as appropriate.

Table 7.9   ORName protocol elements

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Element                          Class     Restrictions and Comments        3

3                                                                            3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3ORName                                                                      3

3   StandardAttributeList           M                                        3

3   DomainDefinedAttributeList      X                                        3

3                                                                            3

3StandardAttributeList (1)                                                   3

3   CountryName                     R     As defined in X.411, Maximum       3

3                                         length = 3 characters.             3

3   AdministrationDomainName (4)    R     Maximum length = 16 characters     3

3                                         or digits.                         3

3   X121Address                     X     Maximum length = 15 digits.        3

3   TerminalID                      X     Maximum length = 24 characters.    3

3   PrivateDomainName (2)           G     Maximum length = 16 characters.    3

3   OrganizationName (2)            G     Maximum length = 64 characters.    3

3   UniqueUAIdentifier              X     Maximum length = 32 digits.        3

3   PersonalName                    G     Maximum length of values of sub-   3

3                                         elements = 64 characters.          3

3                                         Note:  The possibility that this   3

3                                                value may be reduced to 40  3

3                                                characters is for further   3

3                                                study by the CCITT.         3

3                                                                            3

3   OrganizationalUnit (3)          G     Maximum length = 32 characters per 3

3                                         occurrence.  A maximum of four     3

3                                         occurrences are allowed.           3

3                                                                            3

3DomainDefinedAttributeList (5)           Maximum = 4 occurrences.           3

3   type                            M     Maximum length = 8 characters.     3

3   value                           M     Maximum length = 128 characters.   3

3                                                                            3

3PersonalName                                                                3

3   surName                         M     Maximum length = 40 characters.    3

3   givenName                       G     Maximum length = 16 characters.    3

3   initials                        G     Maximum length =  5 characters;    3

3                                         excluding surname initial and      3

3                                         punctuation and spaces.            3

3   generationQualifier             G     Maximum length = 3 characters.     3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7.9  ORName Protocol Elements, Continued

Notes:

1.   The following apply for comparison of the Standard Attributes of

an O/R Name:

a.   Lower case is interpreted as upper case (for IA5).

b.   Multiple spaces may be interpreted as a single space.

Originating domains shall only transmit single

significant spaces.  If multiple spaces are transmitted,

non-delivery may occur.

2.   At least one of these must be supplied.

3.   These should be sent in ascending sequence, from the least

significant <Organizational Unit>  (lowest in organization

hierarchy) to the most significant.  Only those specified should

be sent. (That is, an unspecified <Organizational  Unit> should

not be sent along as a field of [null] content, nor zero length,

etc.)

4.   This attribute shall contain one space in all ORNames of messages

originated in a PRMD that is not  connected to an ADMD,  and in

ORNames of recipients reachable only through a PRMD; otherwise,

this attribute shall contain an appropriate ADMD name.

5.   Some existing systems which will be accessed via an X.400 service

(whether directly connected using X.400 protocols or otherwise)

may require the provision of addressing attributes which are not

adequately supported by Standard Attributes as defined in these

Agreements.  In such cases, Domain Defined Attributes are an

acceptable means of carrying additional addressing information.

Failure to support the specification and relaying of DDAs may

prevent successful interworking with such existing systems until

such time as all systems are capable of relaying and delivery

using only the Standard Attribute list.  Specific recommendations

on the use of DDAs for particular applications are in the

Recommended Practices Section 7.12, Appendix B.

7.5.3.6   P2 Protocol Profile (Based on [X.420])

Tables 7.10 and 7.11 classify the support for the P2

protocol  elements required by this profile.  The tables

give restrictions and comments in addition to X.420.

Restriction on length is one of the types of restrictions.

The reaction of implementations to a violation of this

restriction is not defined by this Profile.

7.5.3.6.1 P2 Protocol - Heading

Table 7.10 below specifies the support for protocol

elements in P2 Headings.

Table 7.10   P2 heading protocol elements

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3   Element                     Class     Restrictions and Comments          3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3UAPDU                                                                       3

3   IM-UAPDU                        G                                        3

3   SR-UAPDU                        X                                        3

3                                                                            3

3IM-UAPDU                                                                    3

3   Heading                         M                                        3

3   Body                            M                                        3

3                                                                            3

3Heading                                                                     3

3   IPMessageId                     M                                        3

3   originator                      R                                        3

3   authorizingUsers                H                                        3

3   primaryRecipients               G     At least one of primaryRecipients, 3

3   copyRecipients                  G     copyRecipients, or                 3

3                                         blindCopyRecipients must be        3

3   blindCopyRecipients             H     present.                           3

3   inReplyTo                       G                                        3

3   obsoletes                       H                                        3

3   crossReferences                 H                                        3

3   subject                         G     Maximum length = 128 T.61          3

3                                         characters (256 octets);the ability3

3                                         to generate the maximum size       3

3                                         subject is not required.           3

3   expiryDate                      H                                        3

3   replyBy                         H                                        3

3   replyToUsers                    H                                        3

3   importance                      H     Appropriate action is for further  3

3                                         study.                             3

3   sensitivity                     H     Appropriate action is for further  3

3                                         study.                             3

3   autoforwarded                   H                                        3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7.10 P2 heading protocol elements, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Element                         Class    Restrictions and Comments          3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3IPmessageId                                                                 3

3   ORName                          H                                        3

3   PrintableString                 M     Maximum length = 64 characters.    3

3ORDescriptor                                                                3

3   ORName                          H     Specify the ORName whenever it is  3

3                                         possible.  See Appendix 7B.        3

3   freeformName                    H     Maximum length = 64 characters,    3

3                                         graphical subset only (128 octets.)3

3   telephoneNumber                 H     Maximum length = 32 characters.    3

3                                         This allows for punctuation. It    3

3                                         does not take into account possible3

3                                         future use by ISDN.                3

3                                                                            3

3Recipient                                                                   3

3   ORDescriptor                    M                                        3

3   reportRequest                   X                                        3

3   replyRequest                    H                                        3

3                                                                            3

3Body                                     No limit on number of BodyParts.   3

3   BodyPart                        G     No limit on length of any BodyPart 3

3                                         or the depth of ForwardedIPMessage 3

3                                         BodyParts nested. Classification is3

3                                         subject to pending CCITT resolution3

3                                                                            3

3                                                                            3

3SR-UAPDU                                                                    3

3   nonReceipt                      H                                        3

3   receipt                         H                                        3

3   reported                        M                                        3

3   actualRecipient                 R                                        3

3   intendedRecipient               H                                        3

3   converted                       X                                        3

3NonReceiptInformation                                                       3

3   reason                          M                                        3

3   nonReceiptQualifier             H                                        3

3   comments                        H     Maximum length = 256 characters.   3

3   returned                        H     May only be issued if specifically 3

3                                         requested by originator.           3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7.10  P2 heading protocol elements, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Element                         Class    Restrictions and Comments          3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3ReceiptInformation                                                          3

3   receipt                         M                                        3

3   typeOfReceipt                   H                                        3

3   SupplementaryInformation        X     Maximum length = 64 characters.    3

3                                         Note:  This value is pending       3

3                                         verification by the CCITT SG       3

3                                         VIII or IX.                        3

3                                                                            3

3                         End of Definitions                                 3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

7.5.3.6.2 P2 Protocol - BodyParts

a)   All BodyParts with identifiers in the range 0 up

to and including 16K -1 are legal and should be

relayed.  BodyPart identifiers corresponding to

X.121 Country Codes should be interpreted as

described in Note 2 of figure 7.4.

o    Implementations are required to generate and

image IA5Text.

o    Implementations should specify the other

BodyPart types supported.

o    If an implementation supports a particular

BodyPart type for reception, it should also

be able to support that BodyPart type for

reception if it is part of a

ForwardedIPMessage.

o    For the BodyPart types currently considered,

support for the protocol elements is as

indicated in table 7.11.

b)   Privately Defined BodyParts

This section describes an interim means for

identifying privately defined BodyParts.  This

section shall be replaced in a future version

taking into account CCITT recommendations with

equivalent functionality.

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3        BodyPart  :: =  CHOICE {                                           3

3          [0]IMPLICIT IA5Text,                                             3

3                     [1]IMPLICIT TLX,                                      3

3                         .                                                 3

3                         .                                                 3

3                         .                                                 3

3                   [234]IMPLICIT UKBodyParts,                              3

3                         .                                                 3

3                         .                                                 3

3                         .                                                 3

3                   [310]IMPLICIT USABodyParts,                             3

3                         .                                                 3

3                         .                                                 3

3                         .                    }                            3

3                   Where UKBodyParts and USABodyParts are defined as:      3

3                                                                           3

3                     SEQUENCE {BodyPartNumber, ANY}                        3

3                                                                           3

3                     BodyPartNumber ::= INTEGER                            3

3                                                                           3

3                                                                           3

3                                                                           3

3                                                                           3

3                                                                           3

3       Note 1:  In the EncodedInformationTypes of the P1 Envelope, the     3

3                undefined bit must be set when a message contains a        3

3                privately defined BodyPart.  Each UA that expects such     3

3                BodyParts should include undefined in the set of           3

3                deliverable EncodedInformationTypes it registers with the  3

3                MTA.                                                       3

3                                                                           3

3       Note 2:  All BodyPartNumbers assigned must be interpreted relative  3

3                to the BodyPart in which they are used, which is that      3

3                tagged with the value [310] for those defined within the   3

3                United States.  The NBS assigns unique message             3

3                BodyPartNumbers for privately defined formats within the   3

3                United States.                                             3

3                                                                           3

3       Note 3:  Refer to section 7.12.6 for recommendations regarding the  3

3                implementaion of USABodyParts.                             3

3                                                                           3

3                                                                           3

3                                                                           3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Figure 7.4  X.409 Definition of Privately Defined BodyParts

7.5.3.6.3 P2 BodyPart Protocol Elements

Table 7.11   P2 BodyParts

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Elements                        Class    Restrictions and Comments          3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3BodyPart                                                                    3

3   IA5Text                         G                                        3

3   TLX                             X                                        3

3   Voice                           X                                        3

3   G3Fax                           X                                        3

3   TIFO                            X                                        3

3   TTX                             X                                        3

3   Videotex                        X                                        3

3   NationallyDefined               X                                        3

3   Encrypted                       X                                        3

3   ForwardedIPMessage              H                                        3

3   SFD                             X                                        3

3   TIF1                            X                                        3

3   unidentified                    X                                        3

3                                                                            3

3IA5Text                                                                     3

3   repertoire                      H                                        3

3   IA5String                       M     For rendition of IA5Text see       3

3                                         Appendix 7C.                       3

3TLX                                      For further study by CCITT.        3

3                                                                            3

3Voice                                                                       3

3   Set                                   For further study by CCITT.        3

3   BitString                       M                                        3

3                                                                            3

3G3Fax                                                                       3

3   numberOfPages                   X                                        3

3   P1.G3NonBasicParameters         X                                        3

3   SEQUENCE (OF BIT STRING)        M                                        3

3   BIT STRING                      H     See Note.                          3

3                                                                            3

3P1.G3NonBasicParameters                  Support for individual elements is 3

3                                         for further study.                 3

3                                                                            3

3TIFO                                                                        3

3   T.73Document                    M                                        3

3   T.73ProtocolElement             H     See Note.                          3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7.11  P2 BodyParts, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Elements                       Class     Restrictions and Comments          3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3TTX                                                                         3

3   numberOfPages                   X                                        3

3   telexCompatible                 X                                        3

3   P1.TeletexNonBasicParams        X                                        3

3   SEQUENCE                        M                                        3

3   T61String                       H     See Note.                          3

3                                                                            3

3P1.TeletexNonBasicParams                                                    3

3   graphicCharacterSets            X                                        3

3   controlCharacterSets            X                                        3

3   pageFormats                     X                                        3

3   miscTerminalCapabilities        X                                        3

3   privateUse                      X                                        3

3                                                                            3

3Videotex                                                                    3

3   SET                                   For further study by CCITT.        3

3   VideotexString                  M                                        3

3                                                                            3

3NationallyDefined                                                           3

3   ANY                             M                                        3

3                                                                            3

3Encrypted                                                                   3

3   SET                                   For further study by CCITT.        3

3   BIT STRING                      M                                        3

3                                                                            3

3ForwardedIPMessage                                                          3

3   delivery                        H                                        3

3   DeliveryInformation             H                                        3

3   IM-UAPDU                        M                                        3

3                                                                            3

3DeliveryInformation                                                         3

3   P1.ContentType                  M                                        3

3   originator                      M                                        3

3   original                        M                                        3

3   P1.Priority                     G                                        3

3   DeliveryFlags                   M                                        3

3   otherRecipients                 H                                        3

3   thisRecipient                   M                                        3

3   intendedRecipient               H                                        3

3   converted                       X                                        3

3   submission                      M                                        3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table  7.11  P2 BodyParts, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Elements                        Class      Restrictions and Comments        3

3                                                                            3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3SFD                                                                         3

3   SFD.Document                    M                                        3

3                                                                            3

3TIF1                                                                        3

3   T73 Document                    M                                        3

3   T73.ProtocolElement             H       See note.                        3

3               DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD                      3

3                                                                            3

3Note:  This element is not an addition to the definition of the BodyPart.   3

3       It is described here to show that the SEQUENCE may contain zero      3

3       elements.  A Problem Report has been submitted to the CCITT to       3

3       clarify whether this is permissible.  The NBS/OSI Workshop will      3

3       adopt the CCITT decision.                                            3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

7.5.4     Reliable Transfer Server (RTS)

7.5.4.1   Implementation Strategy

Based on X.410 clause 3 and X.411 clause 3.5.

7.5.4.2   RTS option selection

a)   The maximum number of simultaneous associations is not

limited in this profile; if the capacity of a system is

exceeded, it should not initiate or accept additional

associations.

b)   Associations are established by the MTA which has

messages to transfer.

c)   Associations are released when they are not needed.

Associations may also be ended prematurely due to

internal problems of the RTS.

d)   For both monologue and two way alternate associations,

the initiator keeps the initial turn.

e)   When establishing an RTS association, the following

rules apply to the use of parameters in addition to

those in X.410 clause 3.2.1:

Dialogue mode: Monologue must be supported for this

profile; two-way alternate is used only

if both partners agree.

Initial turn:  Kept by the initiator of the

association.

f)   The 'priority-mechanism' and the 'transfer-time limit'

are regarded as local matters.

7.5.4.3   RTS Protocol Options and Clarifications

Realization of the RTS protocol is subject to the following

rules in addition to those specified in X.410 clause 4:

a)   One RTS association corresponds to one or more

consecutive session connections (not concurrent ones).

The first is opened with ConnectionData of type OPEN,

and subsequent ones are opened with type RECOVER.

b)   Recovery of a Session connection is only by RTS

initiator.

c)   Checkpoint size:

o    Checkpointing and No Checkpointing should be

supported.  Whenever possible, checkpointing

should be used.

o    The minimum checkpointSize is 1 (that is, 1024

octets).

d)   Window size:

o    Minimal value of 1 (if checkpointing is

supported).

o    WindowSize = 1 means:  After an S-SYNCH-MINOR

request is sent, wait until the confirmation is

received before issuing an S-DATA, S-SYNCH-MINOR,

or S-ACTIVITY-END request.

e)   APDUs should not be blocked into one activity.

f)   Only one SSDU shall be transferred:

o    Between two adjacent minor synch points.

o    Between minor synch points and adjacent

S-ACTIVITY-START and S-ACTIVITY-END requests.

o    Between S-ACTIVITY-START and S-ACTIVITY-END

without checkpoints.

g)   A monologue association is defined as follows:

o    The RTS user responsible for establishing the

association is called the initiator.

o    The initiator keeps the initial turn.

o    APDUs are transferred in the direction of the

initiator to the recipient only.

o    There shall be no token passing.

o    Only the initiator can effect an orderly release

of the association.

h)   A two-way alternate association is as described in

X.410.

i)   In the UserData parameter of the S-U-ABORT, the

ReflectedParameter will not be used in the

AbortInformation element.

j)   When the S-ACTIVITY-RESUME is used to resume an

activity in the same session connection as the one in

which it started, this must happen immediately after

the activity has been interrupted (i.e., no intervening

activity can occur).  Otherwise, X.410 clause 4.3

paragraph 1 may be violated.

k)   When S-ACTIVITY-RESUME is used to resume an activity

started in another session connection, the following

conditions must be met:

o    The current session connection is of type

"recover".

o    The value of OldSessionConnectionIdentifier in

S-ACTIVITY-RESUME must match the value of the

SessionConnectionIdentifier parameter used in the

S-CONNECT of the prior session connection.  This

value is also identical to the

SessionConnectionIdentifier in the ConnectionData

(in PConnect, in SS-UserData) for the current

session connection.

o    This must occur as the first activity of the next

session connection for the same RTS-association.

It must be the first, otherwise X.410 clause 4.5.1

point 1 is violated.

Note: It is in the same RTS-ASSOCIATION because the use of

S-ACTIVITY-RESUME only makes sense within the scope of one

RTS association.

l)   If the transfer of an APDU is interrupted before the

confirmation of the first checkpoint, the value of the

SynchronizationPointSerialNumber in S-ACTIVITY-RESUME

should be zero, and the S-ACTIVITY-RESUME must be

immediately followed by an S-ACTIVITY-DISCARD.

m)   In S-TOKEN-PLEASE, the UserData parameter shall contain

an integer conforming to X.409 which conveys the

priority.

n)   The receiving RTS can use the value of the Reason

parameter in the S-U-EXCEPTION-REPORT to suggest to the

sending RTS that it should either interrupt or discard

the current activity.  S-U-Exception Reports are

handled as stated in Version 5 of the Implementors

Guide pages 12-13, paragraph E-33.

o)   In the case of S-P-ABORT, the current activity (if any)

is regarded as interrupted, rather than discarded.

p)   Table 7.12 illustrates the legal negotiation

possibilities allowed by X.410 clause 4.2.1 regarding

checkpoint size and window size.

q)   These agreements include the provisions of version 6 of

the Implementors Guide identical in all respects to

version 5, except that the following points have been

added to section 3.5:

o    for section 4.4.1 of X.410;

"If the receiving RTS receives an S-ACTIVITY-DISCARD

indication primitive and has already issued a TRANSFER

indication primitive, it aborts the connection (S-U-ABORT

request) with the "transfer completed" version code."

o    for section 4.6.2 of X.410

"The "transfer completed (7)" abort reason is used to

indicate to the sending RTS that the receiving RTS could not

discard the activity because it has already completed the

transfer (issued a TRANSFER indication primitive)."

Transfer completed (7) is also added to the table of abort

reasons in this section.

Table 7.12  Checkpoint window size of IP

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                acceptor answer                   3

CDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDDDD4

3      CS = 0    3    CS = m      3   CS = n       3

3(or unspecified)3    WS = j      3   WS = j       3

3 WS unspecified 3(or unspecified)3(or unspecified)3

ZDDDDDDDDDBDDDDDDDDDDDDDDDABDDDDDDDDDDDDDDDEDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDDDD4

3         3    CS = O      3               3                3                3

3         3(or unspecified)3   legal       3     legal      3      legal     3

3         3    WS = i      3               3                3                3

3initiator3(or unspecified)3               3                3                3

3proposal 3                3               3                3                3

3         CDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDDDEDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDDDD4

3         3    CS = k      3               3                3                3

3         3    WS = i      3   legal       3     legal      3   not allowed  3

3         3(or unspecified)3               3                3                3

@DDDDDDDDDADDDDDDDDDDDDDDDDADDDDDDDDDDDDDDDADDDDDDDDDDDDDDDDADDDDDDDDDDDDDDDDY

Legend:

o    CS means CheckpointSize

o    WS means WindowSize

o    i, j, k, m, and n are integer values with the following

relations:

0 s m s k < n       (values assigned to CS)

0 < j s i           (values assigned to WS)

o    For unspecified parameters, the default applies.  In this

case,the numeric relations apply, that is, the default values

substitutefor the unspecified integer.

7.5.4.4   RTS Protocol Limitations

The RTS Protocol Limitations for this profile are listed in

table 7.13.

Table 7.13 RTS protocol elements

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Element                         Class    Restriction                        3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3PConnect                          M                                         3

3   DataTransferSyntax             M      Value = 0.                         3

3pUserData                         M                                         3

3   checkpointSize                 H                                         3

3   windowSize                     H                                         3

3   dialogueMode                   H                                         3

3   ConnectionData                 M                                         3

3   applicationProtocol            G      Value = 1.                         3

3                                  H      Value = 8883.                      3

3   ConnectionData                                                           3

3    open                          G                                         3

3    recover                       G                                         3

3                                                                            3

3   open                                                                     3

3    RTS user data                 G                                         3

3                                                                            3

3   recover                                                                  3

3    SessionConnectionIdentifier   G                                         3

3                                                                            3

3RTS user data                                                               3

3   mTAName                        G      Maximum length 32 characters       3

3                                         graphic subset of IA5 only.        3

3   password                       G      Maximum length 64 octets           3

3                                         graphic subset of IA5 only.        3

3 < null RTS User Data >           G      Generated if other validation      3

3                                         methods are used.                  3

3                                                                            3

3SessionConnectionIdentifier                                                 3

3   CallingSSUserReference         M      Maximum length 64 octets including 3

3                                         encoding = 62 octets of T.61.      3

3   CommonReference                M                                         3

3   AdditionalReferenceInformation H      Maximum length 4 octets including  3

3                                         encoding = 2 octets of T.61.       3

3                                                                            3

3PAccept                           G                                         3

3   DataTransferSyntax             M      Value = 0.                         3

3   pUserData                      M                                         3

3   checkpointSize                 H                                         3

3   windowSize                     H                                         3

3   ConnectionData                 M                                         3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7.13  RTS protocol elements, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Element                        Class     Restriction                        3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3PRefuse                         G                                           3

3   RefuseReason                 M                                           3

3                                                                            3

3SS User Data                    G        See Note                           3

3   (in S-TOKEN-PLEASE)                                                      3

3                                                                            3

3AbortInformation                G                                           3

3   (in S-U-ABORT)                                                           3

3   AbortReason                  H                                           3

3   reflectedParameter           X        Restricted to 8 bits.              3

3                                                                            3

3                        End of Definitions                                  3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Note: Generated if supplied by the RTS-user.  The RTS use may specify

a priority in the TURN-PLEASE primitive, and if so, it is carried as

the SS-User-Data in S-TOKEN-PLEASE.

7.5.5     Use of Session Services

The session requirements and use of session are covered in

section 5 of this document.

7.5.6     Data Transfer Syntax

This section defines Presentation Transfer Syntax and notation

rules applicable to these agreements.  Implementations must

conform EXACTLY as specified in X.409 with no further

restrictions.  Appendix 7C defines rendition of IA5 Text and T61

characters.

7.6  PRMD to ADMD and ADMD to ADMD

7.6.1     Introduction

This section defines the implementation agreements that apply to

the interface between two management domains when at least one is

an ADMD.  A message arriving at an ADMD has either no recipient

within that domain or one or more recipients within that domain.

In the former case, the ADMD serves as a relay between two or

more domains and the actions required of that ADMD are

independent of the nature (PRMD or ADMD) of the domains.  In the

latter case, the ADMD is responsible for delivering messages to

the proper recipient(s) within its jurisdiction, and may also be

responsible for relaying the message.

Given the two roles for an ADMD, this section describes two

distinct sets of functional requirements for an ADMD.  The first

is the relaying requirement that is needed to provide PRMD and

other ADMD interworking.  The second is analogous to the PRMD's

support to its customers through the integrated UAs.  These are

distinct functional differences.  The services provided to the

UAs of an ADMD are independent of the requirement that an ADMD

provide a function for interworking with any type of Management

Domain (MD).  Figure 7.5 illustrates the two roles played by an

ADMD.

This section is presented in the form of deviations from the

agreements applicable to PRMD-to-PRMD (section 7.5).  Unless

explicitly noted in the remainder of this section, all of the

specifications for PRMD to PRMD apply to PRMD to ADMD and ADMD to

ADMD.

ZDDDDDDDDDDDDDDDDDDD?                                    ZDDDDDDDDDDDDDDDDDDD?

3   PRMD or ADMD    3                                    3       ADMD        3

3   ZDDDDDDDDDD? <--3---------------P2-------------------3--> ZDDDDDDDDD?    3

3   3 IPM - UA 3    3                                    3    3IPM - UA 3    3

3   CDDDDDDDDDD4 <--3---------------P1-------------------3--> CDDDDDDDDD4    3

3   3    MTA   3    3                                    3    3   MTA   3    3

3   @DDDDDDDDDDY    3                                    3    @DDDDDDDDDY    3

@DDDDDDDDDDDDDDDDDDDY                                    @DDDDDDDDDDDDDDDDDDDY

(a)

ZDDDDDDDDDDDDDDDDDDD?                                    ZDDDDDDDDDDDDDDDDDDD?

3   PRMD or ADMD    3                                    3   PRMD or ADMD    3

3   ZDDDDDDDDDD?    3                                    3   ZDDDDDDDDDD?    3

3   3 IPM - UA 3 <--3---------------P2-------------------3-> 3 IPM - UA 3    3

3   CDDDDDDDDDD4    3                                    3   CDDDDDDDDDD4    3

3   3   MTA    3    3                                    3   3    MTA   3    3

3   @DDDDDDDDDDY    3                                    3   @DDDDDDDDDDY    3

@DDDDDDDDDDDDDDDDDDDY                                    @DDDDDDDDDDDDDDDDDDDY

|                                                      |

|                                                      |

|                                                      |

P1                                                     P1

|                                                      |

|                ZDDDDDDDDDDDDDDDDDD?                  |

|                3       ADMD       3                  |

---------------->3      ZDDDD?      3<------------------

3      3 MTA3      3

3      @DDDDY      3

@DDDDDDDDDDDDDDDDDDY

(b)

Figure 7.5  An ADMD may (b) or may not (a) serve as a relay

7.6.2     Additional ADMD Functionality

The following defines the additional ADMD specific functionality

required over and above that specified in the PRMD section.

7.6.2.1   Relay Responsibilities of an ADMD

ADMDs will relay all content types (not just P2) unchanged

in the absence of a request for conversion.

7.6.2.2   P1 Protocol Classification Changes

Table 7.14 describes the changes to the PRMD P1 Protocol

classifications required for a delivering Administration

domain (with respect to the original message; this means the

domain which originates the delivery reports).

Table 7.14  P1 Protocol Classification Changes for a Delivering ADMD

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                                           3

3  Protocol Elements                 Class                                  3

3                                                                           3

3                                                                           3

3        DeliveredInfo                                                      3

3         typeOfUA                          H                               3

3                                                                           3

3        ReportedRecipientInfo                                              3

3         SupplementaryInformation          H   See Note 1.                 3

3                                                                           3

3        GlobalDomainIdentifier                                             3

3         PrivateDomainIdentifier           H                               3

3                                                                           3

3                                                                           3

3                                                                           3

3                                                                           3

3       For relaying Administration domains, the classifications are all "X"3

3                                                                           3

3       For originating Administration domains, these are all               3

3       "NOT APPLICABLE".                                                   3

3                                                                           3

3                                                                           3

3  Note 1:  Domains providing access to TELEX/TELETEX recipients, whether   3

3           directly or indirectly as a result of bilateral agreements      3

3           between domains, must ensure that this information, when        3

3           present, is accessible by the recipient of the delivery report. 3

3                                                                           3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

7.6.2.3   O/R Names

O/R Names shall consist of:

o    CountryName,

o    AdministrationDomainName.

as well as one of the following:

o    PrivateDomainName,

o    PersonalName,

o    OrganizationName,

o    OrganizationalUnit,

o    UniqueUAIdentifier,

o    X121Address.

and permits the optional inclusion of a

o    DomainDefinedAttributeList.

Note: The destination PrivateDomainName or OrganizationName

must be present if destined for a PRMD.  The ADMD relaying

the message to that destination PRMD requires this element.

7.6.2.4   P1 ADMD Name

Management Domains (MDs) must specify in the ADMD name field

of the O/R Name StandardAttributeList in P1, the name of the

Administration domain:

o    to which the message is being sent (in recipient names)

o    from which the message originated (in the originator

name).

7.6.3     Interworking with Integrated UAs

If the message originates at a UA owned by an ADMD, or is

delivered to such a UA, the O/R Name follows the same Form 1

Variant 1 constraints as the base specifications; except that the

ADMD name is the name of the ADMD that owns the UA and instead of

supplying a PRMD Name, one (or more) of the following must be

provided:

o    OrganizationName,

o    OrganizationalUnit,

o    PersonalName.

and may optionally include a

o    DomainDefinedAttributeList.

7.6.4     Differences with Other Profiles

7.6.4.1   TTC Profile

There are no outstanding issues regarding interworking

between TTC-conformant systems and NBS-conformant systems

with the exception of the number of recipients and the

supported MPDU sizes.  The ExtensionIdentifier field may

contain a maximum value of 32K-1; however, according to the

current TTC profile, if a message with more than 256

recipients is received, some TTC-conformant domain may

generate a nondelivery notification.  This also applies to

the ReportedRecipientInfo in a delivery report.  Further, a

TTC MTA is required to handle an MPDU size of at least 32KB.

The NBS required MPDU size is 2MB as covered in section

7.5.3.3.  Other differences are shown in Appendix E.  TTC is

currently based on Version 4 of the Implementor's Guide.

7.6.4.2   CEPT Profile

See Appendix 7E.

7.6.5     Connection of PRMDs to Multiple ADMDs

Given that Management Domain names (both PRMD and ADMD) shall be

unique within the U.S., then when an ADMD is presented a message

for transfer from a PRMD, it will accept O/R Names (both

originator and recipient) which have an AdministrationDomainName

field value different than the Administration's name.  "Accept"

implies the attempt to route/deliver the message shall be made,

as appropriate, based upon the knowledge that MD names are

unique.

Whether this functionality is required by an Administration for

conformance to this agreement is for further study.

If a PRMD is connected to two or more ADMDs which are not

effectively connected (either directly or via a third ADMD), full

X.400 functionality shall not be available.  Problems occur

especially in the areas of:

o    Naming,

o    Routing,

o    Replying.

It should be noted that a single PRMD that is connected to more

than one ADMD can be represented by more than one combination of

country-name, ADMD-name, and PRMD name.  For example, it may

occur that the PRMD-name for a particular PRMD may take different

values, depending on the ADMD-name.  Implementors should be aware

of the consequences of these possibilities on routing.

7.6.6     Connection of an ADMD to a Routing PRMD

It is possible for a collection of interconnected private domains

to establish one domain as the "gateway" to an ADMD, and hence to

the world.

If an ADMD is connected to such a gateway PRMD, the individual

private domains shall be registered with the Administration.

Administrations need not support such connections.

Note also that upon receipt by the ADMD of a message originating

somewhere within the PRMD collection, that the TraceInformation

may contain more than one element.

The X.400 Recommendations specify that an ADMD should not attempt

to relay a message destined for another ADMD through a PRMD, thus

an ADMD should ensure that messages destined for another ADMD are

not relayed through a PRMD.  It should be noted, however, that a

relaying PRMD will relay any such message it receives.

7.6.7     Management Domain Names

o    All Management Domain Names (both Private and

Administration) shall be unique within the U.S.

o    A central naming authority shall be established to register

domain names.

7.6.8     Envelope Validation Errors

For validation errors, a non-delivery notice shall be generated

(if possible) with reason code of 'unableToTransfer' and

diagnostic code of 'invalidParameters' (unless specified

otherwise).

ADMDs will validate P1 Envelopes in the following areas:

a)   The X.409 syntax of all elements should be checked.

b)   The pragmatic constraint limits (lengths of fields and

number of occurrences of fields) should be checked.

c)   Semantic validation of the following elements should be

done:

o    originator O/R Name,

o    recipient O/R Name in the RecipientInfo,

o    Priority.

d)   Only recipient Names with the responsibility flag set should

be validated.  The validation of O/R names is defined in

7.8.3.3; the validation of priority is defined in 7.8.3.7.1.

MPDU Identifier Validation

o    Validation of the GlobalDomainIdentifier component of

the MPDU Identifier is performed upon reception of a

message (i.e., as a result of a TRANSFER.Indication).

o    The country name should be known to the validating

domain, and depending on the country name, validation

of the ADMD name may also be possible.

o    Additional validation of the GlobalDomainIdentifier is

performed against the corresponding first entry in the

TraceInformation.  If inconsistencies are found during

the comparison, a non-delivery notice with the above

defined reason and diagnostic codes is generated.

o    A request will be generated to the CCITT for a more

meaningful diagnostic code (such as

'InconsistentMPDUIdentifier').

7.6.9     Quality of Service

7.6.9.1   Domain Availability

7.6.9.1.1 ADMD Availability

The goal is to provide 24 hour per day availability.

Note that there will be periods of time when an ADMD

may be unavailable due to maintenance windows in its

supporting network or in an MTA within the domain.

7.6.9.1.2 PRMD Availability

Although the goal of PRMD availability is also 24 hours

per day, business reasons are likely to dictate some

different level of availability.  ADMDs shall require a

profile from the PRMD that indicates its schedule of

regular availability to the ADMD.

7.6.9.2   Delivery Times

In the absence of standardized quality of service

parameters, the following are agreed to.  When standardized

parameters from CCITT Study Group I become available, they

shall be adopted.

a)   In table 7.15 the following delivery time targets are

established:

Table 7.15  Delivery Time Targets

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3    Delivery Class                   95% Delivered Before   3

3                                                            3

3   Urgent                                3/4 hour           3

3   Normal                                  4 hours          3

3   Non-Urgent                             24 hours          3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

b)   The interval(s) between retries and the number of retry

attempts that an ADMD uses in attempting delivery to a

PRMD or integrated UA, will be locally determined

domain parameters.  However, the total elapsed times

after which delivery attempts will be stopped are shown

in table 7.16.  This implies that, after these times, a

Non-Delivery Notice will be generated.

An Administration shall continue to attempt delivery until

the forced nondelivery time, even if the recipient domain

has scheduled an unavailability window.

Table 7.16  Forced Nondelivery Times

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3    Delivery Class                   NonDelivery Forced After   3

3                                                                3

3    Urgent                                 4 hours              3

3    Normal                                24 hours              3

3    Non-Urgent                            36 hours              3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Note:Both tables apply to the period between acceptance by

the originating MTA in the originating Administration domain

to the time of delivery in the destination Administration

domain.  Transit time within PRMDs is NOT included in the

above times.

7.6.10    Billing Information

a)   All aspects relating to billing, charging, tariffs,

settlement, and in particular to the use of the

billingInformation field in the delivery report, is subject

to bilateral agreement, and shall not be addressed in these

implementation agreements.

b)   No ADMD shall require a PRMD to supply or process billing

information.

7.6.11    Transparency

a)   No P1 extensions, other than the MOTIS extensions are to be

allowed (Reference A/3211).  Should an ADMD receive a

message containing P1 extensions, it shall generate a

non-delivery notice (if possible) with reason code of

unableToTransfer and diagnostic code of invalidParameters.

If MOTIS elements are present, a relaying MTA can optionally:

o    Relay the message.  If the MTA does relay, it must not

drop any of the protocol elements.

o    Non-Deliver the message.

A receiving MTA can optionally:

o    Deliver the message

o    Non-Deliver the message.

b)   The CCITT has been requested to establish a more meaningful

diagnostic code (such as protocolError) for this occurrence.

Such a code has now been provided in the Implementors Guide.

c)   P2 extensions shall be relayed transparently by ADMDs.

7.6.12    RTS Password Management

RTS password management shall be a local matter.  This includes:

o    password length

o    frequency of changes

o    exchange of passwords with communicating partners

o    loading passwords ( i.e., the timing of password

changes with respect to active associations).

7.6.13    For Further Study

Issues requiring further study are:

o    Intra-Domain Routing

o    Multi-Vendor Domains

7.7  INTER and INTRA PRMD CONNECTIONS

7.7.1     Introduction

This section is limited in scope to issues arising from the

indirect connection of a PRMD to another PRMD or to an ADMD, and

to the interconnection of MTAs to form inter-PRMD connections.

Indirect means that the connection is made via a relaying PRMD.

The X.400 Recommendations describe the way that a PRMD connects

to a ADMD and the way that an ADMD connects to another ADMD.  The

Recommendations do not, however, describe the way that a PRMD

connects indirectly to an ADMD or another PRMD, nor do they

describe the way that MTAs are connected within a PRMD.  These

configurations (shown in Figures 7.6 and 7.7) are useful, for

example, in connecting equipment from different vendors at a

single customer site.

The P1 protocol and its related services for both inter and intra

PRMD connections are addressed in this section.  In addition, a

method for routing within a PRMD is given.  It is recognized that

uniform methods for Administration, maintenance, and quality of

service should be developed for such configurations, and this

work is for further study.

This section describes the minimum that must be provided in order

to implement a relaying PRMD and a MTA within a PRMD.

This section is presented in the form of deviations from

agreements applicable to PRMD to PRMD connection (section 7.5).

That is, unless specifically noted in the remainder of this

section, the agreements in section 7.5 apply to both relaying

PRMDs and MTAs within a PRMD.

It should be noted that the comments regarding ORNames in Section

7.6.5 also apply to this section.

7.7.2     The Relaying PRMD

A PRMD that has the capability of relaying messages to another

PRMD is called a relaying PRMD.  A PRMD implementation need not

claim to be a relaying PRMD.  A PRMD implementation which does

claim to be a relaying PRMD must follow the implementation

agreements in this section.

7.7.2.1   Relay Responsibilities of a PRMD

The responsibilities of a relaying PRMD are the same as

those of an ADMD (as specified in sections 7.6.8 and

7.6.2.1).  In addition, the PRMD will simply deliver

messages routed to it from an ADMD, even if this results in

routing a message from the ADMD to the PRMD to another ADMD.

7.7.2.2   Interaction with an ADMD

In order for an ADMD to route a message to ADMD A via ADMD

B, it must know that A is reachable through B.  Similarly,

in order for any MD to route a message to PRMD A via a

relaying PRMD B, it must know that A is reachable through B

(see Figure 7.8).

ZDDDDDDDD?

3  ADMD  3

@DDDBDDDDY

ZDDDDDDDD?           ZDDDADDDD?            ZDDDDDDDD?

3 PRMD A CDDDDDDDDDDD4 PRMD B CDDDDDDDDDDDD4 PRMD C 3

@DDDDDDDDY           @DDDDDDDDY            @DDDDDDDDY

Relay

Figure 7.6  Relaying PRMD

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3  PRMD                                       3

3           ZDDDDDD?               ZDDDDDD?   3

3           3MTA A 3               3 MTA D3   3

3           @DDBDDDY               @DDBDDDY   3

3              3                      3       3   ZDDDDDDD?

3              3                      3       3   3  ADMD 3

3           ZDDADDD?               ZDDADDD?   3   3       3

3           3MTA B CDDDDDDDDDDDDDDD4 MTA CCDDDEDDD4   or  3

3           @DDDDDDY               @DDDDDDY   3   3       3

3                                             3   3  PRMD 3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY   @DDDDDDDY

Figure 7.7  Intra PRMD connections

Note 1: Section 7.6.6 specifies that ADMDs are not required to

connect to a relaying PRMD, but they are not precluded from doing

so.

Note 2: TraceInformation may have more than one sequence on

submission of a message by a relaying PRMD to an ADMD.

ZDDDDDDD?

3  MD D 3

@DDDBDDDY

ZDDDDDADDDDDDDDD?

3    relay      3       ZDDDDDDDDDD?        ZDDDDDDDD?

3    MD  C with 3DDDDDDD4   relay  CDDDDDDDD4   MD A 3

3    a message  3       3   MD B   3        @DDDDDDDDY

3    for A      3       @DDDDDDDDDDY

@DDDDDDDDDDDDDDDY

Figure 7.8 MD C must know of A to route the message

7.7.3     Intra PRMD Connections

A PRMD is composed of MTAs which cooperate to perform the

functions expected of a domain.  An MTA implementation need not

claim to follow the implementation agreements of this section.

7.7.3.1   Relay Responsibilities of an MTA

The relaying responsibilities of an MTA are the same as

those of an ADMD (as specified in sections 7.6.8 and

7.6.2.1) with one exception: loop suppression within the

domain is done using the MOTIS InternalTraceInfo protocol

element.  The MTA must validate the InternalTraceInfo (see

section 7.8.3.5 for details on validation).  In addition,

the PRMD will simply deliver messages routed to it from an

ADMD, even if this results in routing a message from the

ADMD to the PRMD to another ADMD (please see section 7.6.6).

7.7.3.2   Loop Suppression within a PRMD

a)   The only mechanism defined in the X.400 Recommendations

for suppressing loops is TraceInformation, which is

added on a per domain basis to detect and suppress

loops among domains.  Loops among MTAs within a domain

need to be detected and suppressed.  This implies that

each MTA must add trace information that is meaningful

within the domain.  The MOTIS solution of adding

InternalTraceInfo to the P1 Envelope of a message was

adopted.  The definition of InternalTraceInfo is given

in figure 7.9.  The InternalTraceInfo is added by each

MTA within a PRMD to handle a message, and it is

examined in the same way as TraceInformation to detect

and suppress loops.

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3       InternalTraceInfo ::= [APPLICATION 30]              3

3          IMPLICIT SEQUENCE OF                             3

3          SEQUENCE {                                       3

3             MTAName,                                      3

3             MTASuppliedInfo }                             3

3                                                           3

3       MTAName ::= PrintableString                         3

3                                                           3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Figure 7.9  Definition of InternalTraceInfo

If the MTAName and password of X.411 are used for

validation,  then it is recommended that the MTAName

used for validation also  be used in the

InternalTraceInfo.  However, there is a complication:

in X.411, MTAName is an IA5String, and the MTAname

defined by MOTIS is a PrintableString.  Efforts will be

made to change the MOTIS definition from

PrintableString to IA5String.

b)   Three actions are defined in MTASuppliedInfo: relayed,

rerouted, and recipientReassignment as shown in figure

7.10.  The recipientReassignment action is not

supported in these agreements.  The ability to generate

it is not required, and if it is present on an incoming

message, the action field will be ignored.

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3 MTASuppliedInfo ::= SET {                                  3

3   arrival [0] IMPLICIT Time,                               3

3   deferred [1] IMPLICIT Time OPTIONAL,                     3

3   action [2] IMPLICIT INTEGER                              3

3      { relayed(0), rerouted(1), recipientReassignment(2) } 3

3   previous MTAName OPTIONAL }                              3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Figure 7.10  Defined Actions in MTASuppliedInfo

7.7.3.3   Routing Within a PRMD

a)   Routing within a PRMD is complicated by the lack of a

directory standard.  In particular, it constrains

intra-domain routing decisions to be based on some

combination of the intra-domain attributes of the O/R

Name, Organization Name Organizational Units, and

Personal Name.  In order to enhance interworking and to

reduce the difficulty of configuring intra-domain

connections, it is useful to restrict the ways in which

these may be used in making routing decisions.

b)   However, it is recognized that vendors may wish to

provide MTAs with varying degrees of routing capability

within a PRMD as a temporary expedient until

appropriate standards for automated construction of

directories and routing tables are available.  This

section assigns class numbers to certain levels of

routing capability and discusses the consequences of

using MTAs which fall into each class.  The

classification scheme will allow some diversity in

allocating O/R Name space and in configuring

intra-domain routes.

c)   When other methods are recommended by standards bodies,

the classification scheme described here will become

obsolete.  Large-scale, multi-vendor PRMDs may not be

practical in the absence of standardized methods.

7.7.3.3.1 Class Designations

When it is clear that a message is to be delivered

within a domain, the Country Name, ADMD Name, and PRMD

Name have already served their purpose in determining

the next MTA in the route to the recipient.  The

remaining fields that might be used on making routing

decisions within the PRMD are the Organization Name,

Organizational Units, and Personal Name.

MTAs are classified by their ability to discriminate

between O/R names when making routing decisions within

a PRMD.  Conformant MTAs will be classified as shown in

table 7.17.

Table 7.17  Conformant MTA Classifications

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                      Class 1  Class 2  Class 3 3

3                                                                3

3  Organization Name                      H        H        H    3

3  SEQUENCE OF Organizational Unit        X        H        H    3

3  Personal Name                          X        X        H    3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

a)   An 'H' means that the MTA must be able to base its

intra-domain routing decisions on the given

component of the O/R Name.  In particular, both

Class 2 and Class 3 MTAs must be able to

discriminate on all the members in a supplied

sequence of OrganizationalUnits.  A Class 3 MTA

must be able to discriminate on all of the

elements in a PersonalName.

An 'X' means that the MTA need not have the

ability to discriminate on the given component.

b)   There is a hierarchy in support of components.

The ability to discriminate on a given component

does not imply the requirement to do so: e.g., a

Class 3 MTA is not required to have tables for

every PersonalName in the domain.  Equally, an MTA

which can discriminate on OrganizationalUnits to

make routing decisions need not always use the

full sequence in an O/R Name if a partial sequence

provides enough information.

c)   The above classifications only apply to routing

decisions in selecting a next hop within a domain.

All MTAs are entitled to examine the full O/R Name

when identifying their own directly served UAs.

d)   The routing table of a Class 1 MTA will be

relatively small, because intra-domain routing

decisions are based solely on OrganizationName.

The routing table of a Class 2 MTA may be

substantially larger and more complex because of

its ability to discriminate on OrganizationalUnits

as well as OrganizationName to make routing

decisions.  The routing table of a Class 3 MTA may

be larger still, because its use of the components

of PersonalName in addition to the other

information.

7.7.3.3.2 Specification of MTA Classes

If an MTA implementation claims to follow the

implementation agreements, it must be either a Class 1,

Class 2, or a Class 3 MTA.  The class of an MTA

implementation should be specified so that PRMD

administrators can choose equipment properly.

7.7.3.3.3 Consequences of Using Certain Classes of MTAs

Definition:    An MTA which accepts submission requests

and furnishes delivery indications to a

UA is said to "directly serve" the UA.

a)   The presence in a domain of an MTA acting as a

Class 1 or Class 2 MTA imposes administrative

restrictions on the assignment of O/R Names to UAs

and in the configuration of routes within that

domain.

o    A Class 1 MTA may directly serve UAs from

several OrganizationNames. However, if a

Class 1 MTA directly serves a UA with a given

OrganizationName, no other MTA in the domain

may directly serve a user with the same

OrganizationName.  This means that if all

MTAs in a domain are Class 1, then all UAs

with a given OrganizationName must be

assigned to the same MTA.

o    A Class 2 MTA may directly serve UAs from any

combination of OrganizationName and sequence

of OrganizationalUnits.  However, if a Class

2 MTA directly serves a UA with a given

combination, no other MTA in the domain may

directly serve a user with the same

combination.  This means that if all MTAs in

a domain are Class 2, then all UAs with a

given OrganizationName and sequence of

OrganizationalUnits must be assigned to the

same MTA.

o    A domain consisting entirely of Class 3 MTAs

is free of all the above restrictions.

b)   If Class 1 or Class 2 MTAs are used to perform

relaying within a PRMD containing MTAs of other

classes, care must be exercised in determining the

topology of the domain to avoid leaving certain

UAs inacessible from certain MTAs within the

domain.  The example below shows one of the

configurations that should be avoided.  The

example is intended to stimulate careful

examination of the relationship between network

and organizational topologies.

ZDDDDDDDDDDDDDDD?         ZDDDDDDDDDD?         ZDDDDDDDDDDDDDDD?

3     MTA A     3         3   MTA B  3         3     MTA C     3

3    serving    CDD ... DD4          CDD ... DD4    serving    3

3Organization X 3         3 (Class 1)3         3Organization X 3

@DDDDDDDDDDDDDDDY         @DDDDDDDDDDY         @DDDDDDDDDDDDDDDY

Figure 7.11  Example of a confirguration to be avoided

In Figure 7.11, B will route all messages for

Organization X to either A or C because B is a Class 1

MTA.  The administrator who created this configuration

probably wanted B to route some messages for

Organization X to A, and some to C.  However, B does

not have the capability for this because it is only a

Class 1 MTA.  The configuration in Figure 7.11 can be

corrected by replacing B with a Class 2 or Class 3 MTA.

7.7.3.4   Uniqueness of MPDUidentifiers Within a PRMD

When generating an IA5String in an MPDUIdentifier, each MTA

in a domain must ensure that the string is unique within the

domain.  This shall be done by providing an MTA designator

with a length of 12 octets which is unique within the

domain, to be concatenated to a per message string with

maximum length of 20 octets.

Two pieces of information, the MTA name and MTA designator,

need to be registered within a PRMD to guarantee uniqueness.

This registration facility need not be automated.  If the

MTA name is less than or equal to 12 characters, it is

recommended that it also be used as the MTA designator.

7.7.4     Service Elements and Optional User Facilities

A PRMD made up of MTAs which support varying sets of service

elements in addition to those required in these agreements may

appear to provide inconsistent service for these elements.  For

example, if one MTA supports deferred delivery and another MTA

does not, then deferred delivery can be used by some, but not

all, users in the PRMD.  Similarly, if one MTA supports return of

contents and another does not, then a user outside of the PRMD

will receive returned contents for messages sent to one user, but

not for messages sent to another user.  Note that this same

inconsistency occurs when sending to two PRMDs which support

different additional optional elements.

7.7.5     X.400 Protocol Definitions

This section describes additions and modifications to section

7.5.3 which are required for implementation of a relaying PRMD or

an MTA within a PRMD.

7.7.5.1   Protocol Classification

a)   The classification scheme given in section 7.5.3.1

applies to elements passing from one PRMD to another.

For both relaying PRMDs, and MTAs in a PRMD, the same

classification scheme will be used, but within a PRMD

the classification applies to elements passing from one

MTA to another.

b)   In addition to the classifications given in section

7.5.3.1, a classification of Prohibited has been used.

PROHIBITED = P

This element shall not be used.  Presence of this

element is a protocol violation.

7.7.5.2   P1 Protocol Elements

Table 7.18 contains protocol elements and their classes.  An

* signifies that the classification of the protocol element

has not changed from Table 7.8.

Table 7.18 P1 Protocol Elements

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Element                         Class     Restrictions and Comments         3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3UMPDUEnvelope                                                               3

3  MPDUIdentifier                  M*      This field needs to be unique     3

3                                          within a PRMD.  See sections      3

3                                          7.7.3.4 for the method of         3

3                                          ensuring uniqueness.              3

3                                                                            3

3  originator                      M*      It is recommended that all        3

3                                          components of the originator's    3

3                                          ORName be included to help ensure 3

3                                          that reports can be returned.     3

3                                                                            3

3  TraceInformation                M*      The first MTA in the domain to    3

3                                          receive the message adds the      3

3                                          TraceInformation.  Subsequent     3

3                                          MTAs can update the               3

3                                          TraceInformation in the event of  3

3                                          conversion or deferred delivery.  3

3                                          When a message is generated, the  3

3                                          originating MTA adds the          3

3                                          TraceInformation.                 3

3                                                                            3

3  InternalTraceInfo               M/P     This element is mandatory for     3

3                                          envelopes transferred between     3

3                                          MTAs within a PRMD, and           3

3                                          prohibited in messages            3

3                                          transferred outside the domain.   3

3                                          Elements are always added to the  3

3                                          end of the sequence.  (See Note 1)3

3                                                                            3

3InternalTraceInfo                 M       MTANames within a PRMD must be    3

3  MTAName                                 unique.  See section 7.7.3.4 for  3

3                                          the method of assuring uniqueness 3

3                                          Maximum length = 32 characters.   3

3                                                                            3

3  MTASuppliedInfo                 M                                         3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7.18  P1 Protocol Elements, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Element                         Class     Restrictions and Comments         3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3                                                                            3

3MTASuppliedInfo                                                             3

3  arrival                         M                                         3

3  deferred                        X       This field must be generated by   3

3                                          MTAs which perform deferred       3

3                                          delivery.                         3

3                                                                            3

3  action                          M       See section 7.7.3.2 for           3

3                                          restrictions on values of this    3

3                                          field.                            3

3                                                                            3

3  previous                        X       This field must be generated by   3

3                                          MTAs which perform rerouting.     3

3                                                                            3

3DeliveryReportEnvelope                                                      3

3  TraceInformation                M*      The first MTA in the domain to    3

3                                          receive the report adds the       3

3                                          TraceInformation.  When a report  3

3                                          is generated, the originating MTA 3

3                                          adds the TraceInformation.        3

3                                                                            3

3  InternalTraceInfo               M/P     This field is mandatory for       3

3                                          envelopes transferred between     3

3                                          MTAs within a PRMD, and           3

3                                          prohibited in messages            3

3                                          transferred outside the domain.   3

3                                          (See Note 1)                      3

3DeliveryReportContent                                                       3

3  intermediate InternalTraceInfo  P       If it were possible to include    3

3                                          this field in the delivery report 3

3                                          content, an audit and confirmed   3

3                                          report could be provided to       3

3                                          detect problems within a PRMD.    3

3                                          Efforts are being made to add     3

3                                          this field to the MOTIS           3

3                                          definition.                       3

3                                                                            3

3DeliveredInfo                                                               3

3  typeOFUA                        R*      It is the responsibility of the   3

3                                          MTA generating the report to      3

3                                          generate this element.            3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7.18  P1 Protocol Elements, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3Element                         Class     Restrictions and Comments         3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3                                                                            3

3ProbeEnvelope                                                               3

3  TraceInformation                M*      The first MTA in the domain to    3

3                                          receive the probe adds the        3

3                                          TraceInformation.  When a probe   3

3                                          is generated, the originating MTA 3

3                                          adds the TraceInformation.        3

3                                                                            3

3  InternalTraceInfo               M/P     This field is mandatory for       3

3                                          envelopes transferred between     3

3                                          MTAs within a PRMD, and           3

3                                          prohibited in messages            3

3                                          transferred outside the domain.   3

3                                          (See Note 1)                      3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Note 1: The M classification is only applicable if an

implementation is claiming conformance according to section

7.10.2, point (g) 4th bullet.

7.7.5.3   Reliable Transfer Server (RTS)

In the pUserData of PConnect, the value of

applicationProtocol should be 1.  This value was chosen

because the agreements on intra-domain connections are not

strictly P1, nor are they MOTIS.  Philosophically, it would

be good to choose a new application protocol identifier for

these agreements, but this introduces too many practical

problems.  Since these agreements are closer to P1 than to

MOTIS, the value of 1 will be used.  This will not cause

interworking problems between domains, because the only

deviation from P1 is the InternalTraceInfo, which will not

be present in messages transferred outside of a domain.

7.8  ERROR HANDLING

This section describes appropriate actions to be taken upon receipt of

protocol elements which are not supported in this profile, malformed

MPDUs, unrecognized O/R Name  forms, content errors, errors in

reports, and unexpected values for protocol elements.

7.8.1     MPDU Encoding

The MPDU should have a context-specific tag of 0, 1, or 2.  If it

does not have one of these tags, it is not possible to figure out

who originated the message.  Therefore, the way this error is

reported is a local matter.

7.8.2     Contents

Once delivery to the UA has occurred, it is not possible to

report errors in P2 information to the originator.  In addition,

it seems unreasonable to insist that the MTA that delivers a

message ensure that the P2 content of the message is acceptable.

As a result, the handling of content errors is a local matter.

7.8.3     Envelope

This section describes the handling of errors in message

envelopes.  Some of the error conditions described below may be

detected in a recipient's O/R Name.  This may limit the reporting

MTA's ability to generate a nondelivery notification that

accurately reflects the erroneous O/R Name in the

ReportedRecipientInfo.  This handling of this situation is a

local matter.

7.8.3.1   Pragmatic Constraint Violations

In all cases of pragmatic constraint violation, a

nondelivery report should be generated with a ReasonCode of

unableToTransfer, and a DiagnosticCode of

pragmaticConstraintViolation.

7.8.3.2   Protocol Violations

a)   If all required protocol elements are not present, a

nondelivery report with a ReasonCode of

unableToTransfer and a DiagnosticCode of

protocolViolation should be generated.

b)   If a protocol element is expected to be of one type,

but is encoded as another, then a nondelivery report

with a ReasonCode of unableToTransfer and a

DiagnosticCode of invalidParameters should be

generated.

7.8.3.3   O/R Names

a)   The domain that has responsibility for delivering a

message should also have the responsibility to send the

nondelivery notification if the message cannot be

delivered.  Therefore, each MTA should only validate

the O/R Names of recipients with responsibility flags

set to TRUE.  In addition, a nondelivery notification

can only be sent if the originator's O/R Name is valid.

b)   If any element in the O/R Name is unrecognized or if

the CountryName, AdministrationDomainName, and one of

PrivateDomainName and OrganizationName (and, for ADMDs,

PersonalName and OrganizationalUnit) are not all

present, then a nondelivery report should be generated

with a ReasonCode of unableToTransfer, and a

DiagnosticCode of unrecognizedORName.  If the message

can be delivered even though the ORName is invalid,

delivery is a local matter.  Note, however, that if the

message is delivered, the invalid ORName might be

propagated through the X.400 system (e.g., by

forwarding).

c)   If the O/R Name has all of the appropriate protocol

elements and the message still cannot be delivered to

the recipient, the following DiagnosticCodes may appear

in the nondelivery report: unrecognizedORName,

ambiguousORName, and uaUnavailable.

7.8.3.4   TraceInformation

a)   Since non-relaying domains need not do loop

suppression, domains with responsibility for delivering

the message need not be concerned about the semantics

of the TraceInformation, that is, arrival time and

converted EncodedInformationTypes can be provided to

the UA without inspection by the MTAs of the domain as

long as the TraceInformation is properly encoded

according to X.409.

b)   When a message is accepted for relay, the relaying

domain must check that a TraceInformation SEQUENCE has

been added by the domain that last handled the message.

If the appropriate TraceInformation was not added, this

should be treated as a protocolViolation (section

7.8.3.2).

c)   In addition, the relaying domain must check that the

information was added in the sequence defined by the

rules for adding TraceInformation in the CCITT X.400

Implementor's Guide.  If the sequence is invalid,then a

nondelivery report should be generated with a

ReasonCode of unableToTransfer and a diagnosticCode of

invalidParameters.

Note: It would be desirable for the CCITT to add a

diagnostic code of invalidTraceInformation to allow a more

meaningful description of this problem.  A request for this

new diagnostic code will be submitted.

7.8.3.5   InternalTraceInfo

This section applies only to MTAs which follow the

agreements of section 7.7.

a)   When a message is accepted for relay from another MTA

in the domain, the relaying MTA must check that an

InternalTraceInfo SEQUENCE has been added by the MTA

that last handled the message.  If the appropriate

InternalTraceInfo was not added, this should be treated

as a protocolViolation (section 7.8.3.2).

b)   In addition, the relaying MTA must check that the

information was added in the sequence defined by the

rules for adding TraceInformation in the CCITT X.400

Implementor's Guide.  If the sequence is invalid, then

a nondelivery report should be generated with a

ReasonCode of unableToTransfer and a diagnosticCode of

invalidParameters.

Note:It would be desirable for the CCITT to add a diagnostic

code of invalidTraceInformation to allow for a more

meaningful description of this problem.  A request for this

new diagnostic code will be submitted.

7.8.3.6   Unsupported X.400 Protocol Elements

The protocol elements defined in X.400 but unsupported by

this profile are:  the deferredDelivery and

PerDomainBilateralInfo parameters of the UMPDUEnvelope, the

ExplicitConversion parameter of RecipientInfo, and the

alternateRecipientAllowed and contentReturnRequest bits of

the PerMessageFlag.  Appropriate actions are described below

for domains that do not support the protocol elements.

7.8.3.6.1 deferredDelivery

The delivering domain shall do one of the following:

o    deliver at once,

o    hold for deferred delivery,

o    return a nondelivery notification with a

ReasonCode of unableToTransfer and a

DiagnosticCode of noBilateralAgreement.

7.8.3.6.2 PerDomainBilateralInfo

If a delivering domain receives this element, the

element can be ignored.

7.8.3.6.3 ExplicitConversion

If ExplicitConversion is requested the message should

be delivered if possible.  That is, if the UA is

registered to accept the EncodedInformationTypes of the

message, then the message should be delivered even

though the requested conversion could not be performed

along the route.  If delivery is not possible, then a

nondelivery report should be generated with a

ReasonCode of conversionNotPerformed with no

DiagnosticCode.

7.8.3.6.4 alternateRecipientAllowed

If a delivering domain receives this element the

element can be ignored.

7.8.3.6.5 contentReturnRequest

If a delivering domain receives this element, the

element can be ignored.

7.8.3.7   Unexpected Values for INTEGER Protocol Elements

There are three INTEGERs in the P1 Envelope.  Appropriate

actions are described below for domains receiving unexpected

values for Priority, ExplicitConversion, and ContentType.

7.8.3.7.1 Priority

Additional values for Priority have been suggested by

at least one group of implementors as upward compatible

changes to the X.400 Recommendations.  Therefore, if a

PRMD receives an unexpected value for Priority, and

this value is greater than one byte in length, a

nondelivery report should be generated with a

ReasonCode of unableToTransfer and DiagnosticCode of

invalidParameters.  If the value is less than or equal

to one byte, the PRMD can either generate a nondelivery

report as previously specified or interpret the

Priority as normal and deliver or relay the message.

7.8.3.7.2 ExplicitConversion

When an unexpected value is received for

ExplicitConversion, it should be handled as in section

7.8.3.6.3.

7.8.3.7.3 ContentType

If the ContentType is not supported by the delivering

MTA, then a nondelivery report should be generated with

a ReasonCode of unableToTransfer, and a DiagnosticCode

of contentTypeNotSupported.

7.8.3.8   Additional Elements

In the absence of multilateral agreements to the contrary,

receipt of privately tagged elements and protocol elements

in addition to those defined in X.400 will result in a

nondelivery report with a ReasonCode of unableToTransfer and

a DiagnosticCode of invalidParameters.

The exceptions to this are the MOTIS elements.  The

treatment of MPDU's containing these MOTIS extensions is

described in Section 7.6.11.

7.8.4     Reports

There is no mechanism for returning a delivery or status report

due to errors in the report itself.  Therefore the handling of

errors in reports is a local matter.

7.9  MHS USE OF DIRECTORY SERVICES

7.9.1     Directory Service Elements

a)   Recommendation X.400 recognizes the need of MHS users for a

number of directory service elements.  Directory service

elements are intended to assist users and their UAs in

obtaining information to be used in submitting messages for

delivery by the MTS.  The MTS may also use directory service

elements to obtain information to be used in routing

messages.  Some functional requirements of directories have

been identified and are listed below.

o    Verify the existence of an O/R name.

o    Return the O/R address that corresponds to the O/R name

presented.

o    Determine whether the O/R name presented denotes a user

or a distribution list.

o    Return a list of the members of a distribution list.

o    When given a partial name, return a list of O/R name

possibilities.

o    Allow users to scan directory entries.

o    Allow users to scan directory entries selectively.

o    Return the capabilities of the entity referred to by

the O/R name.

o    Provide maintenance functions to keep the directory

up-to-date.

b)   In addition to functionality, a number of operational

aspects must be considered.  These include

user-friendliness, flexibility, availability, expandability,

and reliability.

c)   Currently, these aspects of directory service elements and

procedures are under study by both the CCITT and the ISO.

Both organizations are committed to the development of a

single Directory Service specification for use by MHS and

all other OSI based applications.

Given the incomplete nature of the ongoing activities within

the CCITT and the ISO, no implementation details will be

provided now for MHS use of Directory Services.

Implementation agreements for MHS Use of Directory Services

will be issued when current activities within the CCITT and

the ISO are stable.

7.9.2     Use of Names and Addresses

a)   It is recognized that these agreements enable a wide variety

of naming and addressing attributes (see section 7.5.3.5

ORName Protocol Elements) wherein each PRMD may adopt

particular routing schemes within its domain.

b)   With the exception of the intra-domain connection

agreements:

These agreements make no attempt to recommend a standard

practice for electronic mail addressing.

c)   Inter-PRMD addressing may be secured according to practices

outside the scope of these agreements, such as:

o  manual directories

o  on-line directories

o  ORName address specifications

o  ORName address translation.

d)   Further, each PRMD may adopt naming and addressing schemes

wherein the user view may take a form entirely different

from the attributes reflected in table 7.9.  And, each PRMD

may have one user view for the originator form and another

for the recipient form, and perhaps other forms of user

addressing.  In some cases (e.g., receipt notification)

these user forms must be preserved within the constraints of

these implementation agreements.  However, mapping between

one PRMD user form to another PRMD user form, via the X.400

ORName attributes of these agreements, is outside the scope

of these agreements.

7.10 CONFORMANCE

7.10.1    Introduction

In order to ensure that products conform to these implementation

agreements, it is necessary to define the types and degrees of

conformance testing that products must pass before they may be

classified as conformant.  This section defines the conformance

requirements and provides guidelines for the interpretation of

the results from this type of testing.

This section is incomplete and will be enhanced in future

versions of this Agreement.  Later versions will reflect the

problems of conformance testing and will outline specific

practices and recommendations to aid the development of

conformance tests and procedures.

7.10.2    Definition of Conformance

For this section, the term conformance is defined by the

following:

a)   The tests indicated for this section are intended to

establish a high degree of confidence in a statement that

the implementation under test (IUT) conforms (or does not

conform) to the agreements of this section.

b)   Conformance to a service element means that the information

associated with the service element is made accessible to

the user (person or process) whenever this agreement says

that this information should be available.

Accessible means that information must be provided

describing how a user (person or process):

o    causes appropriate information to be displayed, or

o    causes appropriate information to be obtained.

c)   Conformance to P1, P2, and RTS as part of an X.400 OSI

application requires that only the external behavior of that

OSI system adheres to the relevant protocol standards.

In order to achieve conformance to this section, it is not

required  that the inter-layer interfaces be available for

testing purposes.

d)   Conformance to the protocols requires:

o    that MPDUs correspond to instances of syntactically

correct data units,

o    MPDUs in which the data present in the fields and the

presence (or absence) of those fields is valid in type

and semantics as defined in X.400, as qualified by this

profile,

o    correct sequences of protocol data units in responses

(resulting from protocol procedures).

e)   Statements regarding the conformance of any one

implementation to this profile are not complete unless a

Protocol Implementation Conformance Statement (PICS) is

supplied.

f)   The term "Implementation Under Test" (IUT) is

interchangeable with the term "system" in the definition of

conformance, and may refer to:

o    a domain, which may be one or more MTA's with

co-located or remote UA's,

o    a single instance of an MTA and co-located UA with

X.400 (P1, P2, RTS and session) software,

o    a relaying product with P1, RTS and session software,

o    a gateway product.

g)   Claiming Implementation Conformance

o    An implementation which claims to be conformant as an

ADMD must adhere to the agreements in sections 7.5 and

7.6.

o    An implementation which claims to be conformant as a

PRMD must adhere to the agreements in section 7.5.

o    An implementation which claims to be conformant as a

relaying PRMD must adhere to the agreements in section

7.5 and the appropriate sections of 7.7.

o    An implementation which claims to be conformant to the

intra-domain connection agreements must adhere to the

agreements in section 7.5 and the appropriate sections

of 7.7.

7.10.3    Conformance Requirements

7.10.3.1  Introduction

Conformance to this specification requires that all the

services listed as supported in sections 7.5, 7.6, and if

appropriate, 7.7 of these agreements are supported in the

manner defined, in either the CCITT X.400 Recommendations or

these agreements.  It is not necessary to implement the

recommended practices of section 7.12, Appendix B, in order

to conform to these agreements.

It is the intention to adopt, where and when appropriate the

testing methodology and/or the abstract test scenarios

currently being defined by the CCITT X.400 Conformance

Group.  However, it is recognized that formal CCITT

Recommendations relating to X.400 Conformance Testing will

not be available until 1988.  It is also recognized that

aspects of these agreements are outside the scope of the

CCITT, and that other organizations will have to provide

conformance tests in these cases.

7.10.3.2  Initial Conformance

This section is intended to provide guidelines to vendors

who envisage having X.400 products available prior to any

formal mechanism, or "Conformance Test Center" being made

accessible that would allow for conformance to this product

specification to be tested.

It is feasible that vendors and carriers will want to enter

bilateral test agreements that will allow for initial trials

to be carried out for the purposes of testing initial

interworking capabilities.  It is equally feasible that for

the purposes of testing interoperability, only a subset of

this specification will initially be tested.

Note: By claiming conformance to this subset of information

the vendor or carrier CANNOT claim conformance to this

entire specification.

There are two aspects to the requirements, interworking and

service, as described in the following sections.

7.10.3.2.1     Interworking

The interworking requirements for conformance implies

that tests be done to check for the syntax and

semantics of protocol data elements for a system as

defined by the classification scheme of sections

7.5.2.1.1 and 7.7.5.2.  For a relay system, the correct

protocol elements should be relayed as appropriate.

For a recipient system, a message with correct protocol

elements must not be rejected where appropriate.

7.10.3.2.2     Service

For information available to the recipients via the

IPMessage Heading and Body, the following should be

made accessible:

o    IPMessage ID - only the PrintableString portion of

the IPMessageId needs to be accessible.

o    subject,

o    primaryRecipients,

o    copyRecipients,

o    blindcopyRecipients,

o    authorizingUsers,

o    originator,

o    inReplyTo,

o    replyToUsers,

o    importance,

o    sensitivity,

o    IA5Text Bodypart.

7.11    APPENDIX A:    INTERPRETATION OF X.400 SERVICE ELEMENTS

The work on service element definitions is limited to those that are

defined as 'supported' in section 7.5 of this specification.  Furthermore

it is not the intent of this section to define how information should be

made available or presented to a MHS user, nor is it intended to define

how individual vendors should design their products.  In addition,

statements on conformance to a specific service element and the

allocation of error codes that are generated as a result of violations of

the service should be defined in the sections on conformance and errors

as part of the main product specification.  The main objective is to

provide clarification, where required, on the functions of a service

element, and in particular what the original intent of the

Recommendations were.

SERVICE ELEMENTS

The following Service Elements defined in X.400 have been examined and

require further text to be added to their definitions to represent the

proposed implementation of these service elements by the X.400 SIG.

The service element clarifications are to be taken in the context of this

profile.

Service elements not referenced in this section are as defined in X.400.

PROBE

A PRMD need not generate probes.

If a probe is addressed to and received by a PRMD, the PRMD must respond

with a Delivery Report as appropriate at the time the probe was

processed.

DEFERRED DELIVERY

In the absence of bilateral agreements to the contrary, Deferred Delivery

and Deferred Delivery Cancellation are local matters (i.e., confined to

the originating domain) and need not be provided.

The extension of Deferred Delivery beyond the boundaries of the

initiating domain is via bilateral agreement as specified in Section

3.4.2.1 of X.411.

Content Type Indication

It is required that both an originating and recipient domain be able to

support P2 content type.  The ability for domains to be able to exchange

content types other than P2 will depend on the existence of bilateral or

multi-lateral agreements.

Original Encoded Information Types Indication

It is required that both an originating and recipient domain be able to

support IA5 text.  Support for other encoded information types, for the

purposes of message transfer between domains, will depend on the

existence of bilateral or multi-lateral agreements.

The use of the 'unspecified' form of encoded information type should only

be used when the UMPDU content represents an SR-UAPDU or contains an

auto-forwarded IM-UAPDU.

The original encoded information type of a message is not meaningful

unless a message is converted en route to the recipient.  These

agreements support only IA5 text, which should not undergo conversion.

The original encoded information types should be made accessible to the

recipient for upward compatibility with the use of non-IA5 text message

body parts.

Registered Encoded Information Types

A UMPDU with an 'unspecified' value for Original Encoded Information Type

shall be delivered to the UA.

Delivery Notification

The UAContentID may be used by the recipient of the delivery notification

for correlation purposes.

Disclosure of Other Recipients

This service is not made available by originating MTAE's to UAE's, but

must be supported by relaying and recipient MTAE's.

By supporting the disclosure of other recipients the message recipient

can be informed of the O/R names of the other recipient(s) of the

message, as defined in the P1 envelope, in addition to the O/R

Descriptors within the P2 header.

These agreements do not support initiation of disclosure of other

recipients, but the information associated with it should be made

accessible to the recipient for upward compatibility with support for the

initiation of this service element.

Typed Body

As defined in X.400 with the addition of the Private Body Types that are

to be supported.  At present there is no mechanism provided within X.420

that would allow you to respond to reception of an unsupported body type.

Action taken in this situation is a local matter.

Blind Copy Recipient Indication

It should be considered that the recipient's UA acts on behalf of the

recipient, and therefore may choose to disclose all BCC recipients to

each other.  Therefore it is the responsibility of the originating domain

to submit two or more messages, depending on whether or not each BCC

should be disclosed to each other BCC.

Auto Forwarded Indication

A UA may choose not to forward a message that was previously

auto-forwarded. In addition there is no requirement for an IPM UA that

does not support non-receipt or receipt notification to respond with a

non-receipt notification when a message is auto-forwarded.

Primary and Copy Recipients Indication

It is required that at least one primary recipient be specified; however,

for a forwarded message this need not be present.  The recipient UA

should be prepared to accept no primary and copy recipients to enable

future interworking with Teletex, Fax, etc.

Sensitivity Indication

A message originator should make no assumptions as to the semantic

interpretation by the recipients UA regarding classifications of

sensitivity. For example, a personal message may be printed on a shared

printer.

Reply Request Indication

In requesting this service an originator may additionally supply a date

by which the reply should be sent and a list of the intended recipients

of the reply. If no such list is provided then the initiator of the reply

sends the reply to the originator of the message and any recipients the

reply initiator wishes to include. The replytoUsers and the replyBy date

may be specified without any explicit reply being requested. This may be

interpreted by the recipient as an implicit reply request. Note that for

an auto-forwarded message an explicit or implicit reply request may not

be meaningful.

Body Part Encryption

The original encoded information type indication includes the encoded

information type(s) of message body parts prior to encryption by the

originating domain.  The ability for the recipient domain to decode an

encrypted body part is a local matter.  Successful use of this facility

can only be guaranteed if there exists bilateral agreements to support

the exchange of encrypted body parts.

Forwarded IP message Indication

The following use of the original encoded information type in the context

of forwarded messages is clarified:

o    If forwarding a private message body part the originator of the

forwarded message shall set the original encoded information

types in the P1 envelope to undefined for that body part.

o    The encoded information types of the message being forwarded

should be reflected in the new original encoded information types

being generated.

o    See Appendix 7B on recommended practices for the use of the

delivery information as part of Forwarded IP-message.

Multipart Body

It is the intent of multipart bodies to allow for the useful and

meaningful structuring of a message that is constructed using differing

body part types.  For example, it is not recommended that a message made

up of only IA5 text should be represented as a number of IA5 body parts,

each one representing a paragraph of text.

7.12   APPENDIX B:    RECOMMENDED X.400 PRACTICES

It is not necessary to follow the recommended practices when claiming

conformance to these agreements.

7.12.1    RECOMMENDED PRACTICES IN P2

1.   ORDescriptor

Vendors following the NBS/OSI Workshop guidelines shall,

whenever possible, generate the ORName portion of an

ORDescriptor in ALL IPM heading fields.

2.   ForwardedIPMessage BodyParts

ForwardedIPMessage BodyParts should be nested no deeper than

eight.  There is no restriction on the number of

ForwardedIPMessage BodyParts at any given depth.

3.   DeliveryInformation

It is strongly recommended that DeliveryInformation be

supplied in both forwarded and autoforwarded message body

parts. DeliveryInformation is useful when a message has

multiple forwarded message body parts because without it,

the EncodedInformationType(s) of the component forwarded

messages cannot be deduced easily.  DeliveryInformation is

useful for autoforwarded messages because the

EncodedInformationType of an autoforwarded message is

"unspecified" and the EncodedInformationType(s) of the

message cannot be determined easily without it. Absence of

the EncodedInformationType(s) makes it difficult for a UA to

easily determine whether the message can be rendered.

7.12.2    RECOMMENDED PRACTICES IN RTS

1.   In the case where S-U-ABORT indicates a temporaryProblem,

reestablishment of the session should not be attempted for a

"sensible" time period (typically not less than five

minutes).

In instances where this delay is not required or necessary,

report a localSystemProblem.

2.   S-U-EXCEPTION-REPORT reason codes can be interpreted as

follows:

o    receiving ability jeopardized (value 1)

Possible meaning: The receiving RTS knows of an

impending system shutdown.

o    local ss-User error (value 5)

Possible meaning:  The receiving RTS needs to

resynchronize the session dialogue.

o    irrecoverable procedure error (value 6)

Possible meaning:  The receiving RTS has had to delete

a partially received APDU, even though some minor

synchronization points have been confirmed.

o    non specific error (value 0)

Possible meaning:  The receiving RTS cannot handle the

APDU (for example, because it was too large) and wishes

to inform the sending RTS not to try again.

o    sequence error (value 3):

Possible meaning: The S-ACTIVITY-RESUME request

specified a minor synchronization point serial  number

which does not match the checkpoint data.

3.   For purposes of identifying an MTA during an RTS Open, OSI

addressing information should be used.  This addressing

information is conveyed by lower layer protocols and is

reflected by the calling and called SSAP parameters of the

S-CONNECT primitives.

MTA validation and identification are related, but separate,

functions.  The mTAName and password protocol elements of

the RTS user data should be used for validation, rather that

identification, of an MTA.  The RTS initiator and responder

may independently require each other to supply mTAName and

password.

The CallingSSUserReference parameter of the S-CONNECT

primitives should only have meaning to the entity that

encoded it and should not be used to identify an MTA.

7.12.3    RECOMMENDED PRACTICES FOR ORName

Table 7.9 stipulates that the StandardAttributeList must contain

either PrivateDomainName or OrganizationName.  It is recommended

that, for both originator and recipients in a private domain, the

PrivateDomainName field be used.

It is recommended that there should be a DomainDefinedAttribute

to be used in addressing UAs in existing mail systems, in order

to curtail the proliferation of different types of

DomainDefinedAttributes used for the same purpose.  The syntax of

this DomainDefinedAttribute conforms to the CCITT Pragmatic

Constraints, and thus has a maximum value length of 128 octets

and a type length of 8 octets, each of type Printable String.

Only one occurrence is allowed.

This DomainDefinedAttribute has the type name "ID" (in

uppercase).  It contains the unique identifier of the UA used in

addressing within the domain.  This DomainDefinedAttribute is to

be exclusively used for routing within the destination domain

(i.e., once routed to that domain via the mandatory components of

the StandardAttributeList); any other components of the

StandardAttributeList may be provided.  If they conflict delivery

is not made.

The contents of this parameter need not be validated in the

originating domain or any relaying domain, but simply transferred

intact to the next MTA or domain.

Class 2 and class 3 MTAs in a PRMD should allow administrators to

decide the number of OrganizationalUnits that should appear in

user names, instead of imposing a software controlled limit which

is less than four.  This is desirable because when two different

vendors impose different limits on the number of

OrganizationalUnits in a name, it becomes difficult for the

administrator to choose a sensible naming scheme.

There are existing mail systems that include a small set of non-

Printable String characters in their identifiers.  For these

systems to communicate with X.400 messaging systems, either for

pass-through service or delivery to X.400 users, gateways will be

employed to encode these special characters into a sequence of

Printable String characters.  This  conversion should be

performed by the gateway according to a common scheme and before

insertion in the ID DDA, which is intended to carry electronic

mail identifiers.  X.400 User Agents may also wish to perform

such conversions.

It is recommended that the following symmetrical encoding and

decoding algorithm for non-Printable String characters be

employed by gateways.  The encoding algorithm maps an ID from an

ASCII representation to a PrintableString representation.  Any

non-printable string characters not specified in the table are

covered by the category "other" in the table below.

The principal conversion table for the mapping is as follows:

Table 7B.1  Printable string to ASCII mapping

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3  ASCII Character     Printable String Character                            3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3  % (percent)              (p)                                              3

3  @ (at sign)              (a)                                              3

3  ! (exclamation)          (b)                                              3

3  " (quote mark)           (q)                                              3

3  _ (underline)            (u)                                              3

3  ( (left paren.)          (l)                                              3

3  ) (right paren.)         (r)                                              3

3  other                    (3DIGIT)                                         3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

where 3DIGIT has the range 000 to 377 and is interpreted as the octal

encoding of an ASCII character.

To encode an ASCII representation to a PrintableString, the table and the

following algorithm should be used:

IF current character is in the encoding set THEN

encode the character according to the table above

ELSE

write the current character;

continue reading;

To decode a PrintableString representation to an ASCII representation,

the table and the following algorithm should be used:

IF current character is not "(" THEN

write character

ELSE

{

look ahead appropriate characters;

IF composite characters are in the above table THEN

decode per above table

ELSE

write current character;

}

continue reading;

Class 2 and class 3 MTAs in a PRMD should allow administrators to

decide the number of OrganizationalUnits that should appear in

user names, instead of imposing a software controlled limit which

is less than four.  This is desirable because when two different

vendors impose different limits on the number of

OrganizationalUnits in a name, it becomes difficult for the

administrator to choose a sensible naming scheme.

7.12.4    POSTAL ADDRESSING

For domains wishing to support postal (or physical) delivery

options, the following interim set of "nationally-defined" domain

defined attributes are recommended.  The CCITT will define

Standard Attributes in support of physical delivery in its 1988

Recommendations; this is only an interim solution.

CCITT will also be addressing the services associated with

physical delivery. This interim solution does not address the

end-to-end service aspects of physical delivery; in particular,

the following IPM service elements do not currently extend

outside of the X.400 environment:

o    alternate Recipient Assignment

o    PROBE

o    Receipt Notification / Non-Receipt Notifications

o    Grade of delivery

"Delivery" means passing a message from the MTS to the physical

delivery system (PDS), and not to the user (or user agent).

The following three DDAs are recommended to be used to specify a

postal (or physical) address:

CNTRPC -  encodes the country and postal code for postal

delivery.  The DDA value is of the form

"Country?Postalcode"  (for example, "USA?22096").  The

country field is optional, the postal code is

optional; the separator ("?") is not.  If both country

and postal code are missing, this DDA should not be

specified.

PDA 1 -   The country and postal code fields are free-form text.

PDA 2 -   These two DDA (signifying Postal Delivery Address

strings 1 and  2) form a 256 character free-form

postal address.  Fields are separated by a question

mark ("?").  There is no implied  separator between

PDA1 and PDA2.  The meaning of the fields are defined

by each domain supporting the physical delivery

interface.  PDA1 contains the first 128 characters,

PDA2 the next 128 characters.  If the PDA string is

less than 128 characters, PDA2 is not used.

For example, if the domain interprets the PDA fields as lines,

the address

Mr. John Smith

Conway Steel

123 Main Street

Reston VA 22096

would be encoded as follows:

type = "PDA1"  value = "Mr. John Smith?Conway Steel?123 Main

Street?Reston VA"

CNTRPC = "?22096"

7.12.5    EDI use of X.400

7.12.5.1  Introduction and Scope

This is a guideline for EDI data transfer in an X.400

environment conforming to the NBS agreements.  These

recommended practices outline procedures for use in

transferring EDI transactions between trading partner

applications in an attempt to facilitate actual X.400

implementation by EDI users.

The scope of this guideline is to describe specific

recommendations for adopting X.400 as the data transfer

mechanism between EDI applications.

7.12.5.2  Model

The MHS recommendations can accommodate EDI through the

approach illustrated below.  Many Message Transfer (MT)

service elements defined in the X.400 recommendations are

particularly useful to the EDI application.

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                  X.400 Message (1 EDI interchange)    3

3                              ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?  3

3                              3                                     3  3

3                              3   ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?  3  3

3                              3   3                              3  3  3

3   Envelope   ------------------->3         P1 Control           3  3  3

3                              3   3         Information          3  3  3

3                              3   @DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY  3  3

3                              3                                     3  3

3                              3                                     3  3

3                              3                                     3  3

3                              3   ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?  3  3

3                              3   3         One                  3  3  3

3   Content    ------------------->3         EDI                  3  3  3

3                              3   3         Interchange          3  3  3

3                              3   @DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY  3  3

3                              3                                     3  3

3                              @DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY  3

3 MHS Message                                                           3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

This diagram depicts an EDI content (1 EDI interchange)

enveloped by the P1 MHS envelope.  All the MT Services

defined in the X.400 Recommendations may be used for EDI.

However, it is not required to support optional or non-

essential services to exchange EDI data between EDI users.

When an EDI user submits an EDI Trade Document to the EDI

User Agent, the EDI UA will submit the EDI content plus P1

envelope to the Message Transfer System (MTS).

ZDDDDDDDDDDD?             ZDDDDDDDDDDDD?              ZDDDDDDDDDDD?

3           3             3    EDI     3              3    EDI    3

3   MTS     3<----------->3    UA      3<------------>3    User   3

3           3             3            3              3           3

@DDDDDDDDDDDY             @DDDDDDDDDDDDY              @DDDDDDDDDDDY

The EDI UA must support the essential MT Services as defined

in these Agreements; for example, as a minimum, to provide

default values for services not elected by the EDI user,

such as Grade of Delivery.

Note: MT Services are not necessarily made available by the

EDI UA to the EDI user.

7.12.5.3  Protocol Elements Supported for EDI

The following P1 protocol elements will be used to support

EDI applications:

Content Type

For EDI applications, the content type will be 0

(undefined content).

Original Encoded Information Types

Any EIT defined in the X.400 Recommendations may be

used to specify the encoding of EDI content.  However,

for ANSI X12 EDI applications in particular, it is

expected that the "undefined" and "Ia5Text" EIT's will

normally be used, with "undefined" used to signify the

EBCDIC character set.

7.12.5.4  Addressing and Routing

It is anticipated that connection of some existing systems

to an X.400 service for EDI purposes will be by other than

X.400 protocols, at least in the short term.

EDI messages entering the X.400 environment will therefore

need to have X.400 O/R Names added to identify the

origination and recipient trading partners, typically by

means of local directory services in the origination domain

which will map EDI identifiers/addresses into O/R Names.

Such O/R Names will contain Standard Attributes as defined

in Table 7.9 and for recipient trading partners will at

least identify the destination domain.

In the case of trading partners outside the X.400

environment, it is expected, however, that there will be

cases where message delivery will require the provision of

addressing information beyond that which can be carried in

Standard Attributes.  In such cases, Domain Defined

Attributes are recommended to be used.

The syntax of this DDA is as defined in Table 7.9, with a

single occurrence having the type name "EDI" (uppercase) and

a value containing the identifier/address of the trading

partner.  For ASC X12 purposes, specifically, this value

will comprise the 2 digit interchange ID qualifier followed

by the interchange ID (max 15 characters).  Routing on this

DDA shall only occur, if at all, in the destination domain.

7.12.6    USA Body Parts

It is recommended that UAs can generate any USA Body Part, as

defined in section 7.5.3.6.2, and that they can receive such body

parts as well.  reception of USA Body Parts does not imply

further processing by the UA, but merely that the body part is

made available, with a indication of its registered body part

identifier, to another process or deposition in a file.

Generation implies the reverse of this process.

7.13   APPENDIX C:    RENDITION OF IA5Text AND T61String CHARACTERS

7.13.1    GENERATING AND IMAGING IA5Text

The characters that may be used in an IA5String are the graphic

characters (including Space), control characters and Delete of

the IA5 character repertoire ISO 646.

The graphic characters that may be used with a guaranteed

rendition are those related with positions 2/0 to 2/2, 2/5 to

3/15, 4/1 to 5/10, 5/15 and 6/1 to 7/10 in the basic 7-bit code

table.

The other graphic characters may be used but have no guaranteed

rendition.

The control characters that may be used but have no guaranteed

effect are a subset consisting of the format effectors 0/10 (LF),

0/12 (FF) and 0/13 (CR) provided they are used in one of the

following combinations:

CR LF          to start a new line

CR FF          to start a new page (and line)

LF .. LF       to show empty lines (always after one of the

preceding combinations).

The other control characters or the above control characters in

different combinations may be used but have no guaranteed effect.

The character Delete may occur but has no guaranteed effect. The

IA5String in a P2 IA5Text BodyPart represents a series of lines

which may be divided into pages.  Each line should contain from 0

to 80 graphic characters for guaranteed rendition.  Longer lines

may be arbitrarily broken for rendition.  Note that X.408 states

that for conversion from IA5Text to Teletex, the maximum line

length is 77 characters.

7.13.2    GENERATING AND IMAGING T61String

For further study.

7.14   APPENDIX D:    DIFFERENCES IN INTERPRETATION DISCOVERED THROUGH

TESTING OF THE MHS FOR THE CeBit 87 DEMONSTRATION

Several interworking problems were discovered through multi-vendor

testing.  These problems, and recommendations for solutions to them are

discussed in this appendix.

7.14.1    ENCODING OF RTS USER DATA

The password is defined as an ANY in the X.400 Recommendations,

and implementor's groups have decided to use an IA5String for

this field.  There was some confusion about what the X.409

encoding for this IA5String would be, and the correct encoding

is:

class:    context specific

form:     constructor

id code:  1

length:   length of contents

contents: (primitive encoding)

IA5String:     16

length:        length of contents

contents:      the password string

class:    context specific

form:     constructor

id code:  1

length:   length of contents

contents: (constructor encoding) left as an exercise for the

reader

Implementations should be prepared to receive any X.409 type for

the password because of its definition as an ANY.

7.14.2    EXTRA SESSION FUNCTIONAL UNITS

One vendor proposed more than the required set of functional

units on opening the session connection, and the receiver

rejected the connection.  All debate aside about whether the

initiator should have proposed units outside of the required set,

or whether the receiver should have rejected the connection, the

set of functional units can be negotiated in a straightforward

way.  The following is recommended.

If the initiator proposes using more than the required set of

functional units, the responder should specify the set of

functional units that it would like to use (which should include

the required set) in the open response.  The session

implementations will automatically use the intersection of the

units proposed by both sides.

If the initiator proposes using less than the required set of

functional units, the responder should reject the connection.

Unfortunately, there is not an appropriate RefuseReason for

rejecting the connection, so instead of refusing the connection

in the response to the S-CONNECT, the receiver should issue an S-

U-ABORT with an AbortReason of protocolError.  Note that it is

valid to issue an S-U-ABORT instead of responding to the S-

CONNECT.  A problem report has been submitted to the CCITT

requesting the addition of a RefuseReason for this situation.

If the responder proposes using less than the required set of

functional units, the session connection is established before

the initiator can check for this.  If too few functional units

have been proposed, the initiator should abort the connection

using S-U-ABORT, with an abort reason of protocolError.

7.14.3    MIXED CASE IN THE MTA NAME

The MTA name is frequently exchanged over the telephone when two

systems are being configured to communicate with one another.  In

one such telephone exchange, the casing of the MTA name was not

specified, the MTA name consisted of both upper and lower case

letters, and one of the implementations compared MTA names for

equality in a case sensitive manner.  Consequently, connections

failed until the problem was detected and repaired.  It is

recommended that the MTA name be compared for equality in a case

insensitive manner, and that the password be compared for

equality in a case sensitive manner.

7.14.4    X.410 ACTIVITY IDENTIFIER

The X.400 Implementor's Guide recommends that the activity

identifier be X.409 encoded, but this is only a recommendation

and not a requirement.  Consequently, receiving systems cannot

assume that the activity identifier will be X.409 encoded.

7.14.5    ENCODING OF PER RECIPIENT FLAG AND PER MESSAGE FLAG

In the definition of the PerRecipientFlag in X.411, there is a

statement that the last three bits are reserved, and should be

set to zero.  It is unclear whether those bits are unused in the

X.409 encoding.  Receivers should accept encodings with either

zero or three unused bits.  A problem report has been submitted

to the CCITT asking for clarification.

Though there is not any statement in X.411 about the last four

bits of the PerMessageFlag, some vendors have encoded this with

zero unused bits, and some have encoded it with four unused bits.

The PerMessageFlag should be encoded with at least four unused

bits.

7.14.6    ENCODING OF EMPTY BITSTRINGS

There are three valid encodings for an empty bitstring: a

constructor of length zero, a constructor of indefinite length

followed by the end-of-contents terminator, and a primitive of

length one with a zero octet as the value.

7.14.7    ADDITIONAL OCTETS FOR BITSTRINGS

Nothing in X.409 constrains an implementation from sending two,

three, four, or even more octets for a bitstring that fits into

one octet, with the undefined bits set to zero.  Note that the

number of excess octets is bounded by the pragmatic constraints

guidelines of the CCITT X.400 Implementor's Guide for all of the

bitstrings in P1.

7.14.8    APPLICATION PROTOCOL IDENTIFIER

If a value other that 1 is received in the applicationProtocol of

the  pUserData in the PConnect, NBS implementations will reject

the connection.  If CEN/CENELEC implementations receive a value

other than 8883 for this field, they will reject the connection.

This is an unfortunate state of affairs, because if NBS

implementations accept the value of 8883 without supporting the

MOTIS service elements, they would be misrepresenting themselves.

To make matters worse, CEPT uses a value of 1, but relays MOTIS

elements, which means that MOTIS elements will be relayed to

implementations using a value of 1 to demonstrate that they do

not support MOTIS.  Work is continuing to try to find a solution

that will allow European implementations to interwork with U.S

implementations.

7.14.9    INITIAL SERIAL NUMBER IN S-CONNECT

This should be implemented in accordance with section 3.5.1 E4 of

the Implementors' Guide.

7.14.10   CONNECTION DATA ON RTS RECOVERY

It is clarified that the ConnectionData is identical in both the

S-CONNECT.request and the S-CONNECT.response.  The value of the

ConnectionData is the old Session Connection Identifier.

7.14.11   ACTIVITY RESUME

If an activity is being resumed on a new session connection, it

is not clear from X.410 and X.225 whether all four of the called-

ss-user reference, the calling-ss-user reference, the common

reference, and the additional reference information should be

specified in the S-ACTIVITY-RESUME, or whether one of the ss-

user-references should be absent.  It is also unclear whether the

called-ss-user reference should be identical to the calling-ss-

user reference if both are present.  Consequently, receivers

should be tolerant of this situation.  Appropriate problem

reports will be submitted to the CCITT asking for clarification.

7.14.12   OLD ACTIVITY IDENTIFIER

The Old Activity Identifier in S-ACTIVITY-RESUME refers to the

original activity identifier.

7.14.13   NEGOTIATION DOWN TO TRANSPORT CLASS 0

For European implementations, X.410 specifies that class 0

transport must be supported.  However, it is permissible for an

initiator to propose a higher class as the preferred class,

provided that class 0 appears as the alternate class in the T-

Connect PDU.  A responding implementation can choose to use

either the preferred or alternate class, but again, must be able

to use class 0.  In other words, for private to private

connections in Europe, class 0 transport is required.

This conflicts with the NBS agreements, since class 0 is only

required if one of the partners in a connection is an ADMD.

7.15APPENDIX E:WORLDWIDE X.400 CONFORMANCE PROFILE MATRIX

Y CONFORMANCE (E)

implies a conformance problem for European products in the U.S.

Y CONFORMANCE (US)

implies a conformance problem for U.S. products in Europe.

o    The A/311 profile is specified in Env 41 202, the A/3211 profile

in Env 41 201

o    No TTC protocol classification for RTS exists.

o    The notation X/Y indicates "X" for PRMDs and "Y" for ADMDs, i.e.

"M/G" would be Mandatory for PRMDs and Generatable for ADMDs.

Table 7E.1  Protocol element comparison of RTS

ZDDDDDDDDDDDDDDDDDDDDDDDBDDDDDDDBDDDDDDDDDDBDDDDDDDDDBDDDDDDDDDDDDDDDDDDDDDDD?

3RTS element            3  NBS  3   A/311  3  A/3211 3  PROBLEM Y/N          3

CDDDDDDDDDDDDDDDDDDDDDDDEDDDDDDDEDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDD4

3PConnect               3   M   3    M     3    M    3  N                    3

3 DataTransferSyntax    3   M 0 3    M 0   3    M 0  3  N                    3

3PUserData              3   M   3    M     3    M    3  N                    3

3 checkpointSize        3   H   3    H     3    H    3  N                    3

3 windowSize            3   H   3    H     3    H    3  N                    3

3 dialogueMode          3   H   3    H     3    H    3  N                    3

3 connectdata           3   M   3    M     3    M    3  N                    3

3 applicationProtocol   3   G 1 3    H 1   3 R 8883  3  N                    3

3                       3 H 88833          3         3                       3

3 ConnectionData        3       3          3         3                       3

3  Open                 3   G   3    G     3    G    3  N                    3

3  Recover              3   G   3    H     3    G    3  N                    3

3                       3       3          3         3                       3

3 Open                  3       3          3         3                       3

3  RTSUserData          3   G   3    G     3    G    3  N                    3

3                       3       3          3         3                       3

3 Recover               3       3          3         3                       3

3  SessionConnectionID  3   G   3    G     3    G    3  N                    3

3                       3       3          3         3                       3

3RTSUserData            3       3          3         3                       3

3  MTAName              3   G   3    G     3    G    3  N                    3

3                       3       3          3         3                       3

3  Password             3   G   3    G     3    G    3  N                    3

3                       3       3          3         3                       3

3  null                 3   G   3    G     3    G    3  N                    3

3                       3       3          3         3                       3

3SessionConnectionID    3       3          3         3                       3

3 CallingUserReference  3   M   3    M     3    M    3  N                    3

3                       3       3          3         3                       3

3 CommonReference       3   M   3    M     3    M    3  N                    3

3 AdditionalRefInfo     3   H   3    H     3    H    3  N                    3

3                       3       3          3         3                       3

3PAccept                3   G   3    G     3    G    3  N                    3

3 DataTransferSyntax    3   M 0 3    M 0   3    M 0  3  N                    3

@DDDDDDDDDDDDDDDDDDDDDDDADDDDDDDADDDDDDDDDDADDDDDDDDDADDDDDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7E.1  Protocol element comparison of RTS, continued

ZDDDDDDDDDDDDDDDDDDDDDDDBDDDDDDBDDDDDDDDDDDBDDDDDDDDDBDDDDDDDDDDDDDDDDDDDDDDD?

3RTS element            3 NBS  3  A/311    3 A/3211  3 PROBLEM (Y/N)         3

CDDDDDDDDDDDDDDDDDDDDDDDEDDDDDDEDDDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDD4

3                       3      3           3         3                       3

3PUserData              3  M   3    M      3    M    3 N                     3

3 CheckpointSize        3  H   3    H      3    H    3 N                     3

3 WindowSize            3  H   3    H      3    H    3 N                     3

3 ConnectionData        3  M   3    M      3    M    3 N                     3

3                       3      3           3         3                       3

3PRefuse                3  G   3    G      3    G    3 N                     3

3 RefuseReason          3  M   3    M      3    M    3 N                     3

3                       3      3           3         3                       3

3SSUserData             3  G   3    G      3    G    3 N                     3

3 (in S-TOKEN-PLEASE)   3      3           3         3                       3

3                       3      3           3         3                       3

3AbortInformation       3  G   3    G      3    G    3 N                     3

3 (in S-U-ABORT)        3      3           3         3                       3

3 AbortReason           3  H   3    H      3    H    3 N                     3

3 reflectedParameter    3  X   3    X      3    X    3 N                     3

@DDDDDDDDDDDDDDDDDDDDDDDADDDDDDADDDDDDDDDDDADDDDDDDDDADDDDDDDDDDDDDDDDDDDDDDDY

Table 7E.2  Protocol element comparison of P1

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDBDDDDDDDBDDDDDDBDDDDDDBDDDDDDDDDDDDDDDDDDDD?

3P1 Protocol                         3 NBS 3 A/311 3A/32113 TTC  3   PROBLEM (Y/N)    3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDEDDDDDDDEDDDDDDEDDDDDDEDDDDDDDDDDDDDDDDDDDD4

3                                    3     3       3      3      3                    3

3        ORname                      3     3       3      3      3                    3

3          StandardAttributeList     3  M  3   M   3   M  3  M   3   N See Note 4     3

3          DomainDefAttributeList    3  X  3   X   3   X  3  G   3   Y See Note 5     3

3                                    3     3       3      3      3                    3

3        StandardAttributeList       3     3       3      3      3                    3

3          CountryName               3  R  3   R   3   R  3  M   3   N                3

3                                    3     3ISO R  3   R  3      3   N                3

3                                    3     3X.121 H3   H  3      3  Y Conformance (E) 3

3                                    3     3Other X3   X  3      3  Y Prot Vio        3

3          AdministrationDomainName  3  R  3   R   3   G  3  M   3   N                3

3             ... if PrintableString 3     3   R   3   G  3      3   N                3

3             ... if numericString   3     3   H   3   H  3      3  Y Conformance (E) 3

3          X.121 Address             3  X  3  X/R  3   X  3      3Y Conf(US)See Note 13

3          Terminal ID               3  X  3  X/G  3   X  3      3Y Conf(US)See Note 13

3          PrivateDomainName         3  G  3   G   3   G  3  G   3   N                3

3                                    3     3       3      3      3                    3

3          OrganizationName          3  G  3   G   3   G  3  G   3   N                3

3          UniqueUAidentifier        3  X  3  X/G  3   X  3      3Y Conf(US)See Note 13

3          PersonalName              3  G  3   G   3   G  3  G   3   N                3

3          OrganizationalUnit        3  G  3   G   3   G  3  G   3   N                3

3                                    3     3       3      3      3                    3

3        DomainDefinedAttribute      3  X  3   X   3   X  3  G   3   N                3

3          Type                      3  M  3   M   3   M  3  M   3   N                3

3          Value                     3  M  3   M   3   M  3  M   3   N                3

3                                    3     3       3      3      3                    3

3        PersonalName                3     3       3      3      3                    3

3          Surname                   3  M  3   M   3   M  3  M   3   N                3

3          GivenName                 3  G  3   G   3   G  3  G   3   N                3

3          Initials                  3  G  3   G   3   G  3  G   3   N                3

3                                    3     3       3      3      3                    3

3          GenerationQualifier       3  G  3   X   3   X  3  X   3  Y Conformance (E) 3

3                                    3     3       3      3      3                    3

3        GlobalDomainIdentifier      3     3       3      3      3                    3

3          CountryName               3  M  3   M   3   M  3  M   3   N                3

3          AdministrationDomainName  3  M  3   M   3   G  3  M   3  Y Proto Vio       3

3          PrivateDomainIdentifier   3 R/H 3   H   3   R  3 M/X  3   N                3

3                                    3     3       3      3      3                    3

3        MPDU                        3     3       3      3      3                    3

3          UserMPDU                  3  G  3   G   3   G  3  G   3   Y TTC required   3

3                                    3     3       3      3      3     MPDU size is   3

3                                    3     3       3      3      3     32K            3

3          DeliveryReportMPDU        3  G  3   G   3   G  3  G   3   N                3

3                                    3     3       3      3      3                    3

3          ProbeMPDU                 3  H  3   H   3   H  3  H   3   N                3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDADDDDDDDADDDDDDADDDDDDADDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7E.2  Protocol element comparison of P1, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDBDDDDDDDBDDDDDDBDDDDDDBDDDDDDDDDDDDDDDDDDDD?

3P1 Protocol                         3 NBS 3 A/311 3A/32113 TTC  3   PROBLEM (Y/N)    3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDEDDDDDDDEDDDDDDEDDDDDDEDDDDDDDDDDDDDDDDDDDD4

3                                    3     3       3      3      3                    3

3        UserMPDU                    3     3       3      3      3                    3

3          UMPDUenvelope             3  M  3   M   3   M  3  M   3   N                3

3          UMPDUcontent              3  M  3   M   3   M  3  M   3   N                3

3                                    3     3       3      3      3                    3

3        UMPDUenvelope               3     3       3      3      3                    3

3          MPDUidentifier            3  M  3   M   3   M  3  M   3   N                3

3          originatorORname          3  M  3   M   3   M  3  M   3   N                3

3          originalEncodedTypes      3  G  3   H   3   H  3  G   3   Y Conformance (E)3

3                                    3     3       3      3      3                    3

3          ContentType               3  M  3   M   3   M  3  M   3   N                3

3          UAcontentID               3  H  3   H   3   H  3  H   3   N                3

3          Priority                  3  G  3   G   3   G  3  G   3   N                3

3          PerMessageFlag            3  G  3   G   3   G  3  G   3   N                3

3          DeferredDelivery          3  X  3   X   3   X  3  X   3   N                3

3          PerDomainBilatInfo        3  X  3   X   3   X  3  X   3   N                3

3          RecipientInfo             3  M  3   M   3   M  3  M   3   Y TTC MPDU 32K   3

3          TraceInformation          3  M  3   M   3   M  3  M   3   N                3

3MOTIS->   LatestDelivery            3     3       3   X  3      3   N                3

3MOTIS->   InternalTraceInfo         3 M/P 3       3   P  3      3   N                3

3        UMPDUcontent                3  M  3   M   3   M  3  M   3   N                3

3                                    3     3       3      3      3                    3

3        MPDUidentifier              3     3       3      3      3                    3

3          GlobalDomainIdent         3  M  3   M   3   M  3  M   3   N                3

3          IA5string                 3  M  3   M   3   M  3  M   3   N                3

3                                    3     3       3      3      3                    3

3        PerMessageFlag              3     3       3      3      3                    3

3          DiscloseRecipients        3  H  3G @ MTL3   H  3  H   3  Y Conformance (US)3

3                                    3     3H at UA3   ?  3      3  Y Conformance (US)3

3          ConversionProhibited      3  G  3   G   3   G  3  G   3   N                3

3          AlternatRecipAllowed      3  H  3G @ MTL3   H  3  X   3  Y Conformance (US)3

3                                    3     3H at UA3   ?  3      3  Y Conformance (US)3

3          ContentReturnRequest      3  X  3   X   3   X  3  X   3                    3

3MOTIS->   redirectionProhibited     3     3       3   X  3      3  N                 3

3                                    3     3       3      3      3                    3

3        PerDomainBilateralInfo      3     3       3      3      3                    3

3          CountryName               3  M  3   M   3   M  3  M   3   N                3

3          AdminDomainName           3  M  3   M   3   G  3  M   3  Y Prot Vio        3

3MOTIS->   PrivateDomainName         3     3       3   G  3      3   N                3

3          BilateralInfo             3  M  3   M   3   M  3  M   3   N                3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDADDDDDDDADDDDDDADDDDDDADDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7E.2  Protocol element comparison of P1, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDBDDDDDDDBDDDDDDBDDDDDDBDDDDDDDDDDDDDDDDDDDD?

3P1 Protocol                         3 NBS 3 A/311 3A/32113 TTC  3   PROBLEM (Y/N)    3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDEDDDDDDDEDDDDDDEDDDDDDEDDDDDDDDDDDDDDDDDDDD4

3        DeliveryReportContent       3     3       3      3      3                    3

3          original MPDUident        3  M  3   M   3   M  3  M   3   N                3

3          intermediate Trace        3 X/G 3   X   3   X  3  X   3  Y Conformance (E) 3

3          UAcontentID               3  G  3   G   3   G  3  G   3   N                3

3          ReportedRecipientInfo     3  M  3   M   3   M  3  M   3   Y TTC 256 max    3

3          returned                  3  H  3   H   3   X  3  X   3  Y Conformance (E) 3

3          billing information       3  X  3   X   3   X  3  X   3   N                3

3                                    3     3       3      3      3                    3

3        ReportedRecipientInfo       3     3       3      3      3                    3

3          recipient ORname          3  M  3   M   3   M  3  M   3   N                3

3          extensionsIdentifier      3  M  3   M   3   M  3  M   3   N                3

3          PerRecipientFlag          3  M  3   M   3   M  3  M   3   N                3

3          LastTraceInformation      3  M  3   M   3   M  3  M   3   N                3

3          intendedRecipient         3  H  3   H   3   H  3  H   3   N                3

3          SupplementaryInfo         3 X/H 3   X   3   X  3  X   3  Y Conformance (E) 3

3MOTIS->   ReassignmentInfo          3     3       3   X  3      3   N                3

3                                    3     3       3      3      3                    3

3MOTIS-> ReassignmentInfo            3     3       3      3      3                    3

3MOTIS->   intendedRecipient         3     3       3   M  3      3   N                3

3MOTIS->   reasonForReassignment     3     3       3   H  3      3   N                3

3                                    3     3       3      3      3                    3

3        LastTraceInformation        3     3       3      3      3                    3

3          arrival                   3  M  3   M   3   M  3  M   3   N                3

3          convertedEncInfoTypes     3  G  3   G   3   H  3  G   3  Y Conformance (E) 3

3          Report                    3  M  3   M   3   M  3  M   3   N                3

3                                    3     3       3      3      3                    3

3        Report                      3     3       3      3      3                    3

3          DeliveredInfo             3  G  3   G   3   G  3 D?   3   N See Note 6     3

3                                    3     3       3      3  CDM 3                    3

3          NonDeliveredInfo          3  G  3   G   3   G  3 DY   3   N                3

3                                    3     3       3      3      3                    3

3        DeliveredInfo               3     3       3      3      3                    3

3          delivery                  3  M  3   M   3   M  3  M   3   N                3

3          TypeofUA                  3 R/H 3   H   3   R  3 M/G  3   N                3

3                                    3     3       3      3      3                    3

3        NonDeliveredInfo            3     3       3      3      3                    3

3          ReasonCode                3  M  3   M   3   M  3  M   3   N                3

3          DiagnosticCode            3  H  3   H   3   H  3  H   3   N                3

3MOTIS->   UAprofileIdentifier       3     3       3   X  3      3   N                3

3                                    3     3       3      3      3                    3

3MOTIS-> UAprofileIdentifier         3     3       3      3      3                    3

3MOTIS->   ContentType               3     3       3   M  3      3   N                3

3MOTIS->   EncodedInfoTypes          3     3       3   M  3      3   N                3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDADDDDDDDADDDDDDADDDDDDADDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7E.2  Protocol element comparison of P1, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDBDDDDDDDBDDDDDDBDDDDDDBDDDDDDDDDDDDDDDDDDDD?

3P1 Protocol                         3 NBS 3 A/311 3A/32113 TTC  3   PROBLEM (Y/N)    3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDEDDDDDDDEDDDDDDEDDDDDDEDDDDDDDDDDDDDDDDDDDD4

3                                    3     3       3      3      3                    3

3        ProbeEnvelope               3     3       3      3      3                    3

3          probe                     3  M  3   M   3   M  3  M   3   N                3

3          originator                3  M  3   M   3   M  3  M   3   N                3

3          ContentType               3  M  3   M   3   M  3  M   3   N                3

3          UAcontentID               3  H  3   H   3   H  3  H   3   N                3

3          originalEncInfoTypes      3  G  3   H   3   H  3  G   3  Y Conformance (E) 3

3          TraceInformation          3  M  3   M   3   M  3  M   3   N                3

3          PerMessageFlag            3  G  3   G   3   G  3  G   3   N                3

3          ContentLength             3  H  3   H   3   H  3  H   3   N                3

3          PerDomainBilatInfo        3  X  3   X   3   X  3  X   3   N                3

3          RecipientInfo             3  M  3   M   3   M  3  M   3   Y TTC 256 max    3

3MOTIS->   InternalTraceInfo         3 M/P 3       3   P  3      3   N                3

3                                    3     3       3      3      3                    3

3        RecipientInfo               3     3       3      3      3                    3

3          RecipientORname           3  M  3   M   3   M  3  M   3    N               3

3          ExtensionIdentifier       3  M  3   M   3   M  3  M   3    N               3

3          PerRecipientFlag          3  M  3   M   3   M  3  M   3    N               3

3          ExplicitConversion        3  X  3   X   3   X  3  X   3    N               3

3MOTIS->   OriginatorReqAlternatRecip3     3       3   X  3      3    N               3

3MOTIS->   ReassignmentInfo          3     3       3   X  3      3    N               3

3                                    3     3       3      3      3                    3

3        PerRecipientFlag            3     3       3      3      3                    3

3          ResponsibilityFlag        3  M  3   M   3   M  3   M  3    N               3

3          ReportRequest             3  M  3   M   3   M  3   M  3    N               3

3          UserReportRequest         3  M  3   M   3   M  3   M  3    N               3

3                                    3     3       3      3      3                    3

3        TraceInformation            3     3       3      3      3                    3

3                                    3     3       3      3      3                    3

3          GlobalDomainIdent         3  M  3   M   3   M  3  M   3    N               3

3          DomainSuppliedInfo        3  M  3   M   3   M  3  M   3    N               3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDADDDDDDDADDDDDDADDDDDDADDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7E.2  Protocol element comparison of P1, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDBDDDDDDDBDDDDDDBDDDDDDBDDDDDDDDDDDDDDDDDDDD?

3P1 Protocol                         3 NBS 3 A/311 3A/32113 TTC  3   PROBLEM (Y/N)    3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDEDDDDDDDEDDDDDDEDDDDDDEDDDDDDDDDDDDDDDDDDDD4

3                                    3     3       3      3      3                    3

3        DomainSuppliedInfo          3     3       3      3      3                    3

3          arrival                   3  M  3   M   3   M  3  M   3    N               3

3          deferred                  3  X  3   X   3   X  3  X   3    N               3

3          action                    3  M  3   M   3   M  3  M   3    N               3

3            (0=relayed)             3  G  3   G   3   G  3      3    N Note:         3

3                                    3     3       3      3      3   Re-routing not   3

3                                    3     3       3      3      3   required.        3

3            (1=rerouted)            3  H  3   H   3   H  3      3    N               3

3MOTIS->     (2=recipientReassigned) 3     3       3   H  3      3    N               3

3          converted                 3  H  3   G   3   H  3  H   3   Y Conformance(US)3

3          previous                  3  H  3   G   3   G  3  X   3   Y Conformance(US)3

3                                    3     3       3      3      3   (Note: G is      3

3                                    3     3       3      3      3   inconsistent with3

3                                    3     3       3      3      3   action (relayed) 3

3                                    3     3       3      3      3   being "H".)      3

3                                    3     3       3      3      3                    3

3        ORname                      3     3       3      3      3                    3

3                                    3     3       3      3      3                    3

3        EncodedInformationTypes     3     3       3      3      3                    3

3          BitString                 3  M  3   M   3   M  3  M   3    N  See Note 3   3

3          G3NonBasicParameters      3  X  3   X   3   X  3  X   3    N               3

3          TeletexNonBasicParams     3  X  3   R   3   X  3  X   3   Y Conformance(US)3

3          PresentationAbilities     3  X  3   X   3   X  3  X   3    N               3

3                                    3     3       3      3      3                    3

3        DeliveryReportMPDU          3  G  3   G   3   M  3  G   3    N               3

3          DeliveryReportEnvelop     3  M  3   M   3   M  3  M   3    N               3

3          DeliveryReportContent     3  M  3   M   3   M  3  M   3    N               3

3                                    3     3       3      3      3                    3

3        DeliveryReportEnvelope      3     3       3      3      3                    3

3          report                    3  M  3   M   3   M  3  M   3    N               3

3          originator ORname         3  M  3   M   3   M  3  M   3    N               3

3          TraceInformation          3  M  3   M   3   M  3  M   3    N               3

3          InternalTraceInfo         3 M/P 3       3   P  3      3    N               3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDADDDDDDDADDDDDDADDDDDDADDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7E.3  Protocol element comparison of P2

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDBDDDDDDDBDDDDDDBDDDDDDBDDDDDDDDDDDDDDDDDDDD?

3P2 Protocol                         3 NBS 3 A/311 3A/32113 TTC  3   PROBLEM (Y/N)    3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDEDDDDDDDEDDDDDDEDDDDDDEDDDDDDDDDDDDDDDDDDDD4

3                                    3     3       3      3      3                    3

3        UAPDU                       3     3       3      3      3                    3

3          IM_UAPDU                  3  G  3   G   3  G   3  G   3   N                3

3          SR_UAPDU                  3  X  3   X   3  X   3  X   3   N                3

3                                    3     3       3      3      3                    3

3        IM_UAPDU                    3     3       3      3      3                    3

3          Heading                   3  M  3   M   3  M   3  M   3   N                3

3          Body                      3  M  3   M   3  M   3  M   3   N                3

3                                    3     3       3      3      3                    3

3        Heading                     3     3       3      3      3                    3

3          IPmessageID               3  M  3   M   3  M   3  M   3   N                3

3          Originator ORname         3  R  3   R   3  R   3 M/G  3   N                3

3          AuthorizingUsers          3  H  3   H   3  H   3  H   3   Y TTC 16 max     3

3          PrimaryRecipients         3  G  3   G   3  G   3  G   3   Y TTC 256 max    3

3          CopyRecipients            3  G  3   G   3  G   3  G   3   Y TTC 256 max    3

3          BlindCopyRecipients       3  H  3   H   3  H   3  H   3   Y TTC 256 max    3

3          InReplyTo                 3  G  3   G   3  G   3  G   3   N                3

3          Obsoletes                 3  H  3   H   3  H   3  H   3   Y TTC 8 max      3

3          CrossReferences           3  H  3   H   3  H   3  H   3   Y TTC 8 max      3

3          Subject                   3  G  3   G   3  G   3  G   3   N                3

3          ExpiryDate                3  H  3   H   3  H   3  H   3   N                3

3          ReplyBy                   3  H  3   H   3  H   3  H   3   N                3

3          ReplyToUsers              3  H  3   H   3  H   3  H   3   Y TTC 32 max     3

3          Importance                3  H  3   H   3  H   3  H   3   N                3

3          Sensitivity               3  H  3   H   3  H   3  H   3   N                3

3          Autoforwarded             3  H  3   H   3  H   3  H   3   N                3

3MOTIS->   CirculationList           3     3       3  X   3      3   N                3

3MOTIS->   ObsoletingTime            3     3       3  X   3      3   N                3

3                                    3     3       3      3      3                    3

3        IPmessageID                 3     3       3      3      3                    3

3          ORname                    3  H  3   H   3  H   3  H   3   N                3

3          PrintableString           3  M  3   M   3  M   3  M   3   N                3

3                                    3     3       3      3      3                    3

3        ORdescriptor                3     3       3      3      3                    3

3          ORname                    3  H  3   H   3  H   3 D?   3   N See Note 6     3

3                                    3     3       3      3  CDM 3                    3

3          FreeFormName              3  H  3   H   3  H   3 DY   3   N                3

3          TelephoneNumber           3  H  3   H   3  H   3  G   3   N                3

3                                    3     3       3      3      3                    3

3        Recipient                   3     3       3      3      3                    3

3          ORdescriptor              3  M  3   M   3  M   3  M   3   N                3

3          ReportRequest             3  X  3   X   3  X   3  X   3   N                3

3          ReplyRequest              3  H  3   H   3  H   3  H   3   N                3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDADDDDDDDADDDDDDADDDDDDADDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7E.3  Protocol element comparison of P2, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDBDDDDDDDBDDDDDDBDDDDDDBDDDDDDDDDDDDDDDDDDDD?

3P2 Protocol                         3 NBS 3 A/311 3A/32113 TTC  3   PROBLEM (Y/N)    3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDEDDDDDDDEDDDDDDEDDDDDDEDDDDDDDDDDDDDDDDDDDD4

3MOTIS-> CirculationList             3     3       3      3      3                    3

3MOTIS->   CirculationMember         3     3       3  X   3      3   N                3

3MOTIS->   checkmark                 3     3       3  M   3      3   N                3

3MOTIS->   membername                3     3       3  M   3      3   N                3

3                                    3     3       3      3      3                    3

3MOTIS-> OBsoletingTime              3     3       3      3      3                    3

3MOTIS->   Time                      3     3       3  H   3      3   N                3

3MOTIS->   IP_MessageID              3     3       3  H   3      3   N                3

3                                    3     3       3      3      3                    3

3        Body                        3     3       3      3      3                    3

3          BodyPart                  3  G  3   M   3  M   3  G   3  Y Conformance (US)3

3                                    3     3       3      3      3                    3

3        SR_UAPDU                    3     3       3      3      3                    3

3          NonReceipt                3  H  3   H   3  H   3 D?   3   N                3

3                                    3     3       3      3  CDM 3                    3

3          Receipt                   3  H  3   H   3  H   3 DY   3   N                3

3          Reported                  3  M  3   M   3  M   3  M   3   N                3

3          ActualRecipient           3  R  3   R   3  R   3  G   3   N                3

3          IntendedRecipient         3  H  3   H   3  H   3  H   3   N                3

3          Converted                 3  X  3   X   3  X   3  G   3   N                3

3MOTIS->   CirculationStatus         3     3       3  X   3      3   N                3

3                                    3     3       3      3      3                    3

3        NonReceiptInformation       3     3       3      3      3                    3

3          Reason                    3  M  3   M   3  M   3  M   3   N                3

3          NonReceiptQualifier       3  H  3   H   3  H   3  H   3   N                3

3          =expired (value)          3  0  3   0   3  0   3  0   3   N                3

3          =obsoleted (value)        3  1  3   1   3  1   3  1   3   N                3

3          =subscriptionTerminated   3  2  3   2   3  2   3  2   3   N                3

3MOTIS->   =timeobsoleted (value)    3     3       3  X   3      3   N                3

3          Comments                  3  H  3   H   3  H   3  X   3   N                3

3          returned                  3  H  3   X   3  X   3  X   3  Y Conformance (E) 3

3                                    3     3       3      3      3                    3

3        ReceiptInformation          3     3       3      3      3                    3

3          Receipt                   3  M  3   M   3  M   3  M   3   N                3

3          TypeOfReceipt             3  H  3   H   3  H   3  G   3   N                3

3          SupplementaryInfo         3  X  3   X   3  X   3  X   3   N                3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDADDDDDDDADDDDDDADDDDDDADDDDDDDDDDDDDDDDDDDDY

(Continued on next page.)

Table 7E.3  Protocol element comparison of P2, continued

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDBDDDDDDDBDDDDDDBDDDDDDBDDDDDDDDDDDDDDDDDDDD?

3P2 Protocol                         3 NBS 3 A/311 3A/32113 TTC  3   PROBLEM (Y/N)    3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDEDDDDDDDEDDDDDDEDDDDDDEDDDDDDDDDDDDDDDDDDDD4

3                                    3     3       3      3      3                    3

3        BODYPART SUPPORT            3     3       3      3      3                    3

3                                    3     3       3      3      3                    3

3        o IA5 Text                  3  G  3   G   3  G   3      3   N See Note 7     3

3        o TLX                       3  X  3   X   3  X   3      3   N                3

3        o Voice                     3  X  3   X   3  X   3      3   N                3

3        o G3FAX                     3  X  3   X   3  X   3      3   N                3

3        o TIFO                      3  X  3   X   3  X   3      3   N                3

3        o TTX                       3  X  3  X/H  3  X   3      3Y Conf(US)See Note 23

3        o VideoTex                  3  X  3   X   3  X   3      3   N                3

3        o NationallyDefined         3  X  3   X   3  X   3      3   N                3

3        o Encrypted                 3  X  3   X   3  X   3      3   N                3

3        o ForwardedIPmessage        3  H  3   H   3  H   3      3   N                3

3        o SFD                       3  X  3   X   3  X   3      3   N                3

3        o TIFI                      3  X  3   X   3  X   3      3   N                3

3                                    3     3       3      3      3                    3

3MOTIS-> o ODA                       3     3       3  X   3      3   N                3

3MOTIS-> o ISO6937 Text              3     3       3  H   3      3   N                3

3                                    3     3       3      3      3                    3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDADDDDDDDADDDDDDADDDDDDADDDDDDDDDDDDDDDDDDDDY

Note 1: It should be noted that the A/311 profile states:  For routing

all ADMDs should support all Form 1 Variants of O/R Name.  All

PRMDs should support at least Form 1, Variant 1 form of OR Name.

Note 2: It should also be noted that the A/311 profile requires that all

ADMDs should support the reception of Teletex body parts for

delivery to their own UAEs.

Note 3: An A/3211 implementation may generate MOTIS encoded information

types.  See 7.6.11.

Note 4: Only Form 1 Variant 1 of O/Rname shown for TTC, but TTC defines

other forms and variants.  Form 1 Variant 1 recommended for PRMDs

and ADMDs, Form 1 Variant 2 also recommended for ADMDs.

Note 5: DDA's can be used to specify recipients in any Japanese domains

other than TTC.  Assignment of DDAs for UAs within TTC domains is

not recommended.

Note 6: One of [DeliveredInfo/NonDeliveredInfo] must be present.  TTC

encodes this as shown.  Other profiles represent this by

classifying both protocol elements as generatable.  A similar

situation exists with the P2 ORdescriptor.

Note 7: TTC is expected to support IA5 for some international MHS

communications.

7.16 APPENDIX F:    INTERWORKING WARNINGS

ADMD name is to be encoded as a single space when configurations with

no ADMD's are  present.  It should be noted that this may change in

January 1988 so that the ADMD name is encoded as a zero length element

in such cases.

The NBS agreements allow implementation to generate MPDUs with no

body parts.  Such MPDUs will be rejected by European-conformant

systems.  (Note this situation may change in January 1988)

In order to optimize the number of recipients you can read and reply

to, it is advisable to be able to generate all standard O/R name

attributes.

8. DIRECTORY SERVICES PROTOCOLS

8.1  INTRODUCTION

This is an Implementation Agreement developed by the Implementor's

Workshop sponsored by the U.S. National Bureau of Standards to promote

the useful exchange of data between devices manufactured by different

vendors.  This agreement is based on and employs protocols developed

in accord with the OSI Reference Model.  While this agreement

introduces no new protocols, it eliminates ambiguities in

interpretations.

This is an Implementation Agreement for Directories based on the ISO

documents cited in the Reference Section 11.1 (hereafter referenced as

Directory documents).  Versions of this document will stay consistent

with the latest drafts of those Directory Documents.  Figure 8.1

displays the structure of this Implementation Agreement.  References

to corresponding CCITT documents are included for information.

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3    Directory Access Protocol     3    Directory System Protocol     3

3              (DAP)               3              (DSP)               3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3              Remote Operations Services and Protocols               3

3            (CCITT X.219 and X.229/ISO 9072/1 and 9072/2)            3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD4

3             Association Control Services and Protocols              3

3             (CCITT X.217 and X.227/ISO 8649 and 8650)               3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Figure 8.1  Structure of this Implementation Agreement

The Directory User Agents (DUAs) and Directory System Agents (DSAs)

provide access to the Directory on behalf of humans and applications

such as Message Handling and File Transfer, Access, and Management.

See the Scope and Field of Application section for more information on

the model used in Directories.

This document covers both the Directory Access Protocol (DAP) and the

Directory System Protocol(DSP) defined in the Directory documents.  A

good working knowledge of the Directory documents is assumed by this

chapter.  All terminology and abbreviations used but not defined in

this text may be found in those documents.

8.2  SCOPE AND FIELD OF APPLICATION

Centralized and distributed directories can both be accommodated in

this Agreement by the appropriate choice of protocols and pragmatic

constraints from those specified.  Figure 8.2 illustrates a

centralized directory and Figure 8.3 illustrates a distributed

directory.

This agreement does not cover interaction between co-located entities,

such as a co-resident DUA and DSA.  It also does not specify the

interface  between a user (person or application) and a DUA.

Bilateral agreements between a DUA and DSA or DSA and DSA may be

implemented in addition to the requirements stated in this document.

Conformance to this agreement requires the ability to interact

without the use of bilateral agreements other than those required in

the Directory documents.

The logical structure of the Directory Information Base (DIB) is

described in the Directory documents.  The manner in which a local

portion of the DIB is organized and accessed by its DSA is not in the

scope of this agreement.

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                           3

3                                   00000000000000000000000 3

3                                   0   The Directory     0 3

3                                   0                     0 3

3                                   0    ZDDDDDDDDDDD?    0 3

3 ZDDDDDDDDDDD?     ZDDDDDDDDDD?    0    3           3    0 3

3 3   USER    3<DDD>3   DUA    3<DDD0DDD>3           3    0 3

3 @DDDDDDDDDDDY     @DDDDDDDDDDY    0    3    DSA    3    0 3

3 ZDDDDDDDDDDD?     ZDDDDDDDDDD?    0    3           3    0 3

3 3   USER    3<DDD>3   DUA    3<DDD0DDD>3           3    0 3

3 @DDDDDDDDDDDY     @DDDDDDDDDDY    0    @DDDDDDDDDDDY    0 3

3                                   0                     0 3

3                                   0                     0 3

3                                   00000000000000000000000 3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Figure 8.2  Centralized Directory Model

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                                     3

3                              ZDDDDDDD?                              3

3                              3 User  3                              3

3                              @DDDBDDDY                              3

3                                  3                                  3

3                              ZDDDADDD?                              3

3                              3  DUA  3                              3

3                              @DDDBDDDY                              3

3                                  3                                  3

3ZDDDDDD?  ZDDDDDDD?0000000000000000000000000000000                   3

33 User CDD4  DUA  30The Directory 3              0                   3

3@DDDDDDY  @DDDDDDDY0              3              0                   3

3                   0          ZDDDADDD?          0                   3

3                   0          3  DSA  3          0                   3

3                   0          @DDDDDDDY          0                   3

3                   0ZDDDDDDD?           ZDDDDDDD?0                   3

3                   03  DSA  3           3  DSA  30                   3

3                   0@DDDDDDDY           @DDDDDDDY0                   3

3ZDDDDDD?  ZDDDDDDD?0          ZDDDDDDD?          0ZDDDDDDD?  ZDDDDDD?3

33 User CDD4  DUA  30          3  DSA  3          03  DUA  CDD4 User 33

3@DDDDDDY  @DDDDDDDY0          @DDDDDDDY          0@DDDDDDDY  @DDDDDDY3

3                   0000000000000000000000000000000                   3

3                                                                     3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Figure 8.3  Distributed Directory Model

8.3  STATUS

This version was completed on December 18, 1987.

8.4  Use of Directories

Given the rapid multiplication and expansion of OSI applications,

telecommunication systems and services, there is growing need for

users of, as well as the applications themselves, to communicate with

each other.  In order to facilitate their communications, a Directory

protocol, as referenced in these agreements, has been tailored to meet

their respective needs.

In one instance, Directories will be used as a service to provide

humans, in an on-line fashion, rapid and easy retrieval of information

useful for determining what telecommunications services are available,

and/or how to access, and address their correspondents.  Further,

service providers offering such a Public Directory may also use this

service internally with other various telecommunications services

(e.g., MHS) for the proper addressing of calls or messages.  Likewise,

this does not preclude the usage of these agreements to similarly

generate a privately operated Directory that supports both human and

application information exchanges.

In another instance, Directories, will be used as a service by

computer applications without direct human involvement.  One important

service is to provide Presentation Address resolution for named

objects, on behalf of OSI applications.  The directory may be used by

applications to search for objects (i.e., Application Entities),

without direct human involvement, by the use of the "search" or

"list" operations.

To support the many possible usages, the Directory is a general

purpose system.  It is capable of storing data of many different

forms as attributes within entries, and is also capable of supporting

simple or complex hierarchical structures, with variations in

structure possibly occurring between one part of the Directory and

another.

Compliant DSA implementations should safeguard this generality, where

possible, by placing the minimum of restrictions in "hard-wired"

form.  The Directory permits the imposition of rules by means of the

Directory Schema (Section 8.6 below);  but the Directory Schema

itself should be capable of alteration by Directory management.

8.5  Directory ASEs, Application Contexts, and Ports

The following section is included for tutorial purposes.  The

functionality of the Directory AEs (DUAs and DSAs) is defined by a set

of ASEs, each Directory ASE specifying a set of Directory operations.

The interaction between these AEs is described in terms of their use

of ASEs.  This specific combination of a set of ASEs and the rules for

their usage defines an application context.

Thus, each Directory application context defines the operations needed

by two communicating Directory entities.

Access to the services provided by an application context is through

one or more directory ports.  The point of access is called an Access

Point (see Figure 8.4).  Each access point corresponds to a

particular combination of port types.

The following ASEs are described in the Directory Document:

o    Directory Read ASE

o    Directory Chained Read ASE

o    Directory Search ASE

o    Directory Chained Search ASE

o    Directory Modify ASE

o    Directory Chained Modify ASE

ROSE and ACSE also form part of the Directory Application Contexts.

The following Application Contexts are described in the Directory

Document:

o    Directory Access Application Context

o    Directory System Application Context

The following Ports are described in the Directory Document:

o    Read Port

o    Modify Port

o    Search Port

o    Chained Read Port

o    Chained Search Port

o    Chained Modify Port

The ports cited above are to be specified for a particular Access

Point to the Directory as illustrated by Figure 8.4.

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3               Access Point DDDDDDDDD?                    3

3                                     3                    3

3 ZDDDDDDDDDDDDDDDD?                  3ZDDDDDDDDDDDDDDD?   3

3 3                3                  33               3   3

3 3 Directory ZDDDDADDDDD?            33               3   3

3 3 user      3          3            #3    The        3   3

3 3           3   DUA    CDDDDDDDDDDDDD4    Directory  3   3

3 3           3          3             3               3   3

3 3           @DDDDBDDDDDY             3               3   3

3 3                3                   3               3   3

3 @DDDDDDDDDDDDDDDDY                   @DDDDDDDDDDDDDDDY   3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Figure 8.4  Access to the Directory

8.6  Schemas

There are four (4) major topics that relate to schemas:

8.6.1     Maintenance of Structures and Naming Rules

DSAs shall be capable of supporting the object classes and the

associated structure and naming rules defined in the Directory

documents, part 7, annex B, in the sense that DITs may be

constructed and within these rules shall be supported.

8.6.2     Maintenance of object classes and subclasses

DSAs shall be able to support the storage and use of the

following object classes defined in the Directory documents part

7, annex B:

top                      alias

country                  application entity

locality                 organizational-person

organization             residential-person

organizational unit      application-process

DSA

Use of the modification and local extension mechanisms provided

by registered and unregistered object classes is optional.

8.6.3     Maintenance of Attribute Types

DSAs shall be able to support the storage and use of attribute

type information, as defined in Part 6, including their use in

naming and access to entries;  they shall also support the

definition of new attribute types, making use of pre-existing

attribute syntaxes.

8.6.4     Maintenance of Attribute Syntaxes

Suggested methods for the interpretation of selected Attribute

Syntaxes are defined in Appendix B.

8.7  Classification of Support for Attribute Types

This section classifies directory support for selected attribute type

specified in the Directory documents.

Classification of support for selected attribute types is either

mandatory or optional.

8.7.1     Mandatory Support

The Directory must be able to support these Attribute Types:

Aliased Object Name      Postal Address

Business Category        Postal Code

Common Name              Preferred Delivery Mode

Country Name             Presentation Address

Locality Name            Role Occupant

ISDN Address             See Also

Member                   Serial Number

Object Class             State or Province Name

Organization Name        Street Address

Organizational Unit Name Supported Application Context

Owner                    Surname

Post Office Box          Telephone Number

Title

User Password

X.121 Address

Note:     Support of these Attribute Types implies full support

of the relevant Attribute Syntaxes.

8.7.2     Optional Support

Directory support of these attribute types is considered

optional:

Authority Revocation List

CA Certificate

Certificate Revocation List

Description

Destination Indicator

Facsimile Telephone Number

Knowledge Information

Non-Basic Parameters

Physical Delivery Office Name

Registered Address

Search Guide

Teletex Terminal Identifier

Telex Number

User Certificate

Note:     DSAs should consider initial support of the Attribute

Syntax relevant to any Attribute Type for which future

support is planned, in addition to those required for

mandatory Attribute Types.

8.8  Introduction to Pragmatic Constraints

The following sections of this document define the pragmatic

constraints to which a conformant implementation must adhere in

addition to those specified in the Directory documents.  The pragmatic

constraints are divided into two areas.  The first includes those

aspects of pragmatic constraints which apply to the scope of service.

The second includes those aspects of pragmatic constraints which are

specific to particular attribute types.

8.9  General Constraints

8.9.1     Character Sets

It is a requirement to support all character sets and other name

forms defined in the Directory Documents Part 6.  Those

character sets include:

o    T.61

o    PrintableString

o    NumericString

8.9.2     APDU Size Considerations

In the process of chaining requests it is possible that a

chaining DSA may receive invoke or return APDUs that exceed its

capacity:

ZDDDDDDDDD?       2k      ZDDDDDDDDD?      2k        ZDDDDDDDDD?

3         CDDDDDDDDDDDDDD>3         CDDDDDDDDDDDDDDD>3         3

3   DUA   3               3   DSA   3                3   DSA   3

3         3<DDDDDDDDDDDDDD4       ZDE<DDDDDDDDDDDDDDD4         3

@DDDDDDDDDY      37K      @DDDDDDDEDY       33K      @DDDDDDDDDY

DSA may pass on          #    DSA may discard

Figure 8.5  APDU Exchange

It is a minimum requirement that invoke and return result APDUs

must be accepted unless they exceed 32767 octets in size;  in

this case they may be discarded as illustrated in the right side

of figure 8.5, and an "unwillingToPerform" error reporting

service shall be used.

8.9.3     Service Control (SC) Considerations

This agreement recognizes that DUAs may automatically supply

defaults for any SC parameter.  The choice of default values

selected (if any) is seen to be a matter of local policy and

consumer needs.

8.9.4     Priority Service Control

Priority is specified as a service control argument in the

Directory documents.  The following statements represent a

clarification of the semantics that may be used by a DSA in

interpreting and operating on this parameter.

The logical model in Figure 8.6 may be considered as an example

by DSAs that implement this Service Control.

o    The DSA maintains three logical queues corresponding to

the three priority levels.

o    The DSA Scheduler is separate and distinct from any

scheduling function provided by the underlying

operating system or control program services.

o    The DSA Scheduler presents jobs to the Underlying

Operating Services for execution and always presents

jobs of a higher priority before those of a lower

priority.

o    The DSA Scheduler will not preempt a request once it

has been passed to the underlying operating system

service.

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                           3

3        ZDDDDDDDDDDDDDDDD?      ZDDDDDDDDDDDDD?            3

3   ZDDD>3     High       CDDDDD>3             #            3

3   3    @DDDDDDDDDDDDDDDDY      3          ZDDDDDDDDDDD?   3

3   3    ZDDDDDDDDDDDDDDDD?      3          3 DSA       3   3

3   CDDD>3     Medium     CDDDDD>3          3 Scheduler 3   3

3   3    @DDDDDDDDDDDDDDDDY      3          @DDBDDDDDDDDY   3

3   3    ZDDDDDDDDDDDDDDDD?      3             3            3

3   CDDD>3     Low        CDDDDD>3             3            3

3   3    @DDDDDDDDDDDDDDDDY                    #            3

3ZDDADDDDDDDDDDDDDDDD?                   ZDDDDDDDDDDDDDDD?  3

33 Underlying        3                   3 Underlying OS 3  3

33 Protocol Services 3                   3 Services      3  3

3@DDDDDDDDDDDDDDDDDDDY                   @DDDDDDDDDDDDDDDY  3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Figure 8.6  Logical DSA Application Environment

8.10 Constraints on Operations

There are no overall constraints upon service arguments or results

except those implied in section 8.9.2 of this document.

8.10.1    Filters

It is required that DSAs, at a minimum, support 8 nested

"Filter" parameters, and a total limit of 32 Filter Items.  If

these limits are exceeded, the recipient of that SearchArgument

may return the ServiceProblem "unwillingToPerform".

8.10.2    Errors

There are no constraints upon any Error service except the APDU

size limit as defined in section 8.9.2.

8.11 Constraints on Attribute Types

This section defines the pragmatic constraints specific to particular

attribute types.

8.11.1    Attribute Values

This section describes the pragmatic constraints for attribute

values.  Each constraint is given in terms of a Length

Constraint.  The Length Constraint for a given attribute value is

the number of units which a sending entity must not exceed and

which a receiving entity must accept and process.  A sending

entity need not be capable of sending attribute values as large

as the Length Constraints.

Use of Pragmatic Constraints for Strings

The Length Constraint for strings is expressed as the number of

allowable characters and the number of allowable octets.  When

using the Printable String ASN.1 data type, the number of octets

equals the number of characters.  When using the T.61 ASN.1 data

type, the number of octets is twice the number of characters.

This is because some T.61 characters occupy two octets per

character.

Attribute Types

Table 8.1 specifies the pragmatic constraints for selected

attribute types specified in the Directory documents; many of

these constraints also appear and are the same in the CCITT

version of the Directory documents.

Alphabets

T.61 Strings used as attribute values shall only encode graphic

characters and spaces.  (They must not contain formatting

characters (such as subscript) or other control characters.

Table 8.1  Pragmatic Constraints for Selected Attributes.

ZDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDBDDDDDDDDBDDDDDDDDDBDDDDDDDDDBDDDDDDDDDDDDDDD?

3 Attribute Type   3   Content   3 Number 3  Number 3  Status 3  Notes        3

3                  3             3 of     3  of     3         3               3

3                  3             3 Print- 3  Allow- 3         3               3

3                  3             3 able   3  able   3         3               3

3                  3             3 Char-  3  Octets 3         3               3

3                  3             3 acters 3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Object Class     3   Object    3        3   256   3         3               3

3                  3   Identifier3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Common Name      3   T.61 or   3   64   3   128   3         3  X.500        3

3                  3   Printable 3        3         3         3               3

3                  3   String    3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Serial Number    3   Printable 3   64   3    64   3         3  X.500        3

3                  3   String    3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Country Name     3   Printable 3        3         3         3  ISO 3166     3

3                  3   String    3   2    3    2    3         3  String only  3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Locality         3   T.61 or   3  128   3   256   3         3  X.500        3

3                  3   Printable 3        3         3         3               3

3                  3   String    3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Postal           3   T.61 or   3 30 x 6 3  60 x 6 3         3  Note 4       3

3 Address          3   Printable 3        3         3         3  X.500 & UPU  3

3                  3   String    3        3         3         3               3

3                  3             3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Postal Code      3   T.61 or   3   40   3    80   3         3  UPU          3

3                  3   Printable 3        3         3         3  X.500        3

3                  3   String    3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Organization     3   T.61 or   3   64   3   128   3         3  X.500        3

3 Name             3   Printable 3        3         3         3               3

3                  3   String    3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Organizational   3   T.61 or   3   64   3   128   3         3  X.500        3

3 Unit Name        3   Printable 3        3         3         3               3

3                  3   String    3        3         3         3               3

@DDDDDDDDDDDDDDDDDDADDDDDDDDDDDDDADDDDDDDDADDDDDDDDDADDDDDDDDDADDDDDDDDDDDDDDDY

Table 8.1  Pragmatic Constraints for Selected Attributes.  (continued)

ZDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDBDDDDDDDDBDDDDDDDDDBDDDDDDDDDBDDDDDDDDDDDDDDD?

3 Attribute Type   3   Content   3 Number 3  Number 3  Status 3  Notes        3

3                  3             3 of     3  of     3         3               3

3                  3             3 Print- 3  Allow- 3         3               3

3                  3             3 able   3  able   3         3               3

3                  3             3 Char-  3  Octets 3         3               3

3                  3             3 acters 3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Title            3   T.61 or   3   64   3   128   3         3  X.500        3

3                  3   Printable 3        3         3         3               3

3                  3   String    3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Description      3   T.61 or   3 1024   3  2048   3         3  X.500 About  3

3                  3   Printable 3        3         3         3  1 Screen full3

3                  3   String    3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Search Guide     3             3        3         3         3  ffs          3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Business Category3   T.61 or   3   64   3   128   3         3               3

3                  3   Printable 3        3         3         3               3

3                  3   String    3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Facsimile Tele-  3   Facsimile 3        3   ffs   3         3  CCITT E.163  3

3 phone Number     3   Telephone 3        3         3         3  X.500        3

3                  3   Number    3        3         3         3  Includes G3  3

3                  3             3        3         3         3  Non Basic    3

3                  3             3        3         3         3  Parameters   3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 ISDN Address     3   Numeric   3  n/a   3  16+40  3         3  CCITT        3

3                  3   String    3        3         3         3  E. 164 X.500 3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Presentation     3   Presen-   3  n/a   3   224   3         3Note 2,ISO 74983

3 Address          3   tation    3        3         3         3X.500, & X.200 3

3                  3   Address   3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Telephone        3   Printable 3   32   3    32   3         3  CCITT E.163  3

3 Number           3   String    3        3         3         3  X.500        3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Teletex Terminal 3   Teletex   3        3    ffs  3         3  CCITT F.200  3

3 Identifier       3   Terminal  3        3         3         3  X.500        3

3                  3   Identifier3        3         3         3  Includes     3

3                  3             3        3         3         3  Teletex Non  3

3                  3             3        3         3         3  Basic        3

3                  3             3        3         3         3  Parameters   3

@DDDDDDDDDDDDDDDDDDADDDDDDDDDDDDDADDDDDDDDADDDDDDDDDADDDDDDDDDADDDDDDDDDDDDDDDY

Table 8.1  Pragmatic Constraints For Selected Attributes. (continued)

ZDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDBDDDDDDDDBDDDDDDDDDBDDDDDDDDDBDDDDDDDDDDDDDDD?

3 Attribute Type   3   Content   3 Number 3  Number 3  Status 3  Notes        3

3                  3             3 of     3  of     3         3               3

3                  3             3 Print- 3  Allow- 3         3               3

3                  3             3 able   3  able   3         3               3

3                  3             3 Char-  3  Octets 3         3               3

3                  3             3 acters 3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3                  3   Telex     3        3         3         3  CCITT X.500  3

3 Telex Number     3   Number    3        3   ffs   3         3  X.121        3

3                  3             3        3         3         3  Contains     3

3                  3             3        3         3         3  telex-number,3

3                  3             3        3         3         3  country code,3

3                  3             3        3         3         3  answer back  3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 X.121 Address    3   Numeric   3   15   3   15    3         3  CCITT X.121  3

3                  3             3        3         3         3  X.500        3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Member           3   Distin-   3        3         3         3  Note 3       3

3                  3   guished   3        3         3         3               3

3                  3   Name      3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Owner            3   Distin-   3        3         3         3  Note 3       3

3                  3   guished   3        3         3         3               3

3                  3   Name      3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Role             3   Distin-   3        3         3         3  Note 3       3

3 Occupant         3   guished   3        3         3         3               3

3                  3   Name      3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 See Also         3   Distin-   3        3         3         3  Note 3       3

3                  3   guished   3        3         3         3               3

3                  3   Name      3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 User             3   Octet     3  n/a   3   128   3         3X.500 Allow    3

3 Password         3   String    3        3         3         3long passwords.3

3                  3             3        3         3         3Machine        3

3                  3             3        3         3         3Generated      3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Aliased Object   3   Distin-   3        3         3         3  Note 3       3

3 Name             3   guished   3        3         3         3               3

3                  3   Name      3        3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3 Knowledge        3   T.61 or   3  1024  3  2048   3         3  About 1      3

3 Information      3   Printable 3        3         3         3  screen full. 3

3                  3   String    3        3         3         3  ffs          3

@DDDDDDDDDDDDDDDDDDADDDDDDDDDDDDDADDDDDDDDADDDDDDDDDADDDDDDDDDADDDDDDDDDDDDDDDY

Table 8.1  Pragmatic Constraints For Selected Attributes. (continued)

ZDDDDDDDDDDDDDDDDDDBDDDDDDDDDDDDDBDDDDDDDDBDDDDDDDDDBDDDDDDDDDBDDDDDDDDDDDDDDD?

3 Attribute Type   3   Content   3 Number 3  Number 3  Status 3  Notes        3

3                  3             3 of     3  of     3         3               3

3                  3             3 Print- 3  Allow- 3         3               3

3                  3             3 able   3  able   3         3               3

3                  3             3 Char-  3  Octets 3         3               3

3                  3             3 acters 3         3         3               3

CDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDEDDDDDDDDEDDDDDDDDDEDDDDDDDDDEDDDDDDDDDDDDDDD4

3                  3             3        3         3         3               3

3 Street Address   3   T.61 or   3        3         3         3UPU. Component 3

3                  3   Printable 3        3         3         3of Postal      3

3                  3   String    3        3         3         3Address        3

@DDDDDDDDDDDDDDDDDDADDDDDDDDDDDDDADDDDDDDDADDDDDDDDDADDDDDDDDDADDDDDDDDDDDDDDDY

Notes:

1.   The pragmatic constraints of these parameters are defined in

other standards.  We will accommodate these values in our

pragmatic constraints.

2.   Presentation address is composed of "X" NSAP addresses, and

three selectors, (20X + 32 + 16 + 16 ), e.g. if X = 1, this would

be 84.  These numbers are based on the most recent implementor's

agreements.  With 8 NSAP addresses this value is 224.

3.   Pragmatic constraints are only applied to the individual

components of DistinguishedName as defined in the Directory

Documents Part 2.  Not all components of a DN will necessarily be

understood by an implementation.

4.   Implementors should be aware that constraints on Postal Address

may not be sufficient for some markets.

8.12 Conformance

The following sections will describe various aspects of Directory

conformance.  It should be noted that conformance to the various ASEs

and conformance to the Authentication Framework are viewed as separate

issues and are presented in that context.

8.12.1    DUA Conformance

Conformance requirements for DUAs are adequately specified in the

Directory documents, Part 5, section 9.1.  It should be noted

that DUA conformance can only be demonstrated by exercising the

interface that it provides to its user.

It is recognized that DUAs will be widely differing in nature:

o    Some are intended to support human users, some application

users

o    Particular DUAs may not support particular operations

because the application that they support has no

requirement;  others will be general purpose, and will

support all operations.

o    Some DUAs will have a fixed view of the Directory content

and structure, reflecting the usage of the Directory by a

particular application;  others will have a more flexible

view which can be adapted to new usages.

o    Some DUAs will provide automatic referral services with

automatic establishment and release of associations;  others

will place the burden on the user.

o    Some DUAs will provide a variety of authentication means;

others will support simple authentication only

o    Some DUAs will handle operations synchronously; others will

have the capability of maintaining several identifiable

dialogues with the Directory at one time.

No general implementation agreements are spelled out in respect

of these possibilities.

8.12.2    DSA Conformance

Basic conformance requirements for a DSA are defined in the

Directory documents, Part 5, section 9.2.

Centralized

A centralized DSA is defined as one that contains its entire

relevant DIT;  it follows that it will not make use of the DSP or

generate referral responses.  Since this model only contains a

single DSA it is not subject to DSA interworking issues and will

always provide a consistent level of service and results.  A

centralized DSA must be fully "protocol" conformant to the DAP.

Cooperating

In a distributed directory, responsibility for various portions

of the DIT may be "distributed" among multiple DSAs.  On a per

operation basis we define a DSA to be "holding" when it is

responsible for the fragment of the DIB in which a given entry

will appear if it exists;  we define a DSA to be "propagating"

when it is unable to complete the name resolution process.  All

DSAs must be capable of acting as a "holder" and a"propagator."

8.12.3    Directory Systems Conformance Classes

A DSA implementation shall satisfy the conformance requirements

as defined in the Directory documents, part 5, section 9.2, and

shall support the "Versions" argument of "Bind".

Additionally, an implementation conformant to these agreements

shall state which of the following conformance classes it

implements:

Conformance Class 0:  Centralized Directory

A DSA implementation conformant to this class only supports the

DirectoryAccessAC.  It must implement all operations in the Read

and Modify ASEs.  Optionally, it may implement the operations in

the Search ASE.

A DSA that does not support the Search and List operations must

reply to such operations with an unwillingToPerform service

error.

Conformance Class 1:  Distributed Directories

A DSA implementation conformant to this class must implement all

the operations in the ASEs that are part of the Application

context for which it claims conformance.  It must support the

DirectoryAccessAC  and it may optionally support the

DirectorySystemAC.

8.12.4    Authentication Conformance

A Directory System may choose to implement various levels of

authentication (Directory documents, Part 8).  We define the

following four (4) levels of authentication in the DS:

o    No authentication at all; (None)

o    Identification of the remote peer entity only, without

verification; (Weak)

o    Simple authentication:  verified identification with or

without a password.  Intended to make masquerading

difficult; and

o    Strong authentication: identification with verification

using cryptographic techniques intended to make

masquerading, in practical terms, nearly impossible.

The Authentication Framework document describes the specific goal

of each authentication level;  we have listed several practical

uses of the various levels:

NONE      No authentication may be required for associations with

a DSA containing public information:  DSAs operating on

a private network in a controlled environment may

implicitly trust all connections and have no

requirement for authentication.

WEAK      authentication may be desired to maintain access

statistics or in a private network where the initiator

is implicitly trusted and there is no need to incur

the additional overhead of more sophisticated

authentication methods.

SIMPLE    authentication may be necessary in situations where

strong authentication is not practical, (i.e.:

international connections, no knowledge of algorithms

in use, etc).

STRONG    authentication will be required for secure

environments.

8.12.5    Authentication Conformance Classes

We define the following two (2) conformance classes for the

support of authentication:

o    None, Weak,

o    None, Weak, and Simple (unprotected)

Consideration of protected simple and strong authentication is

not part of this agreement.

8.13 Distributed Operations

The following requirements apply to DSAs supporting distributed

operations:

1.   DSAs must support the generation of referrals.

2.   DSAs may additionally support chaining.  DSAs that only support

chaining (i.e. no referrals) are not allowed.

3.   DSAs supporting authentication (e.g. simple authentication by

name and password) must be able to invoke DSP operations to carry

out authentication by reference to other DSAs.  Thus all such

DSAs  must support the DSP protocol.

8.13.1    Referrals and Chaining

It is recommended that a DSA which has chained a request act upon

any referrals it receives rather that returning them to the

requestor if the "PreferChaining" service control is present.

8.14 Underlying Services

8.14.1    ROSE

It should be noted that support of "abandon" implies support of

operation class 2.

8.14.2    Session

All directory implementations are required to support Session

Version 2.

8.15 Access Control

Guidelines relation to access control can be found in annex F of the

Directory documents, part 2.

8.16 Authentication

Simple Authentication, as defined in Part 8 of the Directory

documents, encompasses both Weak and Simple Authentication, as called

for by section 8.12.  (This shall be taken to imply unprotected simple

authentication.)

A DSA that implements Simple Authentication as called for in section

8.12, will check a user password, (if one is supplied) by means of a

compare operation on the user's entry or otherwise; if no user

password is supplied, the DSA will validate the presence of an entry

for the user, by a read operation or otherwise; the authentication

will fail if the password is incorrect, or if the user's entry does

not exist.  DSP must be used when the user's entry is not a local

entry; this form of authentication is therefore not normally

appropriate for DAP only DSAs.

A DSA that implements Weak Authentication will accept simple

credentials without validating them.

8.17 Test Considerations

This section outlines some items that implementors may wish to

consider in terms of testing expectations; additionally, future

conformance testers may wish to consider these items when developing

tests.

8.17.1    Major elements of Architecture

One important aspect of testing is to confirm the correct

behavior of DSAs and DUAs in respect of major elements of the

directory architecture.

Such major elements include:

o    Conformance Statement

o    Distinguished names (e.g., name resolution, equivalence of

various forms)

o    Entries and Attributes (e.g., accessibility by operations,

compliance with rules)

o    Handling of distributed operations (e.g.,naming contexts

and knowledge)

o    Schemas

-    Structure rules (e.g., storage and maintenance of

structure and of naming)

-    Object classes and sub-classes (e.g., storage and

extension of rules for object attributes)

-    Attribute types (e.g., storage and maintenance of

syntax classes and rules for multi or single valued

attributes)

-    Attribute syntax (e.g. maintenance and support for

attribute value testing and matching, to specification

for a defined set of attribute types)

o    Operations

-    all operations

-    correct function

-    correct result

-    correct responses

o    Aliases (e.g., correct resolution, error responses)

o    Authentication and Access Control (e.g., limitation of

modify access)

o    ROSE (e.g., correct handling of invokes, results, rejects,

and invoke ids)

o    ACSE (e.g., association establishment / refusal for invalid

application contexts, etc.)

8.17.2    Search Operation

Testing of support for filter items should be reasonable.  It is

not expected that DSAs will be able to handle worst case testing

in this area.

8.18 Errors

This section provides clarification of the semantics of various

operation errors and implementation guidelines on their usage.

8.18.1    Permanent vs. Temporary Service Errors

The usage of the Service Errors busy, unavailable and

unwillingToPerform requires some clarification:

The error busy is particularly transient.  It is returned when

one or more of the Directory's internal resources are being used

to their capacity and, hence, the requested operation cannot, for

the moment, be performed.  The Directory should be able to

recover from this type of resource depletion after a short while.

The error unavailable is also temporary but somewhat less

transient.  It indicates that the Directory (or some part of it)

is currently unavailable and may continue to be unavailable for

a reasonably long period of time.  for example, this error is

returned when a given DSA is functionally disabled, or when a

specific part of the DIB is undergoing reconfiguration.

The error unwillingToPerform has a permanent connotation.  It

indicates that the Directory cannot perform the requested

operation because it would require resources beyond its capacity.

For example, this error may be returned by a DSA if, satisfying a

requested would result in the generation of an APDU in excess of

32767 octets.

8.19 APPENDIX A     Definitions

Any definitions not found in this appendix can be found in the

Directory Documents.

o    Holding

o    Propagating

o    Centralized

o    Cooperating

8.20 APPENDIX B     Attributes and Object Classes

The Contents of this section are incomplete.  Additional work

describing algorithms related to schemas and special attribute types

should be placed here.

The attribute types defined in the Directory documents, Part 6, and

listed in table 8.1 have requirements for underlying algorithms which

relate:

1.   To the checking of attribute values from the viewpoint of

syntactical correctness and compliance with pragmatic

constraints.

2.   To the comparison of attribute values for the purpose of

comparison (for equality, for matching substrings, and as a

preliminary for determining relative ordering)

Sections B.1 and B.2 give brief characteristics of the checking and

comparison algorithms, respectively.  These characteristics are not

currently defined explicitly in the Directory documents.  Section B.3

summarizes their applicability to the attribute syntaxes defined by

the Directory documents.

It should be noted that determining relative ordering requires the

application of a further algorithm appropriate to the type of value

which requires ordering.

B.1  Checking Algorithms

Note.  A particular attribute type in some cases defines more than one

alternative attribute syntax.  In this case, an attribute value is

satisfied if it satisfies at least one checking algorithm, as listed

below; maximum and minimum sizes may be specified for the attribute-

syntax or the attribute type.

B.1.1     T.61 String Check

Checks that the value has the type code for T.61 string, that it

encodes a sequence of valid T.61 characters, and that it complies with

a maximum character length, if this is specified.  Only graphic

characters (including spaces) are permitted.

B.1.2     Printable String Check

Checks that the value has the type code for Printable String, and that

it encodes as a sequence of valid printable string characters, and

that it complies with a maximum character length, if this is

specified.

B.1.3     Numeric String Check

Checks that the value has the type code for numeric string, that it

encodes a sequence of valid numeric string characters, and that it

complies with a maximum character length, if this is specified.

B.1.4     Distinguished Name Check

Checks that the value is a valid ASN.1 encoding for Distinguished

name, and that for each known attribute type (i.e. that is registered

in the DSA) each attribute value satisfies the appropriate checking

algorithm.

B.1.5     Object Identifier Check

Checks that the type is correct.  It is further study whether the

value itself can be validated further.

B.1.6     Criteria Check

Checks that the value is a valid ASN.1 encoding for Criteria.  It is

for further study whether the value itself can be validated further.

B.1.7     Presentation Address Check

Checks that the value is a valid ASN.1 encoding for Presentation

Address, and that each field is within the size limits appropriate to

the field.

B.1.8     Telephone Number Check

For further study.

B.1.9     Telex Number Check

For further study.

B.1.10    Facsimile Telephone Number Check

Checks that the value of the first element is a valid telephone number

(B.1.8), and that the second element is a valid ASN.1 encoding for

Facsimile G3 Non-Basic Parameters.

B.1.11    Teletex Terminal Identifier Check

Checks that the value of the first element is a valid Printable String

(B.1.2), and that the second element is a valid ASN.1 encoding for

Teletex Non-Basic Parameters.

B.1.12    Integer Check

Checks that the value has integer type, and lies between defined or

default integer values.

B.1.13    String List Check

Checks that each component is compliant using T.61 String Check or

Printable String Check.

B.1.14    Preferred Delivery Method Check

For further study.

B.1.15    Octet String Check

No checking.

B.1.16    Country Check

Checks that the value is a 2-character string complaint with IS 3166

codes only.

B.2  Matching Algorithms

Note:     Matching algorithms are conveniently defined in terms of a

two step process.

1.   Take the checked reference value and the value to be matched, and

reduce them to a canonical (i.e. standard) form (normalization),

if necessary.

2.   Carry out the comparison in the specified way (e.g. equality,

substrings or ordering)

Important Note

The brief descriptions below outline the first step.  The algorithms

may be replaced, in a particular implementation, by equivalent

procedures.

B.2.1     String Match

All the specific algorithms below carry out the following basic

normalization:  remove leading and trailing spaces;  intermediate

multiple spaces become single spaces.  For example

"[sp]Time[sp][sp]Flies[sp][sp]" becomes "Time[sp]Flies".

B.2.2     Case Ignore String Match

The basic normalization just described is carried out;  in addition

each lower case letter is converted to its upper case form.  The ASN.1

value is compared octet by octet, disregarding type.  It is possible

in this way to compare, for example, T.61 strings with Printable

Strings.

B.2.3     Case Exact String Match

As above, but without the lower case to upper case conversion.

B.2.4     ASN.1 Match

The ASN.1 is converted to a standardized encoding forms, such as that

given in Part 8 of the Directory documents, section 8.6.

Two ASN.1 values may then be compared octet by octet, including the

initial ASN.1 type.  No matching for substrings or order is possible.

B.2.5     Distinguished Name Match

As for ASN.1 match, except that for each AVA within the distinguished

name, normalization takes place (possibly recursively) in accordance

with the nature of the attribute type.

B.2.6     Case Ignore  List Match

Each component must match as for Case Ignore String Match.

B.2.7     Criteria Match

For further study.

B.2.8     Telephone Number Match

For further study.

B.2.9     Telex Number Match

For further study.

B.3  Mapping of Attribute Syntaxes

The following table maps the attribute syntaxes of Part 6 of the

Directory documents, to the algorithms described in B.1 and B.2 :

Syntax                       3Check                     3Match

DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD

3                          3

undefined                    3None                      3None

caseExactStringSyntax        3T.61 String Check or      3Case Exact String Match

3Printable String Check    3

caseIgnoreStringSyntax       3T.61 String Check or      3Case Ignore String Match

3Printable String Check    3

caseIgnoreListSyntax         3String List Check         3Case Ignore List Match

numericStringSyntax          3Numeric String Check      3Case Exact String Match

printableStringSyntax        3Printable String Check    3Case Exact String Match

distinguishedNameSyntax      3Distinguished Name Check  3Distinguished Name Match

objectIdentifierSyntax       3Object Identifier Check   3No normalization

criteriaSyntax               3Criteria Check            3Criteria Match

presentationAddressSyntax    3Presentation Address Check3Presentation Address Match

telephoneNumberSyntax        3Telephone Number Check    3Telephone Number Match

telexNumberSyntax            3Telex Number Check        3Telex Number Match

countrySyntax                3Country Check             3Case Ignore String Match

integerSyntax                3Integer Check             3No normalization

octetStringSyntax            3Octet String Check        3No normalization

3                          3

8.21 APPENDIX C     The Use of ROSE

The use of ROSE by the BIND and UNBIND macros as described in the

Directory documents, part 5, clause 8.2 will be used with no

additional agreements.

8.22 APPENDIX D     Guidelines

The following guidelines were used to provide a general mechanism for

arriving at pragmatic constraints within the Directory.  These are

included for the reader's information.

1.   Align with other activities

2.   Catch outlandish behavior

o    Infinite Loops

o     Runaway Process

3.   Conserve Resources and Promote Efficiency

o    Memory

o    CPU

o    Bandwidth

o    Effort

4.   Compromise Arbitrary Opinions

5.   Police Actions / Catch Violators

o    Protect the Network

6.   Facilitate Interworking

7.   Syntax Interpretation

o    Errors

8.   Set practical limits for the purpose of testing

8.23 APPENDIX EGlossary

The following abbreviations may be useful; not all are used within

these agreements.

ACL            Access Control List

ACSE           Association Control Service Element

ADDMD          Administration Directory Management Domain

AETitle        Application Entity Title

APDU           Application Protocol Data Unit

ASE            Application Service Element

ASN.1          Abstract Syntax Notation - 1

AVA            Attribute Value Assertion

B-RM           Basic Reference Model

CA             Certification Authority

CCITT          Consultative Committee on International Telegraph &

Telephone

CEN            Committee for European Normalization

CENELEC        Committee for European Normalization Electronique

CEPT           Committee European Postal & Telephonique

COS            Corporation for Open Systems

DAP            Directory Access Protocol

DIB            Directory Information Base

DIT            Directory Information Tree

DMD            Directory Management Domains

DSA            Directory System Agent

DSP            Directory System Protocol

DUA            Directory User Agent

EWOS           European Workshop for Open Systems

FTAM           File Transfer, Access & Management

INTAP          Interoperability Technical Association for Information

Processing, Japan

ISDN           Integrated Services Digital Network

ISO            International Organization for Standardization

KT             Knowledge Tree

LL             Lower layers of OSI model (layers 1-4)

MAP            Manufacturing Automation Protocol

MHS            Message Handling Systems

NBS            National Bureau of Standards

NSAP           Network Services Access Point

OSI            Open Systems Interconnection

PKCS           Public Key Crypto System

POSI           Promotion for Open System Interconnection

PRDMD          Private Directory Management Domain

PSAP           Presentation Service Access Point

RDN            Relative Distinguished Name

ROSE           Remote Operations Service Element

SIG            Special Interest Group

SPAG           Standards Promotion & Application Group

TOP            Technical and Office Protocols

UL             Upper layers of OSI model (layers 5-7)

UPU            Universal Postal Union

9. SECURITY

The Security Architecture specified in ISO 7498/Part 2 - Security

Architecture (as presented in ISO/TC 97/SC 21/N1528) shall be used as a

basis for further work in the Special Interest Group on Security.

The security services that are to be implemented first shall include

confidentiality, integrity, authentication and access control.  Non-

repudiation of the source shall also be included for consideration for

implementation.  These services are defined and discussed in more detail

in ISO 7498/Part 2 - Security Architecture.

9.1  Definitions

The following definitions, based on the definitions in ISO 7498/Part2,

are to be used when interpreting Chapter 9.

Access Control:               The provision of a security system

that establishes and enforces which

users or processes can get access

to what data or processing

facilities.

Authentication

Information:                  Information used to establish the

validity of a claimed identity.

Authorization:                The granting of access rights.

Confidentiality:              A security service that protects

data from unauthorized disclosure.

Connection:                   A state of communication that

exists between two communicating

entities by establishing an

association between them, providing

one or more data paths between them

allowing sequential transfers of

data, and then terminating the

association.

Connectionless:               A state of communication that

provides transfer of data from one

entity to another without a

preestablished association.

Data Integrity:               The property that data has not been

altered or destroyed in an

unauthorized manner.

Data Origin

Authentication:               The corroboration that the source

of data received is as claimed.

Digital Signature:            Data that allows a recipient of

information to verify the source

and integrity of the information.

Peer-entity

Authentication:               The corroboration that peer

entities in an association are as

claimed.

Repudiation:                  Denial by one or both of the

entities of an association of

having participated in all or part

of the association or

communication of the association.

Selective Field

Protection:                   The protection of specified fields

of data in a communication.

Traffic Analysis:             The inference of information from

observation of traffic flow in

communications (presence, absence,

amount, direction and frequency).

Traffic Flow

Confidentiality:              A confidentiality service to

protect against traffic analysis.

9.2  Matrix of Security Services and OSI Layers

The following matrix shows the layers of the OSI architecture at which

certain security services are considered to be desirable.  The entries

in the matrix are "H" for high level of desirability, "M" for medium

desirability, and "L" for low level of desirability.  No entry in the

matrix means that the service is not considered desirable.  This

matrix was produced from a similar matrix in ISO 7498/Part 2 which

showed the layers of the architecture that could be used to provide

the security service.  The level of desirability was established by

the members of the Special Interest Group in Security of the OSI

Implementors Workshop.

Note: The Matrix is a consensus of the opinions of the members as to

where selected security services should be placed.  It should not be

considered restrictive and interpreted as meaning that the security

services cannot be placed elsewhere in the OSI architecture or have

other implementation priorities. This will depend upon the differing

security needs of specific applications.  Also, it should not be

considered complete in that other security services may exist that

should be incorporated in the architecture.

Table 9.1  OSI Layers Desirable for Placing Security

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDBDDDBDDDBDDDBDDDBDDDBDDDBDDD?

3                   SERVICE                    3 1 3 2 3 3 3 4 3 5 3 6 3 7 3

CDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDEDDDEDDDEDDDEDDDEDDDEDDDEDDD4

3                                              3   3   3   3   3   3   3   3

31.(a) Peer entity authentication              3   3   3 L 3 H 3   3   3 H 3

3  (b) Data origin authentication              3   3   3 L 3 L 3   3   3 H 3

3                                              3   3   3   3   3   3   3   3

32. Access Control Service                     3   Authorization Model 3   3

3                                              3   3   3   3   3   3   3   3

33.(a) Connection confidentiality              3 L 3 L 3 L 3 H 3   3 H 3 H 3

3  (b) Connectionless confidentiality          3   3 L 3 H 3 L 3   3 H 3 H 3

3  (c) Selective field confidentiality         3   3   3   3   3   3 H 3 H 3

3  (d) Traffic flow confidentiality            3 M 3   3 L 3   3   3   3 L 3

3                                              3   3   3   3   3   3   3   3

34.(a) Connection integrity with recovery      3   3   3   3 H 3   3   3 L 3

3  (b) Connection integrity without recovery   3   3   3 No Plan   3   3   3

3  (c) Selective field connection integrity    3   3   3 No Plan   3   3   3

3  (d) Connectionless integrity                3   3   3 H 3 L 3   3   3 L 3

3  (e) Selective field connectionless integrity3   3   3   3   3   3   3 H 3

3                                              3   3   3   3   3   3   3   3

35.(a) Non-repudiation:  originator            3   3   3   3   3   3   3 L 3

3  (b) Non-repudiation:  receiver              3   3   3   3   3   3   3 L 3

3                                              3   3   3   3   3   3   3   3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDADDDADDDADDDADDDADDDADDDADDDY

Implementation priority:    H (high)

M (medium)

L (Low)

Table 1  ISO 7498/Part 2:  Security Addendum -- NBS OSI Workshop

Summary Of SIG-SEC Discussions of Security Service Placement, May,

1987

Notes: The following notes are for explanation of the above matrix and

comments.

A security system should be considered to be an integrated set of

security services that are placed at selected OSI layers.  The

services should be selected based on a risk analysis for the computer

system being protected.  Security mechanisms must be then chosen that

will provide the security services and incorporated in the software

and hardware of the computer system and controlled by the OSI software

and hardware at the selected layer(s).

For example, authentication, access control, confidentiality and

integrity are selected as the major security goals for an OSI system.

A connection oriented transport protocol is being implemented.  An

example of the use of the Matrix could be in an electronic mail

system, to illustrate this the following specific services and layers

were chosen:

Peer entity authentication:  Layer 4

Data origin authentication:  Layer 7

Access Control:  Layer 7

Connection confidentiality:  Layer 4

Selective field confidentiality:  Layer 7

Connection integrity with recovery:  Layer 4

Connectionless integrity:  Layer 7

The layer 7 services were chosen to support the mail system that would

protect the selective paragraphs of an electronic message as directed

by the user.  A mail system is considered connectionless.  Access

control is a function of only layer 7.

The layer 4 services were chosen to provide a reliable transport

service from the sender to the intended receive of the electronic

message.  A full connection integrity and confidentiality service with

peer entity authentication will assure that all information gets to

the receiver correctly and confidentially.

Note: The security protocols and mechanisms that provide these

services are beyond the scope of this Chapter at this time.  The

mechanisms and standards for their interoperability are presently

being defined and will be added to this Chapter as they become

available.

11.     REFERENCES

Selected references are grouped by organization publishing the documents

and by Reference Model layer to aid relating standards to the OSI Basic

Reference Model and to aid relating equivalent standards published by

different standards organizations.

10.     OBJECT IDENTIFIER: STRUCTURE AND ALLOCATION

In order to complete a stable version of the NBS OSI Implementation

Agreements, the following objects need to be administered by an ad

hoc registration authority:

Application Context Name

Abstract Syntax Name

Transfer Syntax Name

Document Type Name

Constraint Set Name

File Model

Since all objects to be administered by the NBS Workshop SIGs are

identified by the ASN.1 type OBJECT IDENTIFIER, the following

structure shall be used:

Using the NameAndNumberForm (::= identifier (NumberForm) ) for an

ObjIdComponent we have:

ObjectIdentifierValue ::=     { identifier1 (NumberForm1)

identifier2 (NumberForm2)

identifier3 (NumberForm3)

identifier4 (NumberForm4)

identifier5 (NumberForm5)

identifier6 (NumberForm6) }

The assignment of identifiers and NumberForms is as follows:

identifier1                   NumberForm1

iso                           1

identifier2                   NumberForm2

identified-organization       3

identifier3                   NumberForm3

issuing-organization          9999

identifier4                   NumberForm4

organization-code             1

identifier5                   NumberForm5

application-context           1

abstract-syntax               2

file model                    3

constraint-set                4

document-type                 5

transfer-syntax               6

ftam-nil-ap-title             7

Note 1:   The value of NumberForm3 is selected for use by implementors

of these agreements: it has not been assigned by ISO or by

any official Registration Authority.  It does correspond to

an "ad hoc" issuing organization with an ICD of 9999, as

specified by ISO 6523. We intend to use the procedure

designated in D.7 of the Specification of ASN.1, ISO 8824

once the appropriate Registration Authority has been

established.  This mechanism is subject to change dependent

upon ISO standards.

Note 2:   Specific values for identifier6 and NumberForm6 are chosen

as needed by the .  A table of the currently allocated

values is given later.

Note 3:   The NBS UL SIG will assign values for identifier5 and

NumberForm5 as required by other SIGs.

Note 4:   Companies wishing to interoperate may designate themselves

with an organization code other than 1 under

{ iso (1) identified-organization (3) -

issuing-organization (9999) } for the purpose of defining

private OBJECT IDENTIFIERs.

Table 10.1  TABLE OF ALLOCATED OBJECT IDENTIFIERS

The values of the first 4 NumberForms are constant, so the following

value is defined for use in the table below.

nbs-ad-hoc OBJECT IDENTIFIER ::= {1 3 9999 1}

Note that the only OBJECT IDENTIFIERS herein defined all begin with "nbs-

ad-hoc"; all other OBJECT IDENTIFIERS and their associated

ObjeceDescriptor's are reproduced here solely for the convenience of

implementors.  The standards defining these OBJECT IDENTIFIERSs and

ObjectDescriptor's take precedence over the values specified below.

Application Context

"ISO FTAM"

{ iso (1) standard (0) 8571 application-context (1) iso-ftam (1)

}

Abstract Syntax

"FTAM PCI"

{ iso (1) standard (0) 8571 abstract-syntax (2) ftam-pci (1) }

"FTAM FADU"

{ iso (1) standard (0) 8571 abstract-syntax (2) ftam-fadu (2) }

"FTAM unstructured text abstract syntax"

{ iso (1) standard (0) 8571 abstract-syntax (2) unstructured-text

(3) }

"FTAM unstructured binary abstract syntax"

{ iso (1) standard (0) 8571 abstract-syntax (2) unstructured-

binary (4) }

"NBS abstract syntax AS1"

{ nbs-ad-hoc abstract-syntax (2) nbs-as1 (1) }

"NBS file directory entry abstract syntax"

{ nbs-ad-hoc abstract-syntax (2) nbs-as2 (2) }

"ISO 8650-ACSE1"

{ joint-iso-ccitt association-control (2) abstract-syntax (1)

apdus (0) version1 (1) }

"Directory Services"

File Model

"FTAM hierarchical file model"

{ iso (1) standard (0) 8571 file-model (3) hierarchical (1) }

Constraint Set

"FTAM unstructured constraint set"

{ iso (1) standard (0) 8571 constraint-set (4) unstructured (1) }

"FTAM sequential flat constraint set"

{ iso (1) standard (0) 8571 constraint-set (4) sequential-flat

(2) }

"FTAM ordered flat constraint set"

{ iso (1) standard (0) 8571 constraint-set (4) ordered-flat (3) }

"NBS ordered flat constraint set"

{ nbs-ad-hoc constraint-set (4) nbs_ordered-flat (3) }

Document Type

"ISO FTAM unstructured text"

{ iso (1) standard (0) 8571 document-type (5) unstructured-text

(1) }

"ISO FTAM sequential text"

{ iso (1) standard (0) 8571 document-type (5) sequential-text (2)

}

"ISO FTAM unstructured binary"

{ iso (1) standard (0) 8571 document-type (5) unstructured-binary

(3) }

"NBS-6 FTAM sequential file"

{ nbs-ad-hoc document-type (5) sequential (6) }

"NBS-7 FTAM random access file"

{ nbs-ad-hoc document-type (5) random-file (7) }

"NBS-8 FTAM indexed file"

{ nbs-ad-hoc document-type (5) indexed-file (8) }

"NBS-9 FTAM file directory file"

{ nbs-ad-hoc document-type (5) file-directory (9) }

Transfer Syntax

" Basic Encoding of a single ASN.1 type"

{ joint-iso-ccitt (2) asn1 (1) basic-encoding (1) }

Miscellaneous

"nil AP Title"

{ nbs-ad-hoc ftam-nil-ap-title (7) }

10.1 Specific ASE Requirements for ACSE Presentation and Session

The following list for each ASE the corresponding NBS SIGs

requirements of and restrictions on ACSE, Presentation, and Session.

All listed requirements and restrictions shall be included in an NBS

conformant system and shall be implemented in accordance with these

NBS Implementor's agreements.

All OBJECT IDENTIFIERS are specified in terms of their associated

ObjectDescriptor's.  See the chapter on OBJECT IDENTIFIERs for the

values of the associated OBJECT IDENTIFIERs.

10.1.1    FTAM

10.1.1.1  Phase 2

ACSE Requirements:

all

Application Contexts:

o    "ISO FTAM" - implies the use of the

ACSE and the FTAM ASE.

Abstract Syntaxes:

o    "ISO 8650-ACSE1"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

Presentation Requirements:

Presentation Functional Units:

o    kernel

Presentation Contexts:

o    At least 4 Presentation Contexts

must be supported.

Abstract Syntaxes:

o    "FTAM-PCI"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type:

o    "FTAM-FADU"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

o    "FTAM unstructured text abstract

syntax"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

o    "FTAM unstructured binary abstract

syntax"

Associated Transfer Syntax:

o    "Basic Encoding of a

single Asn.1 type"

o    "NBS abstract syntax AS1"

Associated Transfer Syntax:

o    "Basic Encoding  of a

single ASN.1 type"

o    "NBS file directory entry abstract

syntax"

Associated Transfer Syntax:

o    "Basic Encoding of a

single Asn.1 type"

o    "NBS file directory entry abstract

syntax"

Associated Transfer Syntax:

o    "Basic Encoding of a

single Asn.1 type"

Session Requirements:

Session Functional Units:

o    kernel

o    duplex

Version Number: 2

Maximum size of User Data parameter field:

10,240

Session Options:

Session Functional Units:

o    resynchronize

only a Resynchronize Type

value of "abandon"

o    minor synchronize

Note:     The minor

synchronize

functional unit is

required whenever

the resynchronize

functional unit is

available.

10.1.2    MHS

10.1.2.1  Phase 1

Session Requirements:

Session Functional Units:

o    kernel

o    half-duplex

o    exceptions

o    activity management

o    minor synchronize

Version Number: 1

Maximum size of User Data parameter field:

512

Session Notes:

o    Restricted use is made by the RTS

of the session services implied by

the functional units selected.

Specifically,

.    No use is made of S-TOKEN-

GIVE, and

.    S-PLEASE-TOKENS only asks for

the data token.

o    In the S-CONNECT SPDU, the Initial

Serial Number should not be

present.

o    The format of the Connection

Identifier in the S-CONNECT SPDU is

described in Version 5 of the

X.400-Series Implementors' Guide.

10.1.3    DS

10.1.3.1  Phase 1

ACSE Requirements:

all

Application Context:

Abstract Syntaxes:

o    "ISO 8650-ACSE1"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

o    "Directory Services"

Associated Transfer Syntax:

o    "Basic Encoding of a

single ASN.1 type"

Presentation Requirements:

Presentation Functional Units:

o    kernel

Presentation Contexts:

o    At least 2 Presentation Contexts

must be supported.

Session Requirements:

Session Functional Units

o    kernel

o    duplex

Version Number:2

Maximum size of User Data parameter field:

10,240

11.1    CCITT

Network Layer

CCITT Recommendation X.25 - 1980, Interface Between Data Terminal

Equipment (DTE) and Data Circuit-Terminating Equipment (DCE) for

Terminals Operating in the Packet Mode on Public Data Networks.

CCITT Recommendation X.25 - 1984, Interface Between Data Terminal

Equipment (DTE) and Data Circuit-Terminating Equipment (DCE) for

Terminals Operating in the Packet Mode on Public Data Networks.

Transport Layer

CCITT Recommendation X.214, (Red Book, 1984), Transport Service

Definition for Open Systems Interconnection for CCITT Applications.

CCITT Recommendation X.224, (Red Book, 1984), Transport Protocol Profile

for Open Systems Interconnection for CCITT Applications.

Session Layer

CCITT Recommendation X.215, (Red Book, 1984), Session Service Definition

for Open Systems Interconnection for CCITT Applications.

CCITT Recommendation X.225, (Red Book, 1984), Session Protocol Profile

for Open Systems Interconnection for CCITT Applications.

Presentation

CCITT Recommendation T.50, (Red Book, 1984), International Alphabet No.

5.

Application Layer -- MHS

CCITT Recommendation X.400, (Red Book, 1984), Message Handling Systems:

System Model-Service Elements.

CCITT Recommendation X.401, (Red Book, 1984), Message Handling Systems:

Basic Service Elements and Optional User Facilities.

CCITT Recommendation X.408, (Red Book, 1984), Message Handling Systems:

Encoded Information Type Conversion Rules.

CCITT Recommendation X.409, (Red Book, 1984), Message Handling Systems:

Presentation Transfer Syntax and Notation.

CCITT Recommendation X.410, (Red Book, 1984), Message Handling Systems:

Remote Operations and Reliable Transfer Server.

CCITT Recommendation X.411, (Red Book, 1984), Message Handling Systems:

Message Transfer Layer.

CCITT Recommendation X.420, (Red Book, 1984), Message

Handling Systems: Interpersonal Messaging User Agent Layer.

CCITT Recommendation X.430, (Red Book, 1984), Message Handling Systems:

Access Protocol for Teletex Terminals.

11.2    ISO

Status of ISO work can be determined by the reference number; working

drafts are referenced by committee and number; e.g., TC 97/SC 6 Nxxxx.

Standards are cited by either ISO xxxx or IS xxxx; DIS and DPs are cited

in similar form.  Note: ISO TC 97 is now called ISO/IEC JTC1.

Information Processing Systems - Open Systems Interconnection - Basic

Reference Model.  ISO/IS 7498.  First Edition - Oct. 15, 1984. Ref. No.

ISO 7498-1984(E).

OSI Basic Reference Model - Part 2:  Security Architecture.  ISO/DIS

7498-2 TC 97/SC 21/N1895. Project 97.21.18. May 1987.

OSI Basic Reference Model - Part 3:  Naming and Addressing. ISO/DIS 7498-

3, ISO/TC 97/ SC 21 N2141. May, 1987.

Data Interchange - Structure for the identification of organizations.

ISO 6523.  1984-02-01.

Physical Layer

Information Processing Systems - Local Area Networks - Part 3:  Carrier

Sense Multiple Access with Collision Detection (CSMA/CD) and Physical

Layer Specification ISO/DIS 8802/3

Information Processing Systems - Local Area Networks - Part 4:

Token-Passing Bus Access Method and Physical Layer Specification,

ISO/DIS 8802/4

Information Processing Systems - Local Area Networks - Part 5: Token-Ring

Access Method and Physical Layer Specification ISO/DIS 8802/5

ISO 8802-5 Final Text of ISO/DIS 8802-5: Info proc systems - Local Area

Nets -Part 5: Token ring access method and physical layer specification,

ISO/TC 97/SC 6 N4477,1987

Data Link Layer

Information Processing Systems - Local Area Networks - Part 2: Logical

Link Control ISO/DIS 8802/2 1985

ISO 8802-2 Text for ISO/DIS 8802-2.2, Logical Link Control, ISO/TC 97/SC

6 N4609, 1987

Information Processing Systems - Data Communications - High-Level Data

Link Control Procedures - Description of the X.25 LAPB-compatible DTE

Data Link Procedures, IS 7776.

Network Layer

Information Processing Systems - Open Systems Interconnection - Network

Service Definition, IS 8348.

Information Processing Systems - Open Systems Interconnection - Addendum

to the Network Service Definition Covering Connectionless Data

Transmission, IS 8348/AD1.

Information Processing Systems - Open Systems Interconnection - Addendum

to the Network Service Definition Covering Network Layer Addressing,

IS 8348/AD2.

Information Processing Systems - Open Systems Interconnection - Internal

Organization of the Network Layer, DIS 8648.

Information Processing Systems - Open Systems Interconnection - Protocol

for Providing the Connectionless Network Service, IS 8473.

Information Processing Systems - Open Systems Interconnection -Addendumto

IS 8473 - Provision of the Underlying Service Assumed by ISO 8473, ISO TC

97/SC 6 N 3453.

Information Processing Systems - Open Systems Interconnection, Working

Draft, End System to Intermediate System Routing Exchange Protocol for

use in Conjunction with ISO 8473  ISO/DIS 9542 TC 97/SC 6 N 4053.

Information Processing Systems - Open Systems Interconnection - Data

Communications - X.25 Packet Level Protocol for Data Terminal Equipment,

IS 8208.

Information Processing Systems - Open Systems Interconnection - Data

Communications - Use of X.25 to provide the OSI Connection-mode Network

Service DIS 8878

Transport Layer

Information Processing Systems - Open Systems Interconnection - Transport

Service Definition, IS 8072.

Information Processing Systems - Open Systems Interconnection - Transport

Protocol Specification, IS 8072, 1984.

Session Layer

Information Processing Systems - Open Systems Interconnection - Basic

Connection Oriented Session Service Definition, ISO 8326: 1987 (E).

Information Processing Systems - Open Systems Interconnection - Basic

Connection Oriented Session Protocol Specification, ISO 8327 8326: 1987

(E).

Information Processing Systems - OSI - Basic Oriented Session Service

Definition - DAD 2 to ISO 8326 to Incorporate Unlimited User Data,

ISO/IS 8326. Aug. 27, 1987.

Information Processing Systems - OSI - Basic Oriented Session Protocol

Specification - DAD 2 to ISO 8327 to Incorporate Unlimited User Data,

ISO/IS 8327. Aug. 27, 1987.

Information Processing Systems - Open Systems Interconnection - Basic

Connection Oriented Session Service Definition-AD 2 to ISO 8326 to

Incorporate Unlimited User Data, ISO/IEC JTC1/SC21 N2494.

Information Processing Systems - Open Systems Interconnection - Basic

Connection Oriented Session Protocol Specification - AD 2 to ISO 8327 to

Incorporate Unlimited User Data, ISO/IEC JTC1/SC21 N2495.

Note:     The Session DAD2 Documents This Document, SC21 1987,

may also be obtained from:

Standards Resource Librarian

AT&T Bell Labs

IM304

Crawfords Corner Rd.

Holmdel, NJ

07733

Presentation Layer

Information Processing Systems - Open Systems Interconnection -

Connection-Oriented Presentation Service Definition, ISO 8822:  1987 (E),

(ISO/TC97/SC21 N 2335).  Note: until final text is available, reference

Proof G.

Information Processing Systems - Open Systems Interconnection -

Connection Oriented Presentation Protocol Specification, ISO 8823:  1987

(E), (ISO/TC97/SC21 N 2336).  Note: until final text is available,

reference Proof G.

Information Processing Systems - Open Systems Interconnection -

Specification of Abstract Syntax Notation One (ASN.1), ISO 8824:  1987

(E).

Information Processing Systems - Open Systems Interconnection -

Specification of Basic Encoding Rules for Abstract Syntax Notation

(ASN.1), ISO 8825:  1987 (E).

7-bit Coded Character Set for Information Processing Interchange, ISO

646.

Information Interchange - Representation of Local Time Differentials,

ISO 3307.

Application Layer

Information Processing Systems - Open Systems Interconnection -

Application Layer Structure, ISO/DP 9545, ISO/TC97/SC21/N1743.  July 24,

1987. Revised November 1987.

Application Layer -- FTAM

Information Processing Systems - Open Systems Interconnection - File

Transfer, Access and Management Part I: General Introduction, ISO 8571-1:

1987 (E) (ISO/TC97/SC21 N 2331).

Information Processing Systems - Open Systems Interconnection - File

Transfer, Access and Management Part 2: Virtual Filestore Definition, ISO

8571-2:  1987 (E) (ISO/TC97/SC21 N 2332).

Information Processing Systems - Open Systems Interconnection - File

Transfer, Access and Management Part 3:  The File Service Definition, ISO

8571-3:  1987 (E) (ISO/TC97/SC21 N 2333).

Information Processing Systems - Open Systems Interconnection - File

Transfer, Access and Management Part 4: File Protocol Specification, ISO

8571-4:  1987 (E) (ISO/TC97/SC21 N 2334).

Application Layer -- ASE/ACSE

Information Processing Systems - Open Systems Interconnection - Service

Definition for the Association Control Service Element, ISO 8649:  1987

(E) (ISO/IEC JTC1/SC21 N2326) (ISO/TC97/SC21 N)

Information Processing Systems - Open Systems Interconnection - Protocol

Specification for the Association Control Service Element, ISO 8650:

1987 (E) (ISO/IEC JTC1/SC21 N2327) (ISO/TC97/SC21 N)

Application Layer -- VTP

Information Processing Systems - Open Systems Interconnection - Virtual

Terminal Service - Basic Class, IS 9040.

Information Processing Systems - Open Systems Interconnection - Virtual

Terminal Protocol - Basic Class, IS 9041.

Application Process -- Office Document Interchange -- ODA/ODIF

Information Processing Systems - Text and Office Systems - Office

Document Architecture (ODA) and Interchange Format - Part 1; General

Information, DIS 8613/1.

Information Processing Systems - Text and Office Systems - Office

Document Architecture (ODA) and Interchange Format - Part 2; Document

Structures, DIS 8613/2.

Information Processing Systems - Text and Office Systems - Office

Document Architecture (ODA) and Interchange Format - Part 3; Document

Processing Reference Model, DIS 8613/3.

Information Processing Systems - Text and Office Systems - Office

Document Architecture (ODA) and Interchange Format - Part 4; Document

Profile, DIS 8613/4.

Information Processing Systems - Text and Office Systems - Office

Document Architecture (ODA) and Interchange Format - Part 5; Office

Document Interchange Format, DIS 8613/5.

Information Processing Systems - Text and Office Systems - Office

Document Architecture (ODA) and Interchange Format - Part 6; Character

Content Architecture, DIS 8613/6.

Information Processing Systems - Text and Office Systems - Office

Document Architecture (ODA) and Interchange Format - Part 7; Raster

Graphics Content Architecture, DP 8316/7.

Information Processing Systems - Text and Office Systems - Office

Document Architecture (ODA) and Interchange Format - Part 8; Geometric

Graphics Content Architecture, DP 8613/8.

Information Processing Systems - Text and Office Systems - Standard

Generalized Markup Language (SGML), IS 8879.

Application  Process -- Computer Graphics -- CGM/GKS

Information Processing Systems - Computer Graphics - Metafile (CGM) for

the Storage and Transfer of Picture Description Information, Part 1;

Functional Specification, IS 8632/1

Information Processing Systems - Computer Graphics - Metafile (CGM) for

the Storage and Transfer of Picture Description Information, Part 2;

Character Encoding, IS 8632/2

Information Processing Systems - Computer Graphics - Metafile (CGM) for

the Storage and Transfer of Picture Description Information, Part 3;

Binary Encoding, IS 8632/3.

Information Processing Systems - Computer Graphics - Metafile (CGM) for

the Storage and Transfer of Picture Description Information, Part 4;

Clear Text Encoding, IS 8632/4.

Information Processing Systems - Font and Character Information

Interchange, IS 9541.

Information Processing Systems - 8-Bit Single Byte Coded Graphic

Character Sets, Part 1; Latin Alphabet Part 1, IS 8859/1.

Information Processing Systems - Computer Graphics Functional

Specification of the Graphical Kernel System (GKS), IS 7942.

Information Processing Systems - Computer Graphics - Graphical Kernel

System for Three Dimensions (GKS-3D), Functional Description, DIS 8805.

Information Processing Systems - Computer Graphics - Programmers

Hierarchical Interactive Graphics System (PHIGS), DP 9592.

Information Processing Systems - Computer Graphics - Interfacing

Techniques for Dialogues with Graphical Devices (CGI),ISO TC 97/SC 21 N

1179.

Application Layer -- Directory Services

The Directory--Overview of Concepts, Models, and Services (CCITT

Recommendation X.500, ISO 9594)

The Directory--Information Framework (CCITT Recommendation X.501, ISO

9594)

The Directory--Access and System Services Definition (CCITT

Recommendation X.511, ISO 9594)

The Directory--Procedures For Distributed Operation (CCITT

Recommendation X.518, ISO 9594)

The Directory--Access and System Protocols Specification (CCITT

Recommendation X.519, ISO 9594)

The Directory--Selected Attribute Types (CCITT Recommendation X.520, ISO

9594)

The Directory--Selected Object Classes (CCITT Recommendation X.521, ISO

9594)

The Directory--Authentication Framework (CCITT Recommendation X.509, ISO

9594)

Remote Operations-Part 1: Model, Notation and Service Definition (CCITT

Recommendation X.219, ISO 9072 Version 5)

Remote Operations-Part 2: Protocol Specification (CCITT Recommendation

X.229, ISO 9072 Version 5)

Association Control-Service Definition (CCITT Recommendation X217, ISO

8649)

Association Control-Protocol Definition (CCITT Recommendation X.217, ISO

8650)

Note: ISO 9594, 9072 are preferred texts (over the CCITT counterparts)

and are to be taken as of Gloucester, Nov. 1987.

CCITT documents may be obtained from:

International Telecommunications Union

Place des Nations, CH 1211,

Geneva 20 SWITZERLAND

ISO documents may be obtained from:

Frances E. Schrotter

ANSI

ISO TC 97/SC 6 Secretariat

l430 Broadway

New York, NY.  10018

11.3    IEEE

Physical Layer

IEEE Standard for Local Area Networks:  Carrier Sense Multiple Access

with Collision Detection (CSMA/CD) and Physical Layer Specification,

ANSI/IEEE Standard 802.3 1985, Institute of Electrical and Electronics

Engineers, 345 East 47th St., New York, NY. 10017, 1985.

IEEE Standard for Local Area Networks: Supplements to Carrier Sense

Multiple Access with Collision Detection (CSMA/CD) Access Method and

Physical Layer Specification ANSI.IEEE Standard 802.3a, b, c, e -1988

Supplement a, MAU and Baseband Medium Specification, Type 10BASE2

(Section 10)

Supplement a, Broadband MAU & Medium Specification, Type 10BROAD36

(Section 11)

Supplement c, Repeater Set and Repeater Unit Specification for Use with

10BASE5 and 10 BASE2 Networks

Supplement e, Physical Signaling, Medium Attachment, and Baseband Medium

Specification, Type 10BASE5

IEEE Standard for Local Area Networks:  Token-Passing Bus Access Method

and Physical Layer Specification, ANSI/IEEE Standard 802.4 - 1985, 802.4

Draft 1987, Institute of Electrical and Electronics Engineers, 345 East

47th St., New York, NY. 10017, 1985.

IEEE Standard for Local Area Networks:  Token-Ring Access Method,

ANSI/IEEE Standard 802.5-1985 1986, Institute of Electrical and

Electronics Engineers, 345 East 47th St., New York, NY. 10017, 1985.

Data Link Layer

IEEE Standard for Local Area Networks:  Logical Link Control, ANSI/IEEE

Standard 802.2 - 1985 1987, Institute of Electrical and Electronics

Engineers, 345 East 47th St., New York, NY. 10017, 1985.

11.4    NBS

Local Area Networks:  Baseband Carrier Sense Multiple Access with

Collision Detection Access Method and Physical Layer Profiles and Link

Layer Protocol, FIPS l07, NTIS, U.S. Department of Commerce, 5285 Port

Royal Road, Springfield, VA. 22l6l.

Interface Between Data Terminal Equipment (DTE) and

DataCircuit-Terminating Equipment (DCE) For Operation With

Packet-Switched Data Communications Networks, FIPS l00, NTIS, U.S.

Department of Commerce, 5285 Port Royal Road, Springfield, VA. 22l6l.

Implementation Agreements for Open Systems Interconnection Protocols:

NBS Workshop for Implementors of Open Systems Interconnection, National

Bureau of Standards, NBSIR 86-3385-6, Robert Rosenthal, Editor, Revised

July 1987.

Implementation Agreements Among Participants of OSINET, National Bureau

of Standards, Institute for Computer Sciences and Technology, NBSIR

86-3478, 1987.

U. S. Government Open Systems Interconnection Profile (GOSIP),

National Bureau of Standards, Institute for Computer Sciences and

Technology, 1987.

NBS documents may be obtained from:

NTIS

U.S. Department of Commerce

5285 Port Royal Road

Springfield, VA. 22l6l.

or

National Bureau of Standards

Institute for Computer Sciences and Technology

Gaithersburg, MD. 20899

11.5    MAP

Manufacturing Automation Protocol, General Motors

Corporation,Manufacturing Engineering and Development, Advanced Product

and Manufacturing Engineering Staff (APMES), APMES A/MD-39, GM Technical

Center, Warren, MI. 48090-9040.

11.6    TOP

Technical and Office Protocols Specification Version 3.0, MAP/TOP Users

Group, Attention TOP 3.0 Document.  One SME Drive, P.O. Box 930, Dearborn

Mi. 48121.

11.7    CEN/CENELEC

FTAM Draft Functional Standard A/111 (prENV 41 204) Simple File Transfer-

Unstructrued, 1987-12-01.

ENV 41201 "Private Message Handling Systems"

ENV 41202 "Public Message Handling Systems"

11.8    SPAG

FTAM Draft Profile A/112 Positional File Transfer - Flat, 1987-120-7.

FTAM Draft Profile A/122 Positional File Access - Flat, 1987-12-07.

FTAM Draft Profile A/13 File Management, 1987-12-07.

READER RESPONSE FORM

Please retain my name for the next mailing of the NBS/OSI Implementors

Workshop.

ZDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDD?

3                                                                 3

3NAME                                                             3

3                                                                 3

3ADDRESS                                                          3

3                                                                 3

3                                                                 3

3                                                                 3

3                                                                 3

3                                                                 3

3PHONE NO.                                                        3

3                                                                 3

@DDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDDY

Mail this page to:     National Bureau of Standards

NBS Workshop for Implementors of OSI

Bldg. 225/B-217

Gaithersburg, MD  20899